3GPP TSG-RAN WG2 Meeting #131	R2-2506029

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.4

Source:			ASUSTeK

Title:	Discussion on Ambient IoT data transmission

Document for: 	Discussion and Decision

- Introduction

Currently, the running CRs for Rel-19 ambient IoT has been provided and reviewed. And the remaining issues on A-IoT MAC were discussed in the offline discussion after RAN2#130 [1]. In this contribution, we would like to further discuss some remaining issues regarding ambient IoT data transmission.

- Discussion

For open issue 3-7, as discussed in Q#9 of the offline discussion [1], the “more data indication” should be set to 0 for the scenario that there is no NAS response available. This scenario for Write Command has been discussed in the last meeting and companies agreed to send LS [2] to CT1 to solve the issue in NAS. In A-IoT MAC, the device includes zero SDU in the D2R MAC PDU.

Moreover, the more data indication is introduced for segmentation indicating that there is remaining data after the D2R message. If the size of available D2R data is larger than the resources for a D2R transmission, the device segments the D2R data and sets the segmentation indication to 1. If the size of available D2R data is not larger than the resources for the D2R transmission or the D2R transmission is the last segmentation, the device sets the segmentation indication to 0. Since the NAS response is unavailable at the time when the device transmits the D2R message with zero SDU, the more data indication should be set as 0. Otherwise, since the reader doesn’t know when the delayed NAS response will be available at the device, the reader may continue scheduling D2R transmissions which still includes zero SDU. As a result, setting the indication to 1 would cause signalling overhead and resources waste without substantial benefits.

Proposal 1: [Issue 3-7] In case of no NAS response available for command, the more data indication is set to 0 as current MAC CR.

Secondly, as we propose in Q#12, we think some reserved bits could be added to the D2R Upper Layer Data Transfer message. It has been agreed that RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. Similarly, we propose that the Rel-19 AIoT D2R messages should be extensible for future release. To achieve this, some reserved bits should also be included in the D2R Upper Layer Data Transfer message. Since the D2R Upper Layer Data Transfer message is currently byte-aligned, a byte of reserved bits could be considered.

Several potential usages for the reserved bits in the future release have been raised. For example, as discussed in offline [1], D2R message type may be not necessary in Rel-19. However, it could be beneficial for DO-A D2R transmission in Rel-20. On the other hand, it was agreed that the device may include energy status indication in D2R messages in RAN2 #127bis meeting. As captured in TR 38.769 [3], the device could provide a 1-bit energy status indication to the reader in a D2R message, which indicates that the A-IoT device does not have sufficient energy to perform the remaining/follow-up procedure. This indication could be considered for active devices in Rel-20. Otherwise, if an A-IoT device does not have enough energy to receive/transmit the following transmission(s), the ongoing/follow-up procedure would fail.

Proposal 2: [Issue 4-5] Some reserved bits could be added to D2R Upper Layer Data Transfer message for future extendibility.

Lastly, it hasn’t discussed that whether one R2D MAC PDU could include multiple R2D messages, for example, one R2D MAC PDU includes both NACK Feedback message and Access Trigger message. In our understanding, one R2D MAC PDU seems to include one R2D message. Since there is no length field for a R2D message with variable size, it’s not possible for the device to decode a R2D MAC PDU with multiple R2D messages. As a result, one R2D MAC PDU can include only one R2D message.

Proposal 3: Confirm that one R2D MAC PDU includes one R2D message.

- Conclusion

We discuss some remaining issues and have the following proposals for A-IoT data transmission:

Proposal 1: [Issue 3-7] In case of no NAS response available for command, the more data indication is set to 0 as current MAC CR.

Proposal 2: [Issue 4-5] Some reserved bits could be added to D2R Upper Layer Data Transfer message for future extendibility.

Proposal 3: Confirm that one R2D MAC PDU includes one R2D message.

- Reference

- R2-2505521, “Summary of A-IoT MAC open issues (outcome of [POST130][027][AIoT] MAC Running CR).”

- R2-2504967, “LS on delayed A-IoT D2R NAS messages.”

- 3GPP TR 38.769 V19.0.0, “Study on solutions for Ambient IoT in NR.”