3GPP TSG-RAN WG2 Meeting #131	R2-2506032

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.4

Source: 	ASUSTeK

Title:  	Discussion on remaining issue for C-LTM

Document for:	Discussion and Decision

- Introduction

In the post email discussion [POST130][111], MAC issues regarding conditional LTM were discussed and an issue on candidate TA and timer handling for source Cell was raised:

2. When performing LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid. It should be discussed whether to store the TA for source cell and maintain ltm-Candidate-TimeAlignmentTimer for the source cell if it’s a C-LTM candidate Cell. [Rapp]: No matter LTM or other HO, if the source cell is a CLTM candidate cell, the running TAT for candiate cell will not be stopped based on the current running CR in MAC reset: 1> else if upper layers indicate the reset is triggered by conditional LTM, or triggered by LTM while there is CLTM candidate configuration(s), or triggered by handover while there is CLTM candidate configuration(s): 2> stop (if running) all timers, except MBS broadcast DRX timers, ltm-Candidate-TimeAlignmentTimers, and tm-Candidate-TimeAlignmentTimerTAG2, if configured; 2> consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

In this contribution, we further clarify the raised issue regarding handling TA for Source Cell in Conditional LTM.

- Discussion

Storing TA for LTM source Cell

In the current C-LTM procedure, when receiving a LTM Candidate Timing Advance Command MAC CE indicating a C-LTM candidate cell, the UE stores TA value indicated in the MAC CE for the indicated C-LTM candidate cell and start or restart ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM associated with the indicated LTM candidate cell.

On the other hand, the Source Cell of the C-LTM procedure may also be a C-LTM candidate Cell, and the TA value of the Source Cell can still be valid when performing a C-LTM to a target Cell.

Observation 1:	When performing C-LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

If the source cell is associated with a C-LTM candidate, then it would be beneficial to store the valid TA of the source cell for possible RACH-less C-LTM back to the source cell. In the current running CR, the UE does not stop ltm-Candidate-TimeAlignmentTimers for candidate cells upon MAC reset triggered by C-LTM to maintain TA validity for candidate Cells. However, if the ltm-Candidate-TimeAlignmentTimer for Source Cell’s LTM candidate configuration is not running at the time when C-LTM is triggered, the UE cannot store or maintain the TA for the Source Cell.

Observation 2:	When the UE performs C-LTM execution, the ltm-Candidate-TimeAlignmentTimer for C-LTM candidate configuration of the Source Cell may not be running, and the UE cannot store the valid TA for the C-LTM candidate configuration of the Source Cell.

To fix this scenario where candidate cell’s TA cannot be stored, we propose the UE to start or restart the corresponding C-LTM timer (ltm-Candidate-TimeAlignmentTimer) associated with the C-LTM candidate of the source cell upon C-LTM execution.

Figure 1

As illustrated in Figure 1, while the Source Cell is associated with a CLTM candidate configuration and the TA is valid, the new C-LTM timer (i.e., ltm-Candidate-TimeAlignmentTimer) is not running when it is a Serving Cell (since no LTM Candidate Timing Advance Command MAC CE is received). At timing t1, the UE determines to perform LTM on a candidate cell, where the TAT of the source cell has been running for n ms. The UE then stores the TA of the source cell for the corresponding C-LTM candidate that includes the source cell as the SpCell (e.g., the C-LTM-Candidate of which ltm-CandidatePCI is set to the PCI of the source cell). The UE then (re)starts the ltm-Candidate-TimeAlignmentTimer associated with the corresponding C-LTM candidate, with the length of the remaining time of the timeAlignmentTimer of the source cell (i.e., m-n ms).

Proposal 1:	Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts or restarts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

- Conclusion

We have the following observations and proposal for TA handling on C-LTM:

Observation 1:	When performing LTM to a target cell, the TA associated with a source cell of the LTM, which could be a C-LTM candidate Cell, can still be valid.

Observation 2:	When the UE performs C-LTM execution, the ltm-Candidate-TimeAlignmentTimer for C-LTM candidate configuration of the Source Cell may not be running, and the UE cannot store the valid TA for the C-LTM candidate configuration of the Source Cell.

Proposal 1:	Upon execution of a C-LTM to a candidate cell, if the source cell is a SpCell of a C-LTM candidate configuration, the UE stores the TA of the source cell for the C-LTM candidate configuration, and starts or restarts the ltm-Candidate-TimeAlignmentTimer associated with the C-LTM candidate with the length of the remaining time of the timeAlignmentTimer of the source cell.

- Text proposal (based on running MAC CR for mobility PH4)

## 5.2x	Maintenance of UL Synchronization for CLTM candidate cell

The MAC entity shall for each CLTM candidate cell:

1>	when an LTM Candidate Timing Advance Command MAC CE described in clause 6.1.3.4x is received:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell for the indicated TAG as specified in clause 6.1.3.4x;

3>	start or restart the ltm-Candidate-TimreAlignmentTimer or ltm-Candidate-TimeAlignmentTimerTAG2 associated with the indicated LTM candidate cell for the indicated TAG as specified in clause 6.1.3.4x;

2>	else:

3>	store the TA value in the LTM Candidate Timing Advance Command MAC CE for the indicated CLTM candidate cell as specified in clause 6.1.3.4x;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the indicated LTM candidate cell as specified in clause 6.1.3.4x.

1>	when the Source Cell of a CLTM execution is a CLTM candidate cell:

2>	if two TAGs are configured for the CLTM candidate cell:

3>	store the TA values of the Source Cell;

3>	start or restart the ltm-Candidate-TimreAlignmentTimer and ltm-Candidate-TimeAlignmentTimerTAG2 associated with the CLTM candidate cell with the length of the remaining time of the timeAlignmentTimer of corresponding PTAGs of the Source cell;

2>	else:

3>	store the TA value of the Source Cell;

3>	start or restart the ltm-Candidate-TimeAlignmentTimer associated with the CLTM candidate cell with the length of the remaining time of the timeAlignmentTimer of PTAG of the Source cell.