3GPP TSG-RAN WG2 Meeting #131	R2-2506039

Bangalore, India Aug 25th – 29th, 2025

Source:                Qualcomm Incorporated

Title:	Open issues on LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 LP-WUS in DC/CA

For secondary DRX with LP-WUS, RAN2 has the following working assumption.

RAN2#130 progress

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime.  When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

During RAN2 post-meeting email discussion [2], open additional issue (i.e, open issue 1-2 : If supported, whether to monitor LP-WUS on PCell if the secondary DRX group is in DRX active time) was discussed on whether to monitor LP-WUS on PCell if secondary DRX group is in DRX active time.

For this new case, there are two options discussed during email discussion

- Option 1: UE monitors LP-WUS when both DRX groups are not in DRX active time;

- Option 2: UE monitors LP-WUS when the default DRX group is not in DRX active time and secondary DRX group is in DRX active time.

Most companies prefer option 1 since companies considered RAN1 following working assumption on LR and MR operation simultaneously.

RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19.

However, RAN2 had agreed that LP-WUS can be used in DC as follows.

RAN2 progress:

For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells.

For NR-DC, the LP-WUS in MCG and SCG can be configured independently.

According to RAN2 agreement on DC that LP-WUS in MCG and SCG ban be configured independently, it is understood RAN2 assumed that UE can operate LR and MR in MCG and SCG simultaneously; otherwise, LP-WUS cannot be used in DC case since MN and SN cannot coordinate DL scheduling to avoid MR and LR operation simultaneously.

Considering it is unclear whether RAN1 working assumption is applied for different bands or not, we propose to keep RAN2 understanding that RAN1 working assumption is not applied for different frequency bands and send LS for RAN1 confirmation.

- RAN2 assumes that the following RAN1 assumption is only applied for single frequency band, and RAN1 working assumption does not impact RAN2 agreement on DC LP-WUS. Send LS for RAN1 confirmation.

RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19.

For dual DRX groups, option 2 is easier and more beneficial considering DL traffic may only be scheduled on one DRX group. That means even though there is ongoing DL traffic on one DRFX group, the other DRX group can still save power using LP-WUS. And option 2 is easier for UE implementation since UE only considers the non-active time of the default DRX group.

For option 1, UE has to wait for non-active time on both DRX groups, UE power saving will be lost if one DRX group has always data transmission and always in active time. And it is more complex that UE consider non-active time on both DRX groups. It is proposed that Proposal 2	UE monitors LP-WUS when the default DRX group is not in DRX active time regardless of whether secondary DRX group is in active time or not.

- UE monitors LP-WUS when the default DRX group is not in DRX active time regardless of whether secondary DRX group is in active time or not.

## 2.2 Switching between LP-WUS and legacy PDCCH monitoring

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to gNB. And gNB should send switching command to the UE to enable LP-WUS. Then there is no misalignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, it can be FFS for other conditions, and it can be FFS on whether the entry/exist condition need to be configured to the UE or can be left UE implementation.

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

## 2.3 LP-WUS activation/deactivation

RAN1 decided to study activation/deactivation of LP-WUS monitoring as highlighted below in the RAN1#116 agreement.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

It could be possible that a network configures one or multiple LP-WUS configurations, and one of the LP-WUS configuration is activated by the MAC-CE. The MAC-CE can also deactivate LP-WUS monitoring (i.e., it activates none of the LP-WUS configurations), in which case the UE should switch to legacy C-DRX configuration.

In each of the LP-WUS configurations, various RRC parameters can be provided, then the MAC-CE activation can control the overhead/efficiency, associated beam, etc, in dynamic manner.

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

## 2.4 Paging monitoring in CONNECTED state

For paging, since the paging is cell specific and can be applicable for all RRC state. For CONNECTED state UE, it is possible to monitor paging DRX cycle for SI notification if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace. There are two options for paging DRX handling in CONNECTED state.

Option 1: UE in CONNECTED state monitors paging as legacy. That LP-WUS is not used for paging DRX in CONNECTED state.

Option 2: IDLE/Inactive LP-WUS can be used for the UE in CONNECTED state for paging monitoring. In this option, UE needs to monitor LP-WUS as in IDLE/Inactive state.

Option 2 has more power saving, but UE needs to support monitor two different LP-WUS signalling. For paging for SI update, gNB needs to send paging using both LP-WUS and legacy methods since there are LP-WUS UE and legacy UE in the network. Considering UE may have different capability to monitor two different LP-WUS signalling, then it should be left UE implementation to Proposal 9	It is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

- If the UE is in CONNECTED state, it is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring.

# Conclusion

LP-WUS in DC/CA

- RAN2 assumes that the following RAN1 assumption is only applied for single frequency band, and RAN1 working assumption does not impact RAN2 agreement on DC LP-WUS. Send LS for RAN1 confirmation.

RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19.

- UE monitors LP-WUS when the default DRX group is not in DRX active time regardless of whether secondary DRX group is in active time or not.

Switching between LP-WUS and legacy PDCCH monitoring

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring.

LP-WUS activation/deactivation

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

Paging monitoring

- It is up to UE implementation to monitor paging using legacy paging or LP-WUS monitoring if the UE is in CONNECTED state.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]    [Post130][213][LPWUS] Running CR for 38.321 (Apple).