3GPP TSG-RAN WG2 Meeting #131	R2-2506040

Bangalore, India Aug 25th – 29th, 2025

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

2.1 Exit conditions

In RAN2 #128 meeting, there is FFS on exist condition as following. There were some discussion in RAN2#129bis meeting, but no conclusion is achieved.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

If RRM relaxation exit shares the same condition with entry, then there could be a ping-pong switching between entry and exit. To avoid this ping-pong issue, it is preferred that a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

- To avoid ping-pong switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation and offloading exit condition.

For RRM relaxation mode, one open issue is whether both MR and LR threshold to be configured to the UE. Network can configure both of MR and LR threshold as RRM relaxation mode exit condition, and when either MR or LR measurement is below the corresponding threshold, UE should exit from RRM relaxation mode.

- When any of LR or MR measurement is below a threshold configured for exit condition, UE exits from RRM relaxation mode and enters legacy measurement mode.

## 2.3 (OI:38304-2) Legacy neighboring measurement relaxation criterion

In existing mechanism, relaxed measurement criterion is configured in TS 38.304 to the UE to determine not at cell edge as follows [].

The relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

When in relaxed measurement mode to LP-WUS, UE will measure using both of MR and LR, but the MR measurement is in a relaxed way. One easy way is using existing serving cell MR measurement, but serving cell measurement is in a relaxed way, then no LP-WUS specific measurement criterion needs to be introduced for ‘not at cell edge’.

- No LP-WUS specific measurement criterion is to be introduced for Rel-16 “not at cell edge” evaluation for neighboring measurement relaxation.

## 2.4 (OI: 38304-6) Low mobility

Currently, UE low mobility is determined as follow criterion, there were proposals that UE can also evaluate whether UE is in low mobility in LP-WUS offloading. Considering this is the last RAN2 meeting, and the benefit is not clear, it is proposed that low mobility evaluation is not performed in LP-WUS offloading mode.

The relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

- Low mobility evaluation is not performed in LP-WUS offloading mode.

Another issue is whether UE mobility status should be considered for Rel-19 RRM relaxation/offloading entry evaluation. This issue has not be discussed in RAN4, RAN2 does not need to consider this criterion to evaluate whether to enter or leave Rel-19 RRM relaxation/offloading.

- RAN2 does not consider low mobility or stationary status to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

## 2.4 Merge LP-WUS monitoring and RRM relaxation conditions

In last RAN2 meeting, there were discussion on how to merge LP-WUS and RRM relaxation conditions, and two options are on the table.

      - Direction 3a. merge LPWUS monitoring and serving cell offloading

      - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

If merging LP-WUS monitoring and serving cell offloading entry/exit condition, then LP-WUS monitoring usage could be a limited scope; so merging LP-WUS monitoring and Rel-19 relaxation is preferable.

- Merge LPWUS monitoring and Rel-19 RRM relaxation entry/exit condition.

# Conclusion

- To avoid ping-pong mode switching, a separate threshold or a hysteresis to the entry threshold can be introduced for RRX relaxation/offloading exit condition.

- When any LR or MR measurement is below a threshold configured for exit condition, UE exits from RRM relaxation mode and enters legacy measurement mode.

- No LP-WUS specific measurement criterion is to be introduced for Rel-16 “not at cell edge” evaluation for neighboring measurement relaxation.

- Low mobility evaluation is not performed in LP-WUS offloading mode.

- RAN2 does not consider low mobility or stationary status to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

- Merge LPWUS monitoring and Rel-19 RRM relaxation entry/exit conditions.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)