3GPP TSG-RAN WG2 Meeting #131	R2-2506069

Bengaluru, India, Aug. 25th – 29th, 2025

Agenda Item:	8.7.6

Source:	HONOR

Title:	Discussion on XR rate control

Document for:	Discussion and Decision

1. Introduction

In RAN#105 meeting, the following objective of Rel-19 XR work item was agreed [1]:

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In this contribution, we will further discuss the open issues of XR rate control.

2. Discussion

2.1. [MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE

Regarding the ID design for QoS flows discussed at RAN2 #130 meeting, the following options were considered:

a. Explicit DRB ID + QFI (FFS whether QFI is explicit or implicit)

b. Implicit (e.g., index or mapping)

We advocate for explicit DRB ID + implicit QFI indication in the UL Rate Control MAC CE, based on:

- Explicit DRB ID is necessary in case that XR QoS Flows are mapped to different DRB.

- Option a has a better flexibility, e.g., it can be applied to multiple PDU Sessions (not limit only to XR session).

- Implicit QFI, e.g. mapping, can greatly reduce the overhead of the MAC CE.

Proposal 1: (MAC-03) Explicit DRB ID + implicit QFI to identify QoS flows require throttling.

2.2. [MAC-4] Format of the UL Rate Control MAC CE

We propose a bitmap shall indicate QoS Flows require throttling, where each bit position corresponds to one corresponding QoS Flow:

- Bit value 1: Applies rate control to the corresponding QoS Flow which shall be assigned a recommended bitrate.

- Bit value 0: No action.

Figure 1 illustrates the UL Rate Control MAC CE format. Consider a DRB configured as: mappedQoS-FlowsList = [QFI2, QFI3, QFI4, QFI5]. The bitmap indices correspond to QoS Flow positions in this list:

- Bit 0 → Position 0 (QFI2)

- Bit 1 → Position 1 (QFI3)

- Bit 2 → Position 2 (QFI4)

- Bit 3 → Position 3 (QFI5)

To apply rate control to QFI3 and QFI5, the gNB sets the bitmap to 0b0101(bit0 is the rightmost).

Figure 1 Format of the UL Rate Control MAC CE

Proposal 2: (MAC-04) Use a bitmap to indicate QoS Flows require throttling, where each bit corresponds to one concerned QoS Flow.

Proposal 3: (MAC-04) Each bit value set to 1 applies rate control to the corresponding QoS Flow which shall be assigned an available bitrate.

2.3. [MAC-5] Whether a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the pending queries

For (MAC-5) open issue, we propose a truncated MAC CE may be utilized. For example, such truncated MAC CE may omit the available bitrate field, retaining only the bitmap field. This design allows the MAC CE to:

- Convey all QoS flows requiring rate queries

- Adapt to constrained uplink resources

Proposal 4: (MAC-5) Design a truncated MAC CE to adapt to constrained uplink resources, e.g., retain only the bitmap field to convey all QoS flows information requiring rate queries.

3. Conclusions

This contribution provides the following proposals.

Proposal 1: (MAC-03) Explicit DRB ID + implicit QFI to identify QoS flows require throttling.

Proposal 2: (MAC-04) Use a bitmap to indicate QoS Flows require throttling, where each bit corresponds to one concerned QoS Flow.

Proposal 3: (MAC-04) Each bit value set to 1 applies rate control to the corresponding QoS Flow which shall be assigned an available bitrate.

Proposal 4: (MAC-5) Design a truncated MAC CE to adapt to constrained uplink resources, e.g., retain only the bitmap field to convey all QoS flows information requiring rate queries.

4. References

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3