3GPP TSG-RAN WG2 Meeting #131	R2-2506090

Bengaluru, India, 19 – 23 August 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

The following agreements were made:

Agreements on OD-SIB1: RAN2#129

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

Agreements on OD-SIB1: RAN2#130

- Not to include access related information of NES into WUS configuration.

- Normative text for 2nd check, e.g. “if SIB1 is not broadcast”, is removed. RAN2 captures a note in the specification to clarify it’s up to UE’s implementation whether and how to check if SIB1 is currently being broadcasted or provided on demand for that cell before triggering OD-SIB1 request procedure of that cell.

- TP1 in R2-2503636 is agreed with the removal of “until receiving SIBxx from this cell”.

- Add the UE behaviour in 5.2.2.3.5: The UE supporting OD-SIB1 in RRC_CONNECTED considers the si-BroadcastStatus in the stored SIB1 as the latest one in NES cell. Detailed wordings can be further discussed in the preparation of running CR.

- If there is no common searchspace configuration on current active BWP, the network will ensure the RRC_CONNECTED UE can acquire the updated UL WUS configuration, i.e., the dedicatedSystemInformationDelivery IE in RRCReconfiguration is also applied to new SIB containing UL WUS configuration.

- Include the following parameters from the RAN1 parameter list in SIBxx: frequencyBandList, Prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, zeroCorrelationZoneConfig, preambleReceivedTargetPower, powerRampingStep, ra-ResponseWindow, searchSpaceZero, controlResourceSetZero, ra-SearchSpace, n-TimingAdvanceOffset, ssb-PeriodicityServingCell, od-sib1-WindowDuration, od-sib1-windowStartOffset.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2 – For information only

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 3	Switching between legacy SIB1 transmission and on-demand SIB1 transmission

First, we provide a summary about related agreements and discussion for this aspect. These agreements are used as starting point and assumptions for our proposals and observations for this aspect.

RAN2 agreed about the following:

RAN2 #127bis agreements:

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN2 #127bis agreements:

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

RAN2 #129bis agreements:

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

RAN1#118bis agreed about the following:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

-	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

-	UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Based on RAN2 #129bis agreement, legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. This means, when (during the SIB1 transmission window) the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), the cell can set the K_SSB of the SSB associated with this cell to:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2: Legacy periodic SIB1 transmission

  - Legacy UE and Rel.19 UEs can camp on the cell as legacy UE can acquire broadcasted SIB1.

- K_SSB >23 for FR1 and K_SSB > 11 for FR2.

  - Legacy UE cannot camp on the cell as legacy UE identifies the cell as NCD-SSB.

  - If Rel.19 UE has SIB1 request configuration of a cell, Rel. UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

For NES SIB1 transmission mode switch, the following points are discussed in this contribution:

- UE expectation to the value of K_SSB in NES, if SIB1 is currently being broadcasted or provided on-demand.

- How UE can check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- MIB TTI is 80ms. This means that MIB payload does not change every 80ms. However, with repetition MIB can be transmitted every 20ms.

For point 1) and 2) we summarize in the state diagram below, K_SSB value with respect to NES cell SIB1 transmission mode based on RAN1 and RAN2 agreements.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2, is broadcasting SIB1 as legacy and NES UE will acquire SIB1 as in legacy.

- K_SSB > 23 for FR1 and K_SSB > 11 for FR2, is providing SIB1 on-demand. UE needs to monitor SIB1 to check if it is being broadcasted within a limited window before requesting SIB1 of that cell. If UE does not detect a SIB1, UE to request OD-SIB1 for that cell.

For 3), 38.212 specifies that:

“Data arrives to the coding unit in the form of a maximum of one transport block every 80ms.”

In 38.331, it specifies that SSB periodicity is at least 80 ms, with repetition within 80 ms. Basically from 38.331 and 38.212, UE is expecting to no change in PBCH payload. Based on this, many UEs apply combining for SSB detection/decoding, mainly cell-edge UEs.

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

To avoid impact to legacy UE, NW should ensure MIB TTI of 80ms is respected. As K_SSB is a part of MIB, hence NW cannot switch between SIB1 broadcasting state or SIB1 provided on demand with a periodicity less than 80ms. This implies that when NES cell is operating with OD-SIB1 and K_SSB > 23 for FR1 and K_SSB > 11 for FR2, UE assumes that the same value of K_SSB within 80ms.

To avoid impact to legacy UEs and to guarantee MIB TTI of 80ms (8 radio frames), NW shall consider only specific timing for the switching from broadcasting SIB1 to on-demand SIB1. RAN2 can discuss to define this specific timing as function of SFN, e.g. SFN mod 8 = 0. Hence, Rel. 19 NES UE with SIB1 request configuration intended to transmit a request for OD-SIB1, should check at least every 80ms the SIB1 transmission status based on K_SSB carried by the MIB.

Based on above discussion, it is clear to avoid any misalignment between NES UE and NW, if NES UE has SIB1 request configuration of a cell, NES UE needs to acquire/re-acquire MIB.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed.

# 4	FFSs in running CRs

Regarding:

5.2.1

Editor’s note: FFS if anything is needed for OD-SIB1

Q1: Is there a need to add text for OD-SIB1 in Section 5.2.1 or can the EN be removed?

Our view that it should be mentioned that SIB1 can be transmitted on demand.

SIBxx

Editor’s note:

FFS: value for maxNrofODSIB1, maxPCI, od-sib1-windowStartOffset

Q7: Suggest values for these parameters or indicate if another WG needs to be liaised?

Our view that RAN1 should decide od-sib1-windowStartOffset. For maxNrofODSIB1 and maxPCI, RAN2 to consider the maximum size of SIB in NR cannot exceed 2976 bits. Hence, the maxNrofODSIB1 can be determined based on maximum size of OD-SIB1config per cell and the maximum size of 2976 bits. For example: maxNrofODSIB1 = floor (2976 /maximum size of OD-SIB1 config per cell).

Observation 4: The maximum number of OD_SIB1 that can be provided is at least limited by maximum SIB size.

In the current draft CR it is assumed that each SIB1 for each frequency can have every field as different. It might make sense to consider that apart from PCI quite many fields could have same values. Either one could for each frequency just provide delta of SIB1 configurations or with simple approach one provides list of PCIS sharing same “SIB1”. Likely it is quite difficult to have common understanding that which fields can be shared as it depends on the deployments so much so in the end likely one needs to have maximum flexibility thus the approach rapporteur has taken seems at least very safe but also resource demanding.

Proposal 2: Discuss whether it makes sense to just to provide delta SIB1 for cells of same (or even different frequency) in the OD-SIB1-Config

Simplest way to reduce but not with much benefit is to have another level of IEs that contain just frequency and PCI and nested IE contains SIB1 content. This way still maximum flexibility is provided but reduced overhead although not much reduced.

Proposal 3: Consider having one nested level of IEs with frequency/PCI (see TP) and if one should be able to provide delta configurations between different OD-SIB1 configs in order to avoid overhead

# 6	Conclusions

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed.

Observation 4: The maximum number of OD_SIB1 that can be provided is at least limited by maximum SIB size.

Proposal 2: Discuss whether it makes sense to just to provide delta SIB1 for cells of same (or even different frequency) in the OD-SIB1-Config

Proposal 3: Consider having one nested level of IEs with frequency/PCI (see TP)

# Annex A: TP for camping information provision in SIB1 request configuration

#### –	SIBxx

SIBxx contains information to assist OD-SIB1 acquisition from serving cell or neighbour cells.

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-ConfigList-r19         SEQUENCE (SIZE(1..maxFREQ-r19))  OF OD-SIB1-Config-r19

lateNonCriticalExtension       OCTET STRING                                                     OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellIdList-r19                       SEQUENCE (SIZE(1..maxPCI-r19)) OF PhysCellId,

od-SIB1-Config-r19                       OD-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

OD-SIB1-Config-r19  ::=       SEQUENCE {

sib1-rsrp-ThresholdSSB-r19         RSRP-Range                                                                            OPTIONAL, -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                                 INTEGER (0..837),

l139                                 INTEGER (0..137)

}                                                                                                                        OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}       OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                          OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}            OPTIONAL, -- Need R

offsetToCarrier-r19                      INTEGER (0..2199),

carrierBandwidth-r19                     INTEGER (1..maxNrofPhysicalResourceBlocks)                                      OPTIONAL, -- Need R

locationAndBandwidth-r19                 INTEGER (0..37949)                                                              OPTIONAL, -- Need R

absoluteFrequencyPointA-r19              ARFCN-ValueNR                                                                   OPTIONAL, -- Cond FDD

offsetToPointA-r19                       INTEGER (0..2199)                                                               OPTIONAL, -- Cond TDD

frequencyBandList-r19                    MultiFrequencyBandListNR-SIB                                                OPTIONAL, -- Need R

ul-FrequencyBandList-r19                 MultiFrequencyBandListNR-SIB                                                OPTIONAL, -- Need R

ul-SubCarrierSpacing-r19                 ENUMERATED {kHz15, kHz30, kHz60, kHz120,kHz240, kHz480-v1700, kHz960-v1700} OPTIONAL, -- Need R

p-Max-r19                                P-Max                                                                       OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                   INTEGER (-60..50)                                                           OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                 SEQUENCE {

inOneGroup                             BIT STRING (SIZE (8)),

groupPresence                          BIT STRING (SIZE (8))                                                      OPTIONAL  -- Cond FR2-Only

},

ssb-Periodicity-r19                         ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare1, spare2}           OPTIONAL, -- Need R

od-sib1-WindowDuration-r19                  ENUMERATED {ms20, ms40, ms80, ms160, ms320, spare1, spare2, spare3}       OPTIONAL, -- Need R

od-sib1-windowStartOffset-r19               INTEGER (1..FFS),

k-ssb-r19                                   INTEGER (1..23),

sib1-RequestConfig-r19                      SIB1-RequestConfig-r19

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                  SEQUENCE {

ssb-perRACH-Occasion-r19                  ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}           OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SIB1-RequestResources-r19,

sib1-pdcch-RestrictionToPRACH-r19         ENUMERATED {true, false},

rach-ConfigSIB1-r19                       RACH-ConfigGeneric

controlResourceSetZero-r19                ControlResourceSetZero,

searchSpaceZero-r19                       SearchSpaceZero,

ra-SearchSpace-r19                        SearchSpaceId                                                           OPTIONAL,   -- Need S

n-TimingAdvanceOffset-r19                 ENUMERATED { n0, n25600, n39936, spare1 }                               OPTIONAL    -- Need R

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)                                                      OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)                                                      OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

SIBxx field descriptions

od-SIB1-ConfigList Provides a configuration to request SIB1 for serving cell and neighbor cells.

OD-SIB1-Config field descriptions

absoluteFrequencyPointA Absolute frequency of the reference resource block (Common RB 0). Its lowest subcarrier is also known as Point A (see TS 38.211 [16], clause 4.4.4.2).

carrierBandwidth Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier) (see TS 38.211 [16], clause 4.4.2). For the case that 12PRB/20 PRB transmission bandwidth is used as specified in TS 38.101-1 [15], TS 38.211[16] and TS 38.213[13], the network shall configure the carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB case) respectively and the UE shall use 12PRB/20 PRB as the transmission bandwidth respectively.

carrierFreq Identifies the ARFCN for the SSB of the cell for which this configuration is valid.

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs.

k-ssb Indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block N_CRB^SSB to the lowest-numbered subcarrier of the SS/PBCH block) of the cell indicated by physCellId in IE OD-SIB1-CellConfig for OD-SIB1 transmission, see TS 38.213 [13], clause 23. For FR2, network does not configure values larger than 11.

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

msg1-SubcarrierSpacing Subcarrier spacing of PRACH (see TS 38.211 [16], clause 5.3.2) used for SIB1 request transmission.

od-sib1-WindowDuration Indicate the duration of the time window of type 0 PDCCH monitoring occasions for SIB1 after transmission of SIB1 request.

od-sib1-windowStartOffset Indicate the time offset between the reference time point and the starting time for the time window of on-demand SIB1.

offsetToCarrier Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1. See TS 38.211 [16], clause 4.4.2.

offsetToPointA Represents the offset to Point A as defined in TS 38.211 [16], clause 4.4.4.2.

p-Max Value in dBm applicable for the cell. If absent the UE applies the maximum power according to TS 38.101-1[57] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]. This field is ignored by IAB-MT, the IAB-MT applies output power and emissions requirements, as specified in TS 38.174 [63].

physCellIdList Identifies the physical cell Id(s) for which this configuration is applied.

prach-RootSequenceIndex PRACH root sequence index (see TS 38.211 [16], clause 6.3.3.1) to be used for SIB1 request transmission.

sib1-rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt to transmit SIB1 request, see TS 38.321 [3].

sib1-tdd-UL-DL-ConfigurationCommon A cell-specific TDD UL/DL configuration, see TS 38.213 [13], clause 11.1.

sib1-restrictedSetConfig Configuration of an unrestricted set or one of two types of restricted sets, see TS 38.211 [16], clause 6.3.3.1.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [13], clause 7.

ssb-Periodicity The SSB periodicity in ms for at least the purpose of RO validation determination for SIB1 request transmission.

ssb-PositionsInBurst Time domain positions of the transmitted SS-blocks in an SS-burst as defined in TS 38.213 [13], clause 4.1.

ul-SubCarrierSpacing Indicates the reference SCS for “msg1-FrequencyStart-r19”, “offsetToCarrier-r19”, “CarrierBandwidth-r19“, and “locationAndBandwidth-r19“.

ul-FrequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for UL operation as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field.

SIB1-RequestConfig field descriptions

controlResourceSetZero Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213, clause 13.

n-TimingAdvanceOffset The N_TA-Offset to be applied for random access on this cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this cell. See TS 38.133, table 7.1.2-2.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1).

searchSpaceZero Parameters of the common SearchSpace#0, see TS 38.213 [13], clause 13.

sib1-pdcch-RestrictionToPRACH Indicates whether the UE can assumes that, in the OD-SIB1 window, PDCCH for an OD-SIB1 message is transmitted in PDCCH monitoring occasions corresponding only to the SSB associated with the PRACH for SIB1 request.

sib1-RequestPeriod Periodicity of the SIB1-Request configuration in number of association periods.

sib1-RequestResources Resources for SIB1 request

ssb-perRACH-Occasion Number of SSBs per RACH occasion for OD-SIB1

totalNumberOfRA-Preambles Total number of preambles used for contention based and contention free 4-step or 2-step random access in the RACH resources configured in the corresponding OD-SIB1-Config, excluding preambles used for other purposes (e.g. for SI request, or SIB1 request).

SIB1-RequestResources field descriptions

sib1-ra-AssociationPeriodIndex Index of the association period in the sib1-RequestPeriod in which the UE can send the SIB1 request for SIB using the preambles indicated by sib1-ra-PreambleStartIndex and rach occasions indicated by sib1-ra-ssb-OccasionMaskIndex.

sib1-ra-PreambleStartIndex If N SSBs are associated with a RACH occasion, where N > = 1, for the i-th SSB (i=0, …, N-1) the preamble with preamble index = sib1-ra-PreambleStartIndex + i is used for SIB1 request; For N < 1, the preamble with preamble index = sib1-ra-PreambleStartIndex is used for SIB1 request.

sib1-ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3].

| Conditional Presence | Explanation |
| --- | --- |
| FR2-Only | This field is mandatory present for an FR2 carrier frequency. It is absent otherwise and UE releases any configured value. |
| TDD | The field is optionally present, Need R, for TDD cells; otherwise it is absent. |
| FDD | The field is optionally present, Need R, for FDD cells; otherwise it is absent. |

Editor’s note:

FFS: value for od-sib1-windowStartOffset(up to RAN1)

FFS: optionality of the parameters