3GPP TSG RAN WG2 Meeting #131	 R2-2506103

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.1.3

Source:	Futurewei

Title:	Discussion on Data Collection for NW-side Model Training

Document for:	Discussion

# Introduction

In meeting #130[10], the discussions focused on the RRC framework, handling of data retention during handover (HO), reporting of buffer status, time information in logged data, and dynamic activation.

Detailed aspects included:

- RRC-7/18/35 NW control on retaining logged data at HO

- RRC-19/20/29 Reporting assistance info on logged measurements

- RRC-21 Time related content of logged data

- RRC-22 RAN1 involvement for logged data for NW-side and UE-side data collection

- RRC-23 Cell ID in logged data

- RRC-24 Data collection configuration

- RRC-25/30 Dynamic activation/deactivation of data collection for logging

- RRC-26 Multiplexing legacy logged data and AIML logged data in new SRB

- RRC-27 Logging periodicity

The achieved agreements are on the following aspects.

- Capturing data logging in RRC specifications as a starting point.

- Introducing a 1-bit indication in RRCReconfiguration with sync for data retention during HO.

- Setting the buffer threshold based on specific size (KB) instead of percentage.

- Indicating the Cell ID (CGI or PCI-ARFCN fallback) in logged data.

- Indicating a "gap" in logged data.

The detailed agreements are listed below.

Agreements

- As a starting point, the data logging is captured in RRC specs.

- (RRC-18) 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration with synch is introduced.  In case of HO, the source sends the 1-bit indication to target cell in HO preparation message. This 1-bit indication is included in HO command by target cell (if the target cell wants to keep the data).   We should have single UE behaviour, when it receives the indication it keeps it, otherwise it removes it.   Notify RAN3

- Buffer threshold to trigger data availability indication should be set based on specific size, e.g., KB instead of percentage

- UAI related to buffer status or low power state is triggered only once when specific conditions are met (e.g., buffer full/threshold, and low power state). A prohibit timer is not necessary for UAI related to buffer status or low power state

- No additional signaling from the UE is required when the low power issue is resolved

- No additional signaling from the UE is required when the buffer full issue is resolved

- Data is collected on per data logging configuration basis and UE indicates data logging configuration ID.    An indication of the “gap” is needed.  “Gap” is time interval larger than the configured logging periodicity.    FFS if timestamp and relative time stamp for each group is needed per “group”.

- The UE should report the CGI of the serving cell whenever feasible. If CGI is unavailable, the UE shall log PCI-ARFCN as a fallback.

Two post-meeting email discussions were scheduled during the meeting.

[POST130][031][AI PHY] NW side data collection (Ericsson/ZTE)

- Intended outcome: provide two TP(s) for data logging and configuration in RRC on how to capture this in a simple way to RAN2.  Discuss impacts to RAN1 for each solution and RAN3.

- Deadline:  long

[POST130][034][AI PHY] LS to RAN3 (Nokia)

- Intended outcome: write LS to RAN3 to provide agreements from this meeting that impact RAN3 (e.g. NW sided data collection HO indication,  etc).

- Deadline:  short

In this contribution, our discussion will focus on the mechanisms and principles identified for data collection for network side model training.

Background information of this discussion can be found in Annex: Background.

# Discussion

## Data Logging

### On-Demand Data Logging

In RAN2 meeting #125bis [3], the group reached the following agreements for Data Collection for NW-side Model Training.

- RAN2 supports enhancements to MDT for data collection framework for training.

  - FSS Whether to enhance logged or immediate MDT.

In RAN2 meeting #126 [4], the group agreed to take immediate MDT as the baseline for OAM-centric data collection (see the agreement below).

- Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

In RAN2 meeting #127 [5], the group further agreed the following.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE.

- FFS on event-based data collection/logging.

In RAN2 meeting #127bis [6], the group has agreed the following on data logging.

- Periodic logging is supported for training data collection procedure in R19.

- Event-triggered data logging will be supported.

  - At least radio condition based event triggered logging will be supported.

  - FFS the details of radio condition based event.

  - FFS if other events are supported.

Since RAN2 meeting #128 [7],  no agreements on data logging mechanism was made. Agreement achieved so far can be summarized in the following table.

| Mechanism | Periodic | On-Demand | Event-Triggered |
| --- | --- | --- | --- |
| Data Logging | Yes |  | Yes |

As we can see from the table, the support of on-demand logging has not been determined. It is our understanding that on-demand data logging can be useful in some situations. For example, during the training, we may find it necessary to collect data for a specific configuration or a specific time frame during which that has not been collected before. In this example, the task won’t be event-based, and it may not need to be collected periodically. In this case the on-demand approach may be the most suitable one. We therefore think on-demand data logging should be supported.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

### Data Logging after Event-triggered Logging

In the post-meeting email discussion after meeting #128 ([POST128][019][AI PHY] NW side data collection), Question #6 asks the following,

- Question 6: Should periodical logging after a measurement event is triggered be supported?

It is our understanding that after a measurement event is triggered, in most cases, the measurement should continuously be done as long as the triggering event conditions are met, for example, L1/L3 measured levels continuously higher/lower than certain preset values. When the measured values go back to normal, the logging will stop. However, there may also be events in which the UE will not change back to its original state before the event was triggered. For example, in beam based events,  a beam may change to become the top-1 beam and never change back again to non-top-1 beam. In this case, the event is a one-time event and the periodic measurement after the event may not have a stopping trigger. It is necessary to define a stopping trigger for these one-time events after being triggered, for example, the duration of measurement after being triggered.

Proposal 2: Periodic measurement logging after a one-time triggering event should have a predetermined stopping trigger.

## Data reporting

### Event/trigger-based reporting

In RAN2 meeting #126 [4], one of the agreements we achieved was the following, on reporting mechanisms.

Enhance the immediate MDT framework to support periodical reporting.

- FFS whether and what event-based reporting is supported, and

- FFS on network request reporting.

In RAN2 meeting #127[5], we further achieved the following agreement, on the support of on-demand reporting.

- On-demand request from the network is supported.   FFS details on signaling.

Therefore, up to meeting #127, we have agreed to support both enhanced periodical and on-demand (network request) reporting mechanisms. Meanwhile, the support of event-based reporting is still FFS, as agreed below.

- FFS whether AS buffer event-based reporting is supported.

In RAN2 meeting #127bis [6], the group has agreed the following on data logging.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

During RAN2 meeting #128 [7],  no new agreements on data reporting mechanism was made. As a summary, after Meeting #128, the supported methods for logged data reporting are shown in the following table.

|  | Periodic | On-Demand | Event/Trigger-based |
| --- | --- | --- | --- |
| Logged Data Reporting | No | Yes | undecided |

From the table, we can see that whether event-based/triggered reporting is supported has not been decided.

However, the TR [2]  clearly says event/trigger-based reporting approach is part of the “general data collections principles”, as shown below.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

In addition, if we looked at the data collection methods identified in R18 (Table 7.3.1.2-1 in Section 2 of this TDoc), we can see event-based reporting mechanisms is identified as one of the reporting types. For example, event-triggered reporting is identified for

- Immediate MDT: collected by TEC/OAM

- L3 measurement: collected by gNB

- LPP: collected by LMF

In fact, for some use cases, such as positioning, event-triggered reporting can be more efficient and up-to-date, because measurements only needs to be done when the UE starts to move, which saves the traffic generated by periodical reporting. In addition, allowing event-based reporting provides flexibility of control and reporting.

Based on this consideration, we propose to support event-based reporting mechanisms.

Proposal 3: Support event/trigger-based reporting mechanisms in addition to on-demand request reporting for NW-side data collection.

## UE capability/capacity for data processing and storage

In the TR [2], the “general data collections principles” include principles regarding UE’s capability and capacity for data collection for NW-side models.

A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

We consider this a practical concern because the resources at the UE side are limited, for example, the computational power and memory (the collected data will be in the memory for the BM and Positioning use cases, unlike some of the application-level use cases). We believe it is necessary to provide a mechanism to report a UE’s capability/capacity for storing measured data temporarily for NW-side model training. Logged MDT may only be used when the UE has enough available resources to support it. Otherwise, immediate MDT should be used for data collection.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

## Availability indication

In the last meeting (RAN2 meeting #129bis [9]), there was a discussion on availability indication (triggering). The discussion centered on how and when UEs should trigger data availability indications, particularly regarding buffer status, and whether this should be UE-driven or network-controlled. There was a split between UE-driven and network-controlled approaches for triggering data availability. Some companies suggested network-configured or estimation-based solutions, while others favored threshold-based reporting, with flexibility to include “full” buffer scenarios. At the end, the group reached the following agreement.

Agreements

- Availability indication can be triggered due to:

  - Full buffer being reached (if configured)

  - Buffer threshold being reached (if configured).

  - Low power (if configured)

During the last meeting, it has been discussed what could be the candidate for indication. Some prefer using the buffer size (e.g., remaining buffer size). We think while the absolute buffer size (e.g., in MBs) can be a clear indication of the available storage space, it is not as good an indicator comparing as the percentage of the buffer used. With a single percentage number, the NW has a better understanding of the situation at the UE. For example, whether the buffer is close to full, already full, or above/under the threshold. It also normalizes among UEs with different sizes of buffers. For this reason, we prefer using the percentage of buffer size as availability indication.

Proposal 5: Use percentage of the buffer size as the indication of buffer availability for NW-side data collection.

# Conclusions

In this contribution, we continued to present our observations and views on topics related to NW-side data collection. Based on the discussions in the previous sections, our proposals are as follows.

Proposal 1: Support on-demand data logging and enhancement to logged MDT for data collection for NW-side models.

Proposal 2: Periodic measurement logging after a one-time triggering event should have a predetermined stopping trigger.

Proposal 3: Support event/trigger-based reporting mechanisms in addition to on-demand request reporting for NW-side data collection.

Proposal 4: Design a mechanism for a UE to report its capability and/or capacity to the NW, such as computational power, available memory and storage space. NW decides on whether immediate MDT or logged MDT to be used based on the UE’s capability and capacity.

Proposal 5: Use percentage of the buffer size as the indication of buffer availability for NW-side data collection.

# Annex: Background

In the TR [2], Table 7.3.1.2-1 lists existing data collection mechanisms available in current RAN specifications for the UE to report measurements to another entity acting as termination point for this data. As indicated by the TR, the analysis/selection of the data collection frameworks should focus on the RRC CONNECTED state for both data generation and reporting.

Table 7.3.1.2-1. Existing data collection methods identified.

| Involved network entity (termination point) | RRC state to generate data | Max payload size per reporting* | Contents to be collected | - End-to-End report latency** | Report type | Security and Privacy |
| --- | --- | --- | --- | --- | --- | --- |
| Method: Logged MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | IDLE / INACTIVE | <9kbyte | - L3 cell/beam measurements - location information - sensor information - timing information | - Procedure latency***: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency****: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message Privacy via user consent |
| Method: Immediate MDT |  |  |  |  |  |  |
| TCE/OAM (Data can be utilized by gNB) | CONNECTED | <9kbyte | - L3 cell/beam measurements - location information - sensor information | - Procedure latency: - Report interval: - 120ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and TCE | - Event triggered - Periodic reporting | AS security via RRC message Privacy via user consent |
| Method: L3 measurements |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Report interval: - l20ms~30min for periodic report - TTT for event triggered report - Air interface signaling latency: - 20ms (RRC) | - Event triggered report - Periodic reporting | AS security via RRC message |
| Method: L1 measurement (CSI reporting) |  |  |  |  |  |  |
| gNB | CONNECTED | <1706bit in PUCCH <3840bit in PUSCH | L1 CSI measurement | - Procedure latency: - Report interval: - 4-320 slot for periodic and semi-persistent report - 0-32 slot after reception of DCI for aperiodic report - Air interface signaling latency: - 1 TTI (PUCCH) | - Aperiodic report - Semi-persistent report - Periodic report | No AS security |
| Method: UE Assistance Information (UAI) |  |  |  |  |  |  |
| gNB | CONNECTED | <9kbyte | Assistance information to show UE preference | - Procedure latency: - Upon generation of UE's preference - Air interface signaling latency: - ~20ms (RRC) | Up to UE implementation when to report | AS security via RRC message |
| Method: Early measurements |  |  |  |  |  |  |
| gNB | IDLE / INACTIVE | <9kbyte | L3 cell/beam measurements | - Procedure latency: - Latency to enter CONNECTED state - Latency to receive gNB request signaling (~20ms) - Air interface signaling latency: - ~20ms (RRC) | Upon gNB request after entering RRC_CONNECTED | AS security via RRC message |
| Method: LPP |  |  |  |  |  |  |
| LMF | CONNECTED | <9kbyte | Location information | - Procedure latency: - Latency to get upper layer trigger (for UE triggered) - Or latency to receive network request message (~20ms) - Air interface signaling latency: - ~20ms (RRC) - Other latency: - Forwarding latency between gNB and LMF | - UE-triggered - Network-triggered | AS security via RRC message |

* The payload size doesn't consider signalling overhead.
** The End-to-End report latency is the latency from availability of the measurement report at the UE side to the availability of the measurement report at the terminated network entity. The time to generate data or perform measurements depends on RAN1/RAN4 specification.
*** Procedure latency is the latency caused by procedures, including procedure to ready for reporting (e.g., entering CONNECTED state, report interval).
**** Air interface signalling latency is the latency to transmit one report, e.g., RRC signalling latency or PUCCH signalling latency.

The TR [2] also captured the following considerations for network-side data collection (Section 7.2.1.3.1).

In addition, A set of general data collection principles is expected to be considered for network-side model training. These include:

- UE to support data logging,

- UE to report the collected data periodically, event-based, and on-demand,

- The UE memory, processing power, energy consumption, signalling overhead should be considered.

Furthermore, and regarding the use cases in this study, the following is considered.

For CSI and beam management use cases, the training of network-side models can consider both gNB and OAM-centric data collection mechanisms.

- The gNB-centric data collection implies that the gNB can configure the UE to initiate/terminate the data collection procedure. The potential impact of L3 signalling for the reporting of collected data should be assessed.

- On the other hand, OAM-centric data collection implies that the OAM provides the configuration (via the gNB) needed for the UE to initiate/terminate the data collection procedure. MDT framework can be considered to achieve this. The potential impact on MDT for RRC_CONNECTED state should be assessed.

For positioning use cases, when considering LMF-side inference, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF. While for LMF-side performance monitoring, it is assumed that the LPP protocol should be applied to the data collected by UE and terminated at LMF, while the NRPPa protocol should be applied to the data collected by gNB and terminated at LMF.

# References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024

Chair’s notes, R2_128_ChairNotes_24-11-22_17-00_eom, RAN2 meeting #128, Orlando, Nov 2024

Chair’s notes, R2_129_ChairNotes_25-02-21_final.docx, RAN2 meeting #129, Athens, Feb 2025

Chair’s notes, R2_129b_ChairNotes_25-04-11_16-45 final, RAN2 meeting #129bis, Wuhan, Apr 2025

Chair’s notes, R2_130_Chair_minutes_25-05-23_final, RAN2 meeting #130, Malta, May 2025