3GPP TSG-RAN WG2 Meeting #131	R2-2506104

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.1.3

Source:	Nokia

Title:	Data collection for network side models

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the remaining open issues related to data collection for training of network side models for beam management use case. The significant open issues that cannot be addressed in [POST130][026][AI PHY] email discussion.

# 2	Discussion

## 2.1	Open issues

### 2.1.1 RRC-19: Reporting assistance information related to logged measurements

Issue description: It is not yet clear what the otherConfig should contain to enable the UE to report assistance information via UAI, related to logging of radio measurements. For instance, should the low power, buffer full, and buffer threshold reached indications be all configured with a single bit, or should the configurations be separated? This issue is related to the RAN2#129bis agreement: Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined. The issue is captured as an editor’s notes in the running CR, in clause 5.3.5.9, 5.7.4.3, 6.2.2, and 6.3.4. Based on the comments in the RRC running CR, some companies have a different view on the solution: - Each of the three indications (full buffer being reached, buffer threshold being reached, and low power) is configured separately; or - Full buffer reporting is enabled by default whenever LoggedDataCollectionAssistanceConfig is configured (no additional configuration bit is needed) and only buffer threshold being reached and low power indications are configured each separately.

If UE supports generic data collection for network side models, then UE requires to support reporting of assistance information related to logged measurement via UAI which can be configured by the network in OtherConfig. There are three indications associated with assistance information that needs to be configured, namely, ‘low power’, ‘full buffer being reached’, and ‘buffer threshold being reached’. In the current running CR of TS 38.331, these indications are configured separately by the network and each indication can trigger UAI transmission separately. Moreover, if one of the indications is being configured, UE can still set the flag of other indications even if those others are not being configured.

In our view, if UE supports generic data collection for network side models, then UE should mandatorily support of reporting of assistance information related to logged measurement. This is to ensure the efficient retrieval of the logged data by the network and UE to continue the logging operations. Thus, network needs to configure UE with at least one indication. However, ‘full buffer being reached’ and ‘buffer threshold being reached’ indications are complementary to each other as both indications are related to UE’s buffer status, while ‘low power’ and ‘full buffer being reached’/‘buffer threshold being reached’ are two different conditions of UE which can be triggered independently. For instance, if network configured only ‘low power’ indication and not and ‘full buffer being reached’/‘buffer threshold being reached’, then network cannot estimate the condition of the UE buffer. Therefore, we think that ‘full buffer being reached’ and ‘low power’ should be configured by default. On the other hand, ‘buffer threshold being reached’ can be optionally configured by the network.

Proposal 1: (RRC-19) RAN2 to confirm ‘low power’ indication and ‘full buffer being reached’ to be enabled by default.

Proposal 2: (RRC-19) RAN2 to confirm UE can be optionally configured with ‘buffer threshold being reached’.

Subject to confirmation of the above proposals, we suggest the following modifications in the CR 38.331 of clause 5.7.4.2 and 5.7.4.3, respectively:

#### 5.7.4.2 Initiation <text omitted> Upon initiating the procedure, the UE shall: <text omitted> 1> if configured to provide assistance information related to logging of measurements for network data collection based on loggedDataCollectionPowerLow included in loggedDataCollectionAssistanceConfig: 2> if the UE determines that it has entered a low power state; or 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide assistance information related to low power state for logging of measurements for netwoprk data collection; 1> if configured to provide assistance information related to logging of measurements for network data collection based on loggedDataCollectionFullBuffer included in loggedDataCollectionAssistanceConfig: 2> if the buffer reserved for the logging of radio measurements for network data collection has become full; or 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide assistance information related to full buffer state for logging of measurements for netwoprk data collection; 1> if configured to provide assistance information related to logging of measurements for network data collection based on loggedDataCollectionBufferThreshold included in loggedDataCollectionAssistanceConfig: 2> if loggedDataCollectionBufferThreshold included in loggedDataCollectionAssistanceConfig and if the amount of logged data related to radio measurements for network data collection has become equal to or above the loggedDataCollectionBufferThreshold: 3> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide assistance information related to exceeding the buffer threshold for logging of measurements for network data collection. NOTE: It is up to UE implementation how to determine a low power state and whether the buffer threshold is reached or if the buffer is full.

#### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message The UE shall set the contents of the UEAssistanceInformation message as follows: <text omitted> 1> if transmission of the UEAssistanceInformation message is initiated to provide assistance information related to logging of measurements for network data collection according to 5.7.4.2: 2> if the UE determines to be in low power state: 3> set lowPowerState to true; 2> if the buffer reserved for the logging of L1 radio measurements is full: 3> set bufferStatus to full; 2> else if loggedDataCollectionBufferThreshold included in loggedDataCollectionAssistanceConfig and if the amount of logged data related to L1 radio measurements logging is equal to or above the loggedDataCollectionBufferThreshold: 3> set bufferStatus to aboveThreshold; Editor's Note: FFS the encoding of the data availability indication/UAI and the cause value. Editor's Note: FFS further changes on individual UAI triggers for availability indication and content of availability indication in UAI based on the triggers, taking into account RAN2 progress.

Proposal 3: (RRC-19) Adapt the above procedures for reporting of assistance information related to logged measurements in TS 38.331.

### 2.1.2	RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state is sent, and what cause should be included then

Issue description: Whether data availability indication should be sent when the UE has data and low power state is sent and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

When UE is in low power state, RAN2 agreed to indicate a low power state flag to the network. The issue above states that UE may have some data in the buffer and the buffer limits have not reached a threshold or full size. This data might be useful for the network. Now, the question is whether the low power state indication to be acted as an implicit indication of data availability or not. In our opinion, the UE may assume a threshold equal to one sample of data which can be indicated via the buffer threshold has been exceeded. This would allow network to know that UE has low power and the data is available. Therefore, a separate data availability cause might not be required along with low power state.

Proposal 4: (RRC-29) If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

### 2.1.3	RRC-31: The release of logged AIML data in UE’s buffer

Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB

The first two examples “power off or deregistration” were already implicitly covered when at RAN2#129bis it was agreed:

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

Therefore, there is no need to consider the cases of power off or deregistration.

Observation 1: (RRC-31) Supporting the retention of logged data for NW-side data collection after power off or deregistration is not needed.

The example “48 hours after the release of logged measurement configuration” presents significant challenges because they require the gNB to retain a configuration for a UE which has disconnected for up to 48 hours and they require any gNB in the NW to be able to forward data to a source gNB which may be 10s to 100s of hops away. The connected mode logged measurement mechanism differs from logged MDT: the measurement logs in logged MDT contain enough context to understand the measurements.

Observation 2: (RRC-31) Supporting the retention of logged data for NW-side data collection for 48 hours after the release of a logged measurement configuration is infeasible in practice in the NW.

Proposal 5: (RRC-31) RAN2 confirms that logged data is released as agreed earlier and there is no timing related requirement on data retention.

The last example, which clears data associated with a specific configuration from the log through an explicit indication from the serving gNB would be feasible. The simplest method would be to clear data from the log when a configuration for logging is released and the flag for data retention after handover is not set or when a configuration for logging is modified, if we decide to allow the logging configurations to be modified. The gNB should not take an action that deletes samples prior to retrieving them.

Observation 3: (RRC-31) The only clear time when a gNB would want data to be released is after the data has been transmitted to the gNB or when the gNB releases or modifies a configuration prior to retrieving the data.

Proposal 6: (RRC-31) Data from the UE buffer will be released in the following scenarios:

- when data has been successfully transmitted using a UEInformationResponse message;

- the configuration for logging has been released and the flag to retain measurements after handover has not been set;

- the configuration for logging has been modified;

- All logged measurement configuration and the logged data shall be removed by the UE at switch off or detach.

### 2.1.4	RRC-36: How data is forwarded to OAM or source gNB after HO

Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed.

We sent an LS to RAN3 [POST130][034][AI PHY] and cc’d to SA5 with the following information:

RAN2 would like to clarify that the data collection configuration could originate in the source gNB or from OAM. RAN2 would also like to clarify that the purpose of data availability indication in RRCReconfigurationComplete message sent during handover is for the target gNB to be able to fetch the collected data from the UE.

From that post email discussion we summarize the behaviour of the target node and the impacts on the other WGs as follows:

|  | Agnostic to gNB centric or OAM centric approach | gNB-centric approach | OAM-centric approach |
| --- | --- | --- | --- |
| Behaviour of target gNB after retrieving the logged data from UE | Target gNB can forward the data to source gNB indicated by CGI or PCI-ARFCN | Target gNB can forward the data to source gNB indicated by CGI or PCI-ARFCN | Target gNB can forward the data to OAM or TCE directly indicated by FFS:TCE-ID |
| Impacted WGs | RAN3, SA5 | RAN3 | SA5 |

From this table, it is clear that the target gNB can decide whether to forward to OAM or source gNB and this is out of RAN2 scope. The following proposal is provided to resolve this issue:

Proposal 7: (RRC-36) RAN2 assumes that data can be forwarded from the target node, but the actual solution is not in the scope of RAN2.

### 2.1.5 RRC-39: The naming of IEs related to NW-side data collection

Issue description: The naming of IEs related to NW-side data collection can be revisited. We will have more CSI-based and other use cases, each with different measurement logs and we should minimize the risk of having to replicate the NW-side DC procedures and ASN.1 for every new use case. One solution direction is to make the naming of procedures and ASN.1 related to NW-side DC generic and enclose use case specific IEs / data logs within so that our framework can support future use cases. An alternative is to name the IEs based on the use case, which would help to write the procedural text in a simpler way, while being clear. This issue refers to the following field/IE and variable names: - The name of the variable to store logged data. This can be use case specific (i.e. VarCSI-LogMeasReport as proposed by the rapporteur in the draft RRC running CR) and a new variable would be added in the future for e.g. the AIML mobility use case, or it can be changed to have a more generic name and include lists of logged data for different use cases. Note that defining a variable per use case still allows the UE to use a shared memory size for all use cases. - The name of the field to request logged data from the UE and the name of the field where the UE provides the report with logged measurements can be specific per use case (as proposed by the rapporteur in the draft RRC running CR, i.e. csi-LogMeasReportReq in UEInformationRequest and csi-LogMeasReport in UEInformationResponse, respectively) or can be more generic. The rapporteur sees the benefit in having use case specific names, since this way the NW can select which logged data to retrieve from the UE and the UE can select that logged data to report it. The name of the field to indicate to the NW that the UE has logged data during handover can be use case specific (i.e. csi-LogMeasAvailable in RRCReconfigurationComplete, as proposed by the rapporteur in the draft RRC running CR) or can be more generic. As in the point above, in the rapporteur’s view it is beneficial to have use case specific naming, so that the NW can distinguish which kind of logged data the UE has. Given the comments above, the rapporteur would like to update the proposals as below: - (RRC-39) The UE stores logged data for BM in a variable specific to L1 CSI related measurements. (RRC-39) RAN2 to discuss whether the name of the fields/IEs to request logged data from the UE and to report logged data or the availability of logged data to the NW are use case specific.

Proposal 19 above is not about the naming of the IEs as the name of the open issue hints, but a technical issue whether the logged data availability and the retrieving of the logged data can be use-case specific or not.

Introducing use-case specific availability indication would require some further considerations. The first issue is whether the threshold provided by the network for availability indication is use-case specific as well or not. If the threshold is still a common threshold indicating the total amount of data in the memory for all use-cases is above the threshold, the meaning of use-case specific availability indication becomes confusing, as it may mean that availability of a single sample for a use-case. If the threshold is use-case specific, then the next issue to be discussed whether the network should provide a single threshold that is applicable for all use-cases or use-case specific thresholds to be provided. As the current assumption is that there is a single shared memory, the use-case specific thresholds will create the potential problem that the memory of the UE may become full before any availability indication is sent if collection of logged data for multiple use-cases is configured in a UE.

Observation 4: (RRC-39) Introduction of use-case specific indication would create further issues to be discussed, such as the interpretation of availability indication threshold and may introduce additional complexity how to avoid the case that the availability indication is not sent before the UE memory is full.

The other part of the proposal is to introduce use-case specific logged reporting request. In this case an issue that needs to be addressed whether the network can send report request to multiple use-cases in a single message. If no, then the specification and implementation is simple, but it will lead multiple requests and responses if the UE is configured to collect data for multiple use-cases, which is not efficient. If the network can request logged data for multiple use-case in a single request, then a similar type of multiplexing is needed as with the non-use-case specific request case.

Observation 5: (RRC-39) Introduction of use-case specific requests for sending logged data would create further issues to be discussed, such as requesting reports for multiple use-cases in a single request, and may lead to extra signalling overhead.

Beyond these issues that should be addressed for use-case specific indications, the motivation of introducing them is unclear. We do not think that there is a scenario when the network is not interested to retrieve all collected data. The order of the collection of data is not an issue, as it has been agreed from the beginning that there is not strict delay retrieving the logged data. The ASN.1 naming structure is not a motivation for this extra complexity. Therefore, we think that there is no motivation to introduce this extra complexity and overhead.

Proposal 8: (RRC-39) The logged data availability indication and the request to send logged data are not use-case specific.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: (RRC-31) Supporting the retention of logged data for NW-side data collection after power off or deregistration is not needed.

Observation 2: (RRC-31) Supporting the retention of logged data for NW-side data collection for 48 hours after the release of a logged measurement configuration is infeasible in practice in the NW.

Observation 3: (RRC-31) The only clear time when a gNB would want data to be released is after the data has been transmitted to the gNB or when the gNB releases or modifies a configuration prior to retrieving the data.

Observation 4: (RRC-39) Introduction of use-case specific indication would create further issues to be discussed, such as the interpretation of availability indication threshold and may introduce additional complexity how to avoid the case that the availability indication is not sent before the UE memory is full.

Observation 5: (RRC-39) Introduction of use-case specific requests for sending logged data would create further issues to be discussed, such as requesting reports for multiple use-cases in a single request, and may lead to extra signalling overhead.

And proposed the following:

Proposal 1: (RRC-19) RAN2 to confirm ‘low power’ indication and ‘full buffer being reached’ to be enabled by default.

Proposal 2: (RRC-19) RAN2 to confirm UE can be optionally configured with ‘buffer threshold being reached’.

Proposal 3: (RRC-19) Adapt the above procedures for reporting of assistance information related to logged measurements in TS 38.331.

Proposal 4: (RRC-29) If the UE is in a low power state AND it has data in its buffer, even below the threshold, UE can still indicate both low power state and buffer threshold met indications. That is, no additional cause or flag is required.

Proposal 5: (RRC-31) RAN2 confirms that logged data is released as agreed earlier and there is no timing related requirement on data retention.

Proposal 6: (RRC-31) Data from the UE buffer will be released in the following scenarios:

- when data has been successfully transmitted using a UEInformationResponse message;

- the configuration for logging has been released and the flag to retain measurements after handover has not been set;

- the configuration for logging has been modified;

- All logged measurement configuration and the logged data shall be removed by the UE at switch off or detach.

Proposal 7: (RRC-36) RAN2 assumes that data can be forwarded from the target node, but the actual solution is not in the scope of RAN2.

Proposal 8: (RRC-39) The logged data availability indication and the request to send logged data are not use-case specific.