3GPP TSG-RAN WG2 #131	R2-2506112

Bangalore, India, August 25th– 29th, 2025

Agenda Item:	8.5.2

Source:	Ericsson

Title:	Discussion on on-demand SSB for NES

Document for:	Discussion, Decision

# 1	Introduction

In RAN#105, the WID for network energy savings in Rel-19 was revised [1]. The first objective concerns the on-demand SSB SCell operation, as shown below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, SCell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Agreements from RAN2#130:

Agreements on OD-SSB

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

- The OD-SSB MAC CE is a variable size MAC CE. For each activated OD-SSB SCell, an additional field is added to indicate the applicable OD-SSB configuration index. TP in R2-2504280 will be baseline for the running CR. Detailed wordings can be further discussed.

- When UE receives MAC CE with Cj bit set to 0 for an SCell for which OD-SSB is currently activated:

• OD-SSB is deactivated for the jth SCell, irrespective of whether od-ssb-nrofBurst is included or not included in configuration applied for the activated OD-SSB. Running timer/counter for implicit deactivation (if any) for the SCell is stopped.

- For implicit deactivation of an OD-SSB transmission after the transmission of N occasions, reuse to SCell deactivation timer behaviour: UE (re)-applies the indicated N value in the MAC CE for each activated OD-SSB transmission upon reception of the OD-SSB Activation/Deactivation MAC CE.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

- RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4.

# 2	Open issue list:

Open issues from [POST130][107][NES] (Ericsson) [3] related to the RRC CR in [2]:

OD-SSB:

Issue 1: Q9

Issue 2: CATT003

Issue 3: Fujitsu006, HW002

## 2.1 value for maxNrofOD-SSB

IE CellGroupConfig

Editor’s note:

FFS value for maxNrofOD-SSB

Q9: Suggest value for this parameter or indicate if another WG needs to be liaised?

In the email discussion, there was one company responding “We think it can be 16 (4bit) or 32 (5bit)”. The corresponding MAC CE has the following format in current running 38.321 CR:

=====================================NEXT CHANGE===================================

# 6	Protocol Data Units, formats and parameters

### 6.1.3	MAC Control Elements (CEs)

#### 6.1.3.x	On-demand SSB Activation/Deactivation MAC CE

The On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size; it includes an Activation/Deactivation bitmap and, in ascending order based on the ServCellIndex, an on-demand SSB configuration index field for each SCell activated in the bitmap. A single octet bitmap is used for Activation/Deactivation of the On-Demand SSB transmission and indicating the presence of an on-demand SSB configuration index per Serving Cell, containing seven C-fields and one R-field. The On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet is defined as follows (Figure 6.1.3.x-1).

The On-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size; it includes an Activation/Deactivation bitmap and, in ascending order based on the ServCellIndex, an on-demand SSB configuration index field for each SCell activated in the bitmap. A four-octet bitmap is used for Activation/Deactivation of the On-Demand SSB transmission and indicating the presence of an on-demand SSB configuration index per Serving Cell, containing 31 C-fields and one R-field. The On-Demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets is defined as follows (Figure 6.1.3.x-2).

-	Ci: If there is an SCell configured for the MAC entity with SCellIndex i as specified in TS 38.331 [5], this field indicates the on-demand SSB activation/deactivation status for the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to 1 to indicate that On-Demand SSB transmission on the SCell with SCellIndex i shall be activated. The Ci field is set to 0 to indicate that On-Demand SSB transmission on the SCell with SCellIndex i shall be deactivated;

-	ConfigIndexi: The on-demand SSB configuration index to apply for SCellIndex i as. This field is present for SCell with SCellIndex i if Ci is 1, and not present if Ci is 0.

-	R: Reserved bit, set to 0.

Figure 6.1.3.x-1: On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet

Figure 6.1.3.x-2: On-demand SSB Activation/Deactivation MAC CE of four Activation/Deactivation octets

Editor’s note: the size of OD-SSB ConfigIndexi is FFS

=====================================NEXT CHANGE===================================

As can be seen, if the OD-SSB ConfigIndex takes less than 4 bits the MAC CE can fit two indexes per octet and the MAC CE can be considerably shorter. However, whether 16 is enough or e.g. 32 should be supported should be decided first.

Agree the max value of maxNrofOD-SSB and if it is less or equal to 4 bits, optimize the size of the MAC CE.

## 2.2 CATT003

| CATT003 | od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.. Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication. According to the conclusion of RAN1, od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE, and RAN2 has not discussed this issue. The od-ssb-nrofBurst is configured in parallel with od-ssb-ActivationStatus in OD-SSB-Config, so od-ssb-nrofBurst should be able to be configured and activated through RRC? We think we need to make a clarification. |
| --- | --- |

It seems there is one parameter that may have presence conditioned on the value of another parameter. Namely, field od-ssb-NrofBursts can be present only when od-ssb-ActivationStatus is indicating not activated and activation happens later via MAC CE.

RAN2 to discuss if the presence condition of field od-ssb-nrofBurst can can be confirmed or if LS is needed to RAN1.

## 2.3 Fujitsu006, HW002

| Fujitsu 006 | – OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact.. [Fujitsu] RAN2 agreed to introduce a new servingCellMO in ServingCellConfig in RAN2#130. Although the current CR may work, we think the original agreement is simpler for implementation, because it can keep the same message structure. We also think even if it is introcuded in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). We would like to add it into open issue list and ask other company’s views. [Apple] We have same view as Fujisu. |
| --- | --- |

| HW002 | Besides, there should be some procedure texts about when the UE uses the new servingCellMO, when the UE uses the legacy servingCellMO: In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation: - • The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation - • The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly. Spec impact can be further discussed in running CR preparation. [Apple] We have same view as Huawei. |
| --- | --- |

However, currently the CR is implemented having the OD-SSB specific servingcell MO association in the corresponding OD-SSB configuration itself. The association is a link to a measurement object configured in the list of MOs in the MeasConfig which is a per UE configuration.

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},                                                    OPTIONAL, -- Need S

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssb-SubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

smtc1 Primary measurement timing configuration (see clause 5.5.2.10) to be used instead of smtc1 configured in servingCellMO in IE servingCellConfig when this OD-SSB is activated and the serving cell is activated

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. The OD-SSB pattern is deactive if this field is absent. Only one OD-SSB pattern can be activated.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated.

od-ssb-Periodicity The SSB periodicity in ms. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1).

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ss-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7.

od-ssbSubcarrierSpacing Indicates subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell when this OD-SSB is activated instead of servingCellMO in IE ServingCellConfig.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is optionally present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |
| ODssbAOssb | The field is mandatory present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |
| IntraFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |

The corresponding RAN2 agreements are:

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

The above agreements could be further clarified for following things. For both Case1 and Case2, is smtc the only parameter that can be different between OD-SSB configurations.

RAN2 to confirm if smtc1 is the only parameter that can be different between OD-SSB configurations for both Case1 and Case2

Further, to confirm, does agreement “RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.” apply to both Case1 and Case2.

RAN2 to confirm agreement “RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.” apply to both Case1 and Case2.

If both Proposal 3 and Proposal 4 can be confirmed, it means that both od-ssb-absoluteFrequency-r19 and the servingCellMO-r19 are configured together and apply to more than one OD-SSB configurations. That is, only when the OD-SSB configuration does not adopt od-ssb-absoluteFrequency-r19 and the servingCellMO-r19 from the servingcell, these are configured for the OD-SSB. Further, it means that only one pair can be configured by servingcell.

The od-ssb-absoluteFrequency-r19 was placed in OD-SSB configuration to follow the RAN1 excel but servingCellMO-r19 and smtc1-r19 were placed there to have those activated automatically when given OD-SSB is activated and thus to avoid separate procedural text. This approach, whether automatic or described in procedural text (that only one servingcellMO is active) is aligned with RAN2 agreement “When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.”.

RAN2 to agree to have the smtc1-r19, od-ssb-absoluteFrequency-r19 and the servingCellMO-r19 in OD-SSBConfig as in the current RRC CR (update conditions to outcome of proposal3 and proposal4)

# 4		Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Agree the max value of maxNrofOD-SSB and if it is less or equal to 4 bits, optimize the size of the MAC CE.

Proposal 2	RAN2 to discuss if the presence condition of field od-ssb-nrofBurst can can be confirmed or if LS is needed to RAN1.

Proposal 3	RAN2 to confirm if smtc1 is the only parameter that can be different between OD-SSB configurations for both Case1 and Case2

Proposal 4	RAN2 to confirm agreement “RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.” apply to both Case1 and Case2.

Proposal 5	RAN2 to agree to have the smtc1-r19, od-ssb-absoluteFrequency-r19 and the servingCellMO-r19 in OD-SSBConfig as in the current RRC CR (update conditions to outcome of proposal3 and proposal4)

# References

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105, October 2024.

R2-2505699, “Introduction of enhancements for network energy efficiency

R2-2505708, “Report of collection of comments to 38.331 CR for NES”

R4-xxxxx, “Discussion on OD-SSB in Rel-19 NES”, RAN4#114 Wuhan, China Ericsson

# Appendix