3GPP TSG-RAN2 Meeting #131	R2-2506127

Bangalore, India  Aug 25th – 29th , 2025

Source: 			ZTE Corporation

Title: 	Discussion on functionality management for measurement event prediction

Agenda item:	8.3.2.2

Document for: 	Discussion and Decision

- Introduction

At the RAN 102 meeting, the SID on AI/ML for mobility was approved, and the measurement event prediction is one of the important objects. During the SI study, we evaluated the prediction performance of the measurement event prediction, and the simulation results showed that the measurement event prediction can bring gain. Although some functionality management related issues were discussed in the past a few meetings, most of them are for RRM measurement prediction. In this contribution, we focus on the measurement event prediction, and share our view on the inference report, applicability report and performance monitoring for UE side measurement event prediction.

- Discussion

2.1 The difference between indirect and direct event prediction

The following agreement was made in RAN2#130 meeting regarding inference configuration,

Agreements - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Companies think whether indirect or direct prediction is used can be up to UE implementation. We agree this is the case for RRM prediction, but for measurement event prediction, it highly depends on the model output. Only if the model inputs are the same for direct and indirect prediction, there is no need to differentiate them.

As discuss during the study, the output of the direct measurement event prediction and indirect measurement event prediction are as follows:

RAN2 127bis: For direct measurement event prediction, the model output is the probability of event occurrence within a time window. RAN2 #128: Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model.

As listed above, the output of indirect measurement event prediction is whether a measurement event at one time instance or one future time instance occurs, while the output of the direct measurement event prediction is the probability of an event occurrence within a window.

Observation 1: According to RAN2 agreements, the model outputs for direct event prediction and indirect event prediction are different.

With different output, the network may have different implementation on how to handle the direct UE models and indirect UE models. For example, with reported probability, the network may have implementations on how to use the reported probability value. Besides, the UE needs to indicate the event will occur within which window, and the network may have different implementation on different window length and different window type (e.g. whether the window starts from current time instance).

Observation 2 Because of the different model outputs, the network implementation will be different in handling the direct UE models and indirect UE models.

Based on the analysis above, we think the network shall be aware of the indirect and indirect measurement event models, unless direct and indirect event prediction have the same output.

Proposal 1: 	The network should be aware of the direct or indirect measurement event models, unless the same model outputs are defined for direct and indirect event prediction.

2.2 Applicability configuration and reporting

In the last meeting, we have agreed to consider option A and Option B like scheme, as follows. In this sub-section, we’d like to discuss the applicability configuration and reporting for option A and option B, separately.

RAN2 130: - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

For Option A-Full inference configuration

Similar to RRM measurement prediction, for option A, we think the existing RRM framework can be reused to provide full inference configuration to the UE with introducing prediction-specific parameters. Upon receiving a list of full inference configuration, the UE can check whether the corresponding inference configuration is applicable or not per measId, and sends the applicability report to the network via RRCReconfigurationComplete or UAI.

Proposal 2: 	For option A in measurement event prediction, the full inference configuration will be provided to the UE. And the existing RRM framework can be reused with introducing prediction-specific parameters.

Proposal 3: 	Upon receiving a list of full inference configuration, the UE determines the applicability per measId and sends applicability report to the network via RRCReconfigurationComplete or UAI

As described above, the model output of direct and indirect measurement event prediction is different and the network shall aware of it. So, we think the network can configure the UE to perform indirect measurement event prediction or direct measurement event prediction, in order to make the UE to report the prediction results preferred/used by the network.

Proposal 4: 	The network can configure the UE to perform indirect measurement event prediction or direct measurement event prediction.

For indirect measurement event prediction, the RRM measurement is performed first to obtain prediction results, and then predicted results and (optional) actual measurement results are used to derive whether the measurement event at one time instance occurs, without further AI/ML involvement. So, the agreed inference parameters for RRM measurement prediction can also apply to indirect measurement event prediction.

In the existing RRM measurement configuration, measurement event is configured via setting reportType to ‘eventTriggered’. Similarly, in the AI-mobility, the network can indicate the UE whether to perform indirect measurement event prediction via providing inference parameters (e.g. PW length, skipping pattern, measured frequency) and setting the corresponding reportType to eventTriggered.

Proposal 5: 	The same inference configuration can be used to indicate RRM measurement prediction or indirect measurement event prediction, e.g. the existing reportType can be reused to indicate whether the UE shall perform indirect measurement event prediction.

### 2.2 Inference Report

For indirect measurement event prediction:

For inference report for indirect measurement event prediction with case B, we can follow legacy measurement report mechanism, i.e. the UE sends inference report at event-triggered time instances, and the inference report includes the latest cell/beam results. This is most simple way, and has no spec impact.

Proposal 6: For indirect measurement event prediction with Case B, the inference report is initiated at predicted event-triggered time, and the inference report includes the latest cell/beam results (regardless of actual results or prediction results).

For inference report for indirect measurement event prediction with case A, since the motivation of case A is to improve handover performance, in this case that the measurement event is predicted in advance, the UE sends the measurement report to the network earlier than the event-triggered time. In order to inform the network of event-triggered time, the UE needs to report it to the network.

In addition, the inference results shall also include future measurement results. There are the following two options can be considered:

- Option 1: The inference results at event-triggered-time;

- Option 2: A list of future measurement results;

In our understanding, option 1 follows legacy rules, and the signaling overhead is smaller compared with option 2. While the drawback is that: the network implementation flexibility will be impacted. For example, with the knowledge of when the measurement event is triggered, the network may move the UE to the target cell before event-triggered time to avoid too late handover. In this case, the target cell needs to know the measurement results of the time instance before event-triggered time, in order to prepare suitable RRM measurement configuration (e.g. RACH configuration). Another benefit for option 2 is that: with multiple future measurement results, the network can know the tendency of the cell quality change. Similar to RRM measurement prediction, we think the latest actual measurement result shall also be included in the measurement report.

Proposal 7: For indirect measurement event prediction with temporal Case A, the inference report includes event-triggered time, multiple future measurement results and actual measurement results.

Since the prediction window is sliding, there are multiple time instance at which the measurement event can be predicted. Considering that the prediction accuracy is improved with the decreased of the prediction window length, the prediction results (e.g. whether the event will be triggered, when the measurement event will be triggered, the prediction results) may be different at different time instance. For example, as shown in the figure below, at time instance 1, the UE predicts that event will be triggered at time instance 6; while at the time instance 2, the UE predicts that event will be triggered at time instance 5. With the help of actual measurement results and shorter prediction distance, the second prediction is more reliable. It is redundant and unnecessary if the UE reports predicted results to the network every time the event is predicted to be triggered.

Figure: An example for measurement event prediction

Observation 3: Regarding temporal domain prediction, since the prediction window is sliding, there are multiple time instances at which the measurement event can be predicted, it is redundant and unnecessary if the UE reports prediction results to the network every time the event is predicted.

In our view, the network can indicate the UE when to report the predicted results. Specifically, the network can configure the UE to report the measurement results upon:

  - The measurement event is predicted to be triggered;

  - The entering condition of the measurement event is met based on the actual measurement;

In this way, the network implementation becomes more flexible. For example, if the network is concerned about the handover failure caused by false measurement event prediction, the network can configure the UE to report predicted results upon the entering condition of the measurement event is met for a time duration based on the actual measurement results. If the network wants to obtain the best AI gain, the network can configure the UE to report predicted results upon the measurement event is predicted to be triggered.

Proposal 8: Regarding measurement event prediction with temporal domain prediction case A, the network can configure when to report prediction results after the measurement event is predicted, e.g. immediately after measurement event is predicted, or entering condition of the measurement event is met based on the actual measurement.

For frequency domain prediction, the output is measurement results at the frequency to be predicted for every time instance. So, the legacy measurement report can be reused.

Proposal 9: For indirect measurement event prediction with frequency domain prediction, the inference report includes the latest measurement results of predicted cell. (No change to existing measurement report).

For direct measurement event prediction:

Since the output of the direct measurement event prediction is likelihood of an event occurrence within a window, the UE needs to report probability value, and the window related information (e.g. when the window starts, when the window ends, the length of the window).

Proposal 10: For indirect measurement event prediction with frequency domain prediction, the inference report includes probability value and the window related information (e.g. when the window starts, when the window ends, the length of the window).

### 2.3 performance monitoring

In the last meeting, we have agreed that the RSRP difference can be used as the performance metric for monitoring in the RRM measurement prediction. Similarly, for indirect measurement event prediction, the RSRP difference can also be considered as monitoring metric.

Proposal 11：For indirect measurement event prediction, RSRP difference can be used as performance metric for indirect measurement event prediction.

In the measurement event prediction simulation, F1 score and system level metric (i.e. HO failure rate, the number of HOF per UE per second) are used as performance metric, However, these metrics can not be used as monitoring metric in the reality. Specifically, for F1 score calculation, the definition for ‘true event prediction’, ‘false event prediction’ and ‘missed event detection’ are needed, as follows:

For indirect prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa; - Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]; - Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]. For direct prediction, the counter n1, n2 and n3 in the formula are defined as following: - Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold; - Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold; - Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold.

The ground-truth event is used to determine whether the event is correctly predicted, However, if the measurement event prediction is performed, the UE may be not able to get the ground-truth event. This is because after receiving reported prediction results, the network may determine to move the UE to another cell before the event is actual triggered. In this case, the UE is unable to know whether the event will be actual triggered or not, and the F1 score calculation is infeasible.

Observation 4: With measurement event prediction, the UE may be not able to get ground-truth event, which is used for F1 score calculation, since the network may move the UE to another Cell before ground-truth event occurs.

For system level metric (e.g. HOF rate) calculation, the handover execution shall be performed several times. In this case, the UE can not perceive the prediction performance change in time, because (1) usually, the UE does not perform handover frequently. This means that it takes a long time for the UE to obtain sufficient data to calculate system level metric calculation; (2) the historical data (e.g. whether the last handover / the second-to-last handover successes or not) may be out-of-dated, which is not helpful to evaluate current prediction performance; (3) Due to the mobility of the UE, the handover is normally performed at different cell(s), whose deployment (e.g. Uma/Umi, LOS/NOS) may be different. This information can not help the current gNB to determine whether the prediction is applicable at current deployment.

Observation 5: System level metric (e.g. HOF rate) is not applicable for measurement event prediction monitoring at the UE side due to time consumption, out-of-dated historical monitor data, and the mobility of UE.

Based on the analysis above, we think F1 score and system level metric (e.g. HOF rate) can not be used as performance metric. For indirect measurement event prediction, we can use RSRP difference to evaluate the prediction performance. Considering the RSRP difference is not available for direct measurement prediction, companies need to clarify how to perform monitoring. If no available performance metric, RAN2 needs to re-consider the feasibility of direct measurement event prediction.

Proposal 12: RAN2 to confirm that F1 score and system level metric (e.g. HOF rate) can not be used as performance metric for both indirect and direct measurement event prediction.

- Conclusion and proposals

Observation 1: According to RAN2 agreements, the model outputs for direct event prediction and indirect event prediction are different.

Observation 2 Because of the different model outputs, the network implementation will be different in handling the direct UE models and indirect UE models.

Observation 3: Regarding temporal domain prediction, since the prediction window is sliding, there are multiple time instances at which the measurement event can be predicted, it is redundant and unnecessary if the UE reports prediction results to the network every time the event is predicted.

Observation 4: With measurement event prediction, the UE may be not able to get ground-truth event, which is used for F1 score calculation, since the network may move the UE to another Cell before ground-truth event occurs.

Observation 5: System level metric (e.g. HOF rate) is not applicable for measurement event prediction monitoring at the UE side due to time consumption, out-of-dated historical monitor data, and the mobility of UE.

Proposal 1: 	The network should be aware of the direct or indirect measurement event models, unless the same model outputs are defined for direct and indirect event prediction.

Proposal 2: 	For option A in measurement event prediction, the full inference configuration will be provided to the UE. And the existing RRM framework can be reused with introducing prediction-specific parameters.

Proposal 3: 	Upon receiving a list of full inference configuration, the UE determines the applicability per measId and sends applicability report to the network via RRCReconfigurationComplete or UAI

Proposal 4: 	The network can configure the UE to perform indirect measurement event prediction or direct measurement event prediction.

Proposal 5: 	The same inference configuration can be used to indicate RRM measurement prediction or indirect measurement event prediction, e.g. the existing reportType can be reused to indicate whether the UE shall perform indirect measurement event prediction.

Proposal 6: For indirect measurement event prediction with Case B, the inference report is initiated at predicted event-triggered time, and the inference report includes the latest cell/beam results (regardless of actual results or prediction results).

Proposal 7: For indirect measurement event prediction with temporal Case A, the inference report includes event-triggered time, multiple future measurement results and actual measurement results.

Proposal 8: Regarding measurement event prediction with temporal domain prediction case A, the network can configure when to report prediction results after the measurement event is predicted, e.g. immediately after measurement event is predicted, or entering condition of the measurement event is met based on the actual measurement.

Proposal 9: For indirect measurement event prediction with frequency domain prediction, the inference report includes the latest measurement results of predicted cell. (No change to existing measurement report).

Proposal 10: For indirect measurement event prediction with frequency domain prediction, the inference report includes probability value and the window related information (e.g. when the window starts, when the window ends, the length of the window).

Proposal 11：For indirect measurement event prediction, RSRP difference can be used as performance metric for indirect measurement event prediction.

Proposal 12: RAN2 to confirm that F1 score and system level metric (e.g. HOF rate) can not be used as performance metric for both indirect and direct measurement event prediction.

- Reference

- Report of RAN2 129 meeting.