3GPP TSG-RAN WG2 Meeting #131                                  R2-2506138

Bangalore, India, Aug 25th – 29th , 2025

Agenda item:	5.1.3.2

Title:	Discussion on R16 mandatory gap capability

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#131 meeting, RAN2 receives an LS [1] from RAN4, in the LS, RAN4 indicates the new agreement regarding the Rel-15 and Rel-16 gap pattern capabilities, and asks RAN2 to update the capability accordingly. The content of the LS is shown below:

# 1. Overall Description: RAN4 agrees with the following clarification in Rel-16 mandatory gaps capability. - When UE reports ‘1’ to support a gap pattern in Rel-15 supportedGapPatterns but reports ‘0’ for the same gap pattern in supportedGapPatterns-NRonly-r16, UE shall support the gap pattern which is reported in Rel-15 supportedGapPatterns..

In this contribution, we discussed this issue and shared our views.

Discussion

The ASN.1 of the two capabilities are

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

According to TS 38.306, the definition of the two capabilities are shown below:

| supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE for NR SA, for NR-DC, for NE-DC and for independent measurement gap configuration on FR2 in (NG)EN-DC. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2. | UE | CY | No | No |
| --- | --- | --- | --- | --- |
| supportedGapPattern-NRonly-r16 Indicates measurement gap pattern(s) optionally supported by the UE for NR SA and NR-DC when the frequencies to be measured within this measurement gap are all NR frequencies. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3 and so on. The UE shall set the bits corresponding to the measurement gap pattern 2, 3 and 11 to 1. | UE | FD | No | No |

The Rel-15 is used to indicate the support of gap pattern 2~23, in Rel-16, based on RAN4 LS [2], RAN4 agreed to mandate some gap patterns (i.e. 2, 3 and 11) for Rel-16 UEs when the UE is configured with NR-only measurements, therefore, a separate capability supportedGapPattern-NRonly-r16 was introduced. Although there is overlapped gap patterns (2~11) indicated by the two capabilities, the Rel-16 capability is limited to NR SA and NR-only measurements.

So, for a given gap pattern, if the UE indicates the support via supportedGapPattern, it means the UE supports the gap pattern when the UE is in NR SA, NR-DC/NE-DC and when the UE is configured with NR measurements and/or E-UTRAN measurements; but if the UE indicates support of the gap pattern via supportedGapPattern-NRonly-r16, it only means the UE supports the gap pattern when the UE is in NR SA and only configured with NR measurements. Therefore, the Rel-15 capability can represent higher capability than the Rel-16 one. In theory, the UE supports supportedGapPattern should also indicate the support of the same gap pattern in supportedGapPattern-NRonly-r16, if signaled.

Observation 1: Rel-15 UE capability supportedGapPattern can represent higher gap capability than Rel-16 UE capability supportedGapPattern-NRonly-r16.

Observation 2: Technically, a UE supporting a gap pattern in Rel-15 supportedGapPattern should also indicate the support of the same gap pattern in Rel-16 supportedGapPattern-NRonly-r16.

In last RAN4 meeting, one company raised the issue that UE may report the capabilities inconsistently, and wanted to clarify the UE behavior, the problematic case is highlighted in below table. (Table cited from RAN4 paper [3])

|  |  | Support for GP2-GP11 |  |  |
| --- | --- | --- | --- | --- |
| supportedGapPatterns | supportedGapPatterns-NRonly-r16 | LTE measurements only | NR FR1 + LTE measurements | NR FR1 measurements only |
| 0 | 0 | No | No | No |
| 0 | 1 | No | No | Yes |
| 1 | 0 | Yes | Yes | Yes |
| 1 | 1 | Yes | Yes | Yes |

The RAN4 paper initially propose to disallow wrong UE implementation, but based on RAN4 discussion, they seem to accept such reporting, and just confirm in that case, the UE is supposed to support the gap pattern according to the Rel-15 capability.

This implies that when the UE is configured with only NR FR1 measurements, the network needs to check both Rel-15 and Rel-16 capabilities, in order to understand the real UE gap capability, this is not aligned with the current network implementation.

Observation 3: The problematic case was not precluded by RAN4, this implies that network needs to check both Rel-15 and Rel-16 capabilities when UE is configured with only NR FR1 measurements, and it is not aligned with the current network implementation.

In our understanding, the RAN2 spec TS 38.306 is clear on the definition of the two UE capabilities, even if RAN2 accepts the conclusion made by RAN4, we should not encourage any new UEs to implement like this.

Observation 4: The RAN2 spec is clear and we should not encourage UEs to report mismatch capabilities.

So, we suggest to note the LS, and do not make any change to TS 38.306. In addition, it is up to network to decide whether and when to update their implementation.

Proposal 1: 	Note the RAN4 LS, do not capture anything in RAN2 spec.

In the reply LS, we can inform RAN4 about our decision, and also suggest RAN4 to avoid mismatch issue as much as possible in future.

Proposal 2: 	In the reply LS, inform RAN4 about our decision and ask RAN4 to avoid mismatch issue in future.

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following observations and proposals:

Observation 1: Rel-15 UE capability supportedGapPattern can represent higher gap capability than Rel-16 UE capability supportedGapPattern-NRonly-r16.

Observation 2: Technically, a UE supporting a gap pattern in Rel-15 supportedGapPattern should also indicate the support of the same gap pattern in Rel-16 supportedGapPattern-NRonly-r16.

Observation 3: The problematic case was not precluded by RAN4, this implies that network needs to check both Rel-15 and Rel-16 capabilities when UE is configured with only NR FR1 measurements, and it is not aligned with the current network implementation.

Observation 4: The RAN2 spec is clear and we should not encourage UEs to report mismatch capabilities.

Proposal 1: 	Note the RAN4 LS, do not capture anything in RAN2 spec.

Proposal 2: 	In the reply LS, inform RAN4 about our decision and ask RAN4 to avoid mismatch issue in future.

References

- R2-2505049	 LS on mandatory gaps capability (R4-2508379; contact: Ericsson)

- R2-2004378	 LS on mandatory of measurement gap patterns (R4-2005846; contact: ZTE)

- R4-2506898	 Clarification on Rel-16 mandatory gap patterns in Rel-16  Ericsson