3GPP TSG RAN WG2 Meeting #131            	                      R2-2506143

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.10.3

Source:	Xiaomi

Title:	Discussion on other aspects of Rel-19 SON/MDT

Document for:	Discussion and Decision

Introduction

In the post RAN2 #130 meeting [Post130][605][SONMDT] Running NR RRC CR (Ericsson) [1], some proposals are proposed, including MDT for NTN, which needs further discussion in this paper.

Discussion

MDT for NTN

MDT for NTN

In the post email discussion [1], rapporteur propose two options for the discussion on FFS on how to handle the logging of MDT in NTN when location is not available:

- Option A. UE stops logging if it cannot obtain the location - Option B. UE continues logging if it is registered within a PLMN included in the PLMN identity list or is registered within a registered PLMN at the time of receiving MDT configuration (this option follows legacy behavior).

Some companies prefer to choose Option B, while most of them do not have a strong opinion on this. To recall, one of the reasons RAN2 introduced the use of only geographic area scope to indicate the applicable logging area to the UE is that the coverage of an NTN cell is too large for the UE to log all MDT reports. This was intended to reduce the burden on the UE’s battery and add flexibility to MDT logging in NTN scenarios.

Therefore, for Option B, if the UE cannot obtain its location (e.g., the UE is in a remote area without access to GNSS positioning) and remains within the coverage of the NTN cell (and not within TN cell coverage), the UE will consume excessive power logging useless MDT information that lacks location data. This contradicts the original goal of energy conservation for NTN UEs.

Besides, without location information, the network will inevitably have to discard such MDT reports without checking the geographical area scope, as these reports are unreliable. If Option B is specified, the UE will end up doing a lot of unnecessary work to collect useless data for the network, which is very unfriendly to both the terminal and the network.

Some companies also suggest that continuing to log the UE’s MDT reports is a legacy behavior. However, this legacy behavior only applies to scenarios where the UE is within TN coverage. In such cases, the UE can determine whether it is within the target area by comparing PLMN and TAC information broadcast by the network—this is different from the MDT scenario for NTN.

Thus, it’s reasonable and efficient to just stop the MDT logging while no location is obtained by UE itself.

Proposal 1: (RRC-4) If the UE is unable to obtain its location, the UE stops logging MDT for NTN.

Conclusion

In this contribution, we propose:

Proposal 1: (RRC-4) If the UE is unable to obtain its location, the UE stops logging MDT for NTN.

Reference

[1] [Post130][605][SONMDT] Running NR RRC CR (Ericsson)