3GPP TSG-RAN WG2 Meeting #131	R2-2506148

# Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.7.4

Source:	Ericsson

Title:	Discussion on DSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to DSR:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#130 the enhanced DSR solution was further clarified but a few open issues remain. This paper will discuss the remaining open issues.

# 2	Discussion of DSR cancellation

## 2.1 New Rel19 multi threshold DSR format

It was agreed in previous meetings that there can be multiple reporting thresholds configured for each LCG. For different traffic requirements and scheduler implementations there may be varying usefulness in the granularity of the reported information. It is therefore up to network decision how many thresholds are used for each LCG. It has also been agreed that not all configured thresholds will be reported every time and that an extension bit in the DSR format will be used to convey if further thresholds for the LCG is present in the report. Through this solution only the thresholds that contains data need to be reported. How this works is shown in Figure 1. For each LCG this solution enables to start at any threshold, skip any threshold and report for any number of thresholds.

Figure 1: Mapping of data from the LCGs, with different remaining times and buffer sizes, to a new DSR format using multiple thresholds and a n-bit indicating if a further threshold is present for the current LCG. In the example for LCG 0 threshold D and B is reported. For LCG 1 only threshold C is reported.

## 2.2 Capability dependency between Rel18 and Rel19

It was discussed in RAN2#129bis if there should be dependency between supporting Rel19 and Rel18 DSR and this has been listed as an open issue for UE capabilities. However, there is little reason to create such coupling. First it should be noted that because of the flexibility of the Rel19 DSR it is possible through configuration to report the same information as in Rel18 DSR, so if network want Rel18 behaviour that can be achieved with the Rel19 DSR. This also implies that there is no additional benefit to have both types of DSR configured at the same time. Lastly Rel19 provides more detailed reporting options than Rel18, thus for a UE supporting Rel19 DSR there is no need also support Rel18 DSR. We therefore propose:

[UE Cap-1] For a UE supporting Rel-19 DSR there is no pre-requisite to support Rel-18 DSR.

## 2.3 Relation between DSR and BSR

During the MAC CR review it was again raised an optimization proposal that a BSR should be cancelled if the DSR will report the same buffer size as the BSR. This exact scenario was already discussed during Rel18 when introducing the DSR feature (e.g. in RAN2 #125bis) and it was then agreed to not do any coupling between the BSR and DSR. Creating such interdependencies introduces unnecessary complexity for very little gains. To start with, it is a rare corner case when such cancellation could happen (i.e. the DSR and BSR reports are triggered at the same time, and both reports include the exact same buffer size information). Furthermore, the gains would only be a small reduction in overhead as the BSR should be rather small, i.e. only reporting for the LCG configured with delay reporting. Combining these two aspects the gains from doing such coupling would be minimal. Furthermore, this can create complexities, such as uncertainty for gNB to know how to decode the reports. Lastly and importantly, there is no new reason to do this optimization for the Rel19 DSR and there is nothing broken that needs to be fixed with the Rel18 behaviour, thus the Rel19 behaviour should not differ from the already agreed Rel18 behaviour.

To summarize, we should not introduce this proposed optimization because:

- Coupling of BSR and DSR has already been discussed and discarded earlier.

- The overhead reduction is tiny, and such tiny reduction is not relevant in XR scenarios.

- Coupling would only come in to play in rare corner cases.

- It causes complexities and uncertainties for the gNB to decode the BSRs and DSRs.

- No new reasons in Rel19, same situation could already happen in Rel18.

We therefore propose:

[MAC-2] BSR cancellation should not have dependencies to DSR transmission.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	[UE Cap-1] For a UE supporting Rel-19 DSR there is no pre-requisite to support Rel-18 DSR.

Proposal 2	[MAC-2] BSR cancellation should not have dependencies to DSR transmission.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024