3GPP TSG-RAN WG2 Meeting #131	R2-2506154

# Bengaluru, India, August 25–29, 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Discussion on Reply LS on SMTC enhancements

Document for:	Discussion/Decision

# Introduction

During RAN2#130, RAN2 made good progress on the necessary changes to the SMTC framework to accommodate the newly introduced extended initial SSB periodicity and link-level beam hopping. The corresponding agreements are cited in the Annex.

Besides increasing the maximum number of configurable SMTCs, it was agreed at RAN2#129bis to consider supporting the configuration of two different SMTC periodicities (per frequency). These agreements are, however, subject to feedback from RAN4. An LS has been sent to RAN4 in R2-2503068.

In the meantime, a Reply LS has been received from RAN4 in R2-2505050 (R4-2508433), comprising feedback on the feasibility of the following:

- SMTC enhancement for configuring SMTCs with two different periodicities (and different offsets) per frequency

- SMTC enhancement for configuring more than 4 SMTCs per frequency (i.e., 6)

In what follows, we discuss RAN4’s feedback on these potential SMTC enhancements and how RAN2 can take it into consideration.

# Discussion

Regarding the first SMTC enhancement, RAN4 has clarified that, from RAN4 perspective, it is feasible to support configuring SMTCs with up to two different periodicities and different offsets. However, RAN4 has concerns about the workload (for necessary changes of the RRM requirements) to support this in the Rel-19 timeframe.

- RAN4 has clarified that it is feasible to support configuring SMTCs with up to two different periodicities and different offsets but has concerns about the workload to support this in the Rel-19 timeframe.

Moreover, RAN4 has clarified that the configured SMTC periodicity must be equal to or greater than the largest SSB periodicity among the cells associated with the SMTC for the frequency.

In Figure 1 we analyze the implications of RAN4’s feedback for valid SMTC options considering the legacy SMTC periodicity and optionally an additional SMTC periodicity. For the sake of simplicity, we refrain from visualizing SMTC offsets as they are not relevant to the discussion on SMTC periodicities. We assume that two SSB periodicities, here 20ms and 160ms, are used among the serving cell and intra-frequency neighbor cells. Besides, SSB transmissions are visualized by arrows (collectively per SSB periodicity) and SMTC windows are visualized by rectangles.

Figure 1: Implications of RAN4’s feedback for valid SMTC options considering the legacy SMTC periodicity and optionally an additional SMTC periodicity.

In Option 1 and Option 2 only one (the legacy) SMTC periodicity is used, while in Option 3 two (the legacy plus one additional) SMTC periodicity is used.

Having only one SMTC periodicity available, one option (here Option 1) is to configure the SMTC periodicity as the smallest SSB periodicity among the relevant cells. While this option allows to capture all SSB transmissions in SMTC windows, it does not comply with the above RAN4 clarification. The problem is that the UE cannot distinguish between the cases of 1.) SSB was not transmitted or 2.) SSB was not detected due to channel impairments. The cells with 160ms SSB periodicity cannot be measured reliably. Hence this option is not viable.

A viable alternative is Option 2. Here, the SMTC periodicity is configured as the largest SSB periodicity among the relevant cells, which complies with the 3GPP specifications. However, one drawback is that part of the SSBs transmitted with shorter periodicity are not covered by SMTC windows and hence remain undetected by the UE. While this is in principle not an issue (as clarified by RAN4), it is a missed opportunity for RRM measurements, (re)synchronization, etc. The additional SSB transmissions are, nevertheless, beneficial for the UE during initial cell search, e.g., they enable legacy UEs to reliably detect a cell with a 20ms SSB periodicity.

- With only one (the legacy) SMTC periodicity, the configured SMTC periodicity has to be equal to or greater than the largest SSB periodicity among the cells that are to be measured by the UE.

In case of two SMTC periodicities, another option (here Option 3, which goes against the legacy RAN4 requirements) is to configure the legacy SMTC periodicity as the largest SSB periodicity among the relevant cells (like Option 2) and the additional SMTC periodicity such that the remaining SSB transmissions are also covered by SMTC windows and can thus be used by UEs capable of the SMTC enhancement. While it would in principle be possible to configure the additional SMTC periodicity as a value that is larger than largest SSB periodicity among the relevant cells, we see no apparent reason to do so. Hence this option is not depicted separately.

From this we conclude that configuring an additional SMTC periodicity has the benefit of enabling the capable UEs to measure and sync faster on those cells. However, it remains unclear whether this benefit is significant. We observe that the measurement or synchronization delays are not considered a pressing issue for NTN operating with Rel-19 DL CE, otherwise all cells should use the smaller SSB periodicity. The network needs to configure the legacy SMTC period in system information with the largest SSB period, 160ms in Figure 1, so as to enable legacy UE operation. Therefore, there is little incentive for the network to spend additional energy to provide other neighbor cells with shorter SSB periodicity if only a subset of UEs, Rel-19 capable, can benefit from this.

- There is little incentive for the network to provide SSB with shorter period in some cells while providing SSB with longer period in other cells (as per Rel-19 DL CE), considering the additional network energy expenditure.

- Configuring an additional (second) SMTC periodicity is optional even in a scenario with mixed SSB periodicities among intra-frequency neighbour cells, and the benefit in doing so remains to be clarified.

Given RAN4’s concerns about the workload and Observations 3 and 4, we propose first clarifying the benefits of introducing an additional SMTC periodicity before deciding to introduce it. RAN2 should be clear about the scenarios in which the additional SMTC periodicity brings significant benefits.

Clarify the target scenario as well as the benefits of introducing an additional SMTC periodicity considering the target scenario.

Concerning the second SMTC enhancement, RAN4 has clarified that it is feasible to support configuring more than 4 SMTCs per frequency (i.e., 6) for UEs in RRC_IDLE/INACTIVE. In this context, RAN4 points out that, as per current RRM requirements, the following applies to both intra- and inter-frequency cell measurements: If the number of configured SMTCs exceeds the parallelSMTC-r17, the UE may sequentially select and measure using a subset of SMTCs, until all SMTCs can be measured. In this case, the selection of SMTCs to be used is up to UE implementation, and the measurement delay is longer. We note that the word “may” indicates that the UE does not have to (i.e., may not) measure all SMTCs.

- RAN4 has clarified that it is feasible to support configuring more than 4 SMTCs per frequency (i.e., 6) for UEs in RRC_IDLE/INACTIVE, while highlighting that the selection of SMTCs (including which SMTCs to measure and in which order) is finally up to UE implementation.

- RAN4 has raised no concerns about RAN2’s intention of introducing configuration of up to 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE.

In our understanding of the previous discussions and latest agreements in RAN2, the intention of RAN2 is to support the configuration of up to 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE, and, at the same time, introduce assistance information (in the form of reference locations associated with the configured SMTCs) for the UEs to guide the selection of SMTCs. The motivation is to enable the UE, if provided with reference locations, to take an educated decision on which SMTCs to use for measuring, and which SMTCs to omit (i.e., not use). Given that SMTC selection is already a matter of UE implementation, we think the intention of RAN2 is, in general, in line with the RRM requirements in RAN4.

Considering this, we propose proceeding with the introduction of the SMTC enhancement for configuring up to 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE in Rel-19.

RAN2 confirms the introduction of 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE.

RAN2 confirms that a UE is not expected to measure all the SMTCs when location-based SMTCs are configured.

A UE using location-based SMTCs should only measure the closest SMTCs to its current location. The number of SMTCs to measure depends on UE capability.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4 has clarified that it is feasible to support configuring SMTCs with up to two different periodicities and different offsets but has concerns about the workload to support this in the Rel-19 timeframe.

Observation 2	With only one (the legacy) SMTC periodicity, the configured SMTC periodicity has to be equal to or greater than the largest SSB periodicity among the cells that are to be measured by the UE.

Observation 3	There is little incentive for the network to provide SSB with shorter period in some cells while providing SSB with longer period in other cells (as per Rel-19 DL CE), considering the additional network energy expenditure.

Observation 4	Configuring an additional (second) SMTC periodicity is optional even in a scenario with mixed SSB periodicities among intra-frequency neighbour cells, and the benefit in doing so remains to be clarified.

Observation 5	RAN4 has clarified that it is feasible to support configuring more than 4 SMTCs per frequency (i.e., 6) for UEs in RRC_IDLE/INACTIVE, while highlighting that the selection of SMTCs (including which SMTCs to measure and in which order) is finally up to UE implementation.

Observation 6	RAN4 has raised no concerns about RAN2’s intention of introducing configuration of up to 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify the target scenario as well as the benefits of introducing an additional SMTC periodicity considering the target scenario.

Proposal 2	RAN2 confirms the introduction of 6 SMTCs per frequency for UEs in RRC_IDLE/INACTIVE.

Proposal 3	RAN2 confirms that a UE is not expected to measure all the SMTCs when location-based SMTCs are configured.

Proposal 4	A UE using location-based SMTCs should only measure the closest SMTCs to its current location. The number of SMTCs to measure depends on UE capability.

# References

R2-2501415, Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements, Huawei, RAN2#129, February 2025.

RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN#106, Dec. 2024.

# Annex

During RAN2#130, the following agreements related to DL coverage enhancements were captured:

WA: We introduce a mechanism to assist the NW to configure the SMTCs in connected mode

RAN2 to agree that the maximum configured SMTCs per frequency for idle/inactive UEs is 6.

To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)