3GPP RAN WG2 Meeting #131 	R2-2506157

# Bangalore, India,  August 25th – 29th , 2025

Agenda Item:	8.9.3

Source:	InterDigital, Inc.

Title:	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete.

Document for:	Discussion

# Introduction

A revised WID was approved to further enhance the support of NB-IoT and eMTC [1]

The following objective is provided

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2] - Multi-tone support for 15 kHz SCS should also be considered Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction [RAN2]: - Msg3 transmission without msg1/RAR - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In RAN2#130, the following was agreed:

Agreements:

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

# Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

If PUR is used, it is possible to terminate EDT without using RRC. As described in 36.300 [2] :

7a	If the (ng-)eNB is aware that there is no pending downlink data or signalling, the (ng-)eNB can send a Layer 1 ACK optionally containing a Time Advance Adjustment to the UE to update the TA and terminate the procedure.

7b	If the (ng-)eNB is aware that there is no further data or signalling, the (ng-)eNB can send a Time Advance Command to update the TA and terminate the procedure.

Observation 1: PUR feature can terminate the EDT procedure without using RRC, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

In the previous meeting, it was agreed CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution. In other words, the CB-EDT procedure can be terminated without RRC. The RRC response message (Msg4) contains the container for NAS to include the application layer response. The MAC CE with contention resolution ID can be used to terminate the procedure, if there is no pending application layer response for DL transmission (if there is a pending response, it does not make sense to terminate the procedure, because another connection would be required for the application layer response which would defeat the purpose of CB-EDT.

The main question, therefore, as to whether early termination of EDT (or PUR) works in practise is “how does eNB know that there is no pending downlink data from the application layer?“

Proposal 1: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

The UE can transmit an indication (L1Ack) in PURConfigurationRequest, which informs the eNB a preference for using RRCEarlyDataComplete. Note that, although the figure states “L1 ACK” this is named “rrc-ACK” in the RRC specification and it indicates RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR. This is an indirect way to specify that the UE can send some information about whether it expects downlink data in response to the uplink. It is then up to the eNB implementation what (if anything) to do with that knowledge. The main issue with terminating EDT early is when there is some downlink application layer data to send in response. If the procedure is terminated early then the network must page the UE in order to transmit this, which is even more inefficient than waiting and terminating using RRC.

Observation 2: With the PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRC.

Since for CB-Msg3 we will not be using PURConfigurationRequest, there needs to be another way to indicate from the UE whether RRC response message is preferred by the UE for acknowledging the reception of a transmission. The most convenient place would be in the RRCEarlyDataRequest.

Proposal 2: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

One problem that still remains, is what happens when there is a pending application layer response, but UE is expecting a Msg4 confirmation sooner in order to determine whether the UL transmission was a success. Waiting for the RRC response would mean UE determines the UL fails. Sending the MAC CE with contention resolution would terminate the procedure and require a subsequent connection for the application layer response.

UE needs to be able to receive the MAC CE to confirm uplink success in time, but it also needs to know whether to terminate only the uplink transmissions and wait for an RRC response, or to terminate the procedure entirely and return to RRC_IDLE. For this purpose, an additional flag would be needed in the contention resolution MAC CE.

Proposal 3: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# Conclusion

In this contribution we discuss uplink capacity enhancements and make the following proposals for discussion:

Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

Observation 1: PUR feature can terminate the EDT procedure without using RRC, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 1: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Observation 2: With the PUR feature it is possible for the UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRC.

Proposal 2: Introduce “rrc-ACK”, to RRCEarlyDataRequest to indicate whether an RRC Response Message is preferred by the UE for CB-Msg3, similar to PURConfigurationRequest

Proposal 3: Contention Resolution MAC CE includes a new flag indicating whether to terminate only the uplink transmissions, or to terminate the entire procedure and return to RRC_IDLE.

# References

- RP-242397 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

- 36300-i00 , “E-UTRA/E-UTRAN;Overall description;Stage 2”