3GPP TSG-RAN-WG2 Meeting #131	R2-2506180

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item	:	  8.3.2.2

Source	: 	Sharp

Title	: 	Discussion on performance monitoring

Document for	:	Discussion and Decision

# Introduction

In RAN2#130, the following agreements were made for performance monitoring configurations.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In this contribution, aspects related to UE-sided monitoring and UE-decision are discussed.

# Discussion

The following cases for performance monitoring procedures were agreed in RAN2#130 [1].

- For UE-sided model, NW-sided monitoring

- For UE-sided model, UE-sided monitoring, and network-decision

- FFS: UE-sided model, UE sided monitoring, and UE-decision

In the UE-sided model with network-sided monitoring, the performance monitoring procedure involves the network sending both monitoring and inference configurations to the user equipment (UE). The UE then reports legacy measurement data and inference outputs according to these configurations, enabling the NW to carry out monitoring independently from the UE.

In contrast, under UE-sided monitoring, the network provides the monitoring configuration to the UE, which measures the monitoring data (i.e., ground-truth) and compares it with its inference output. The specific inference outputs to be compared to ground truth is yet to be discussed. The monitoring configuration may include criteria such as L3 RSRP difference, as agreed previously.

Once the UE generates monitoring results, two decision-making options are available: in the network-decision approach, the UE reports the results to the network, which then makes the management decision. In the UE-decision approach, the UE itself makes the decision and informs the NW. While the network-decision method allows centralized control over AI functionality performance, it introduces latency, as the UE must wait for NW signaling before reacting. Although this approach may reduce signaling compared to the network-sided monitoring case, it also reduces the flexibility available to the UE to react to insufficient AI/ML functionality performance.

Observation 1: Although the UE-monitoring and network-decision option may reduce signalling when compared to network-sided monitoring, it may introduce latency and inefficiencies when the AI/ML functionality performance is insufficient.

Conversely, the UE-decision method enables faster response to UE-sided conditions, allowing the UE to promptly switch to legacy methods to maintain connection quality. For the case with UE-sided model, UE sided monitoring, and UE-decision, it was agreed that the UE can make the management decision based on network configuration and send the decision to NW. The use cases to be considered were left for future study.

As described in our other contribution [2], the UE may be provided with different inference configurations and related parameters for measurement reduction. In this context, two general approaches are proposed for measurement reduction. Network-controlled measurement reduction (e.g. with SSB configuration, and skipping pattern) or UE-flexible measurement reduction, e.g. based on the network-provided MRRT/MRRS configuration can be considered.

Observation 2: For measurement reduction scenarios, the UE may be provided with some flexibility in the decision to perform or skip reference signal measurements based on the configurations provided by the network (skipping pattern/MRRT etc.).

In addition, it has already been agreed [1] that several aspects, such as model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation. In these cases, the network may not be aware of the UE-sided decisions for the listed parameters. Therefore, network-sided decisions may not be efficient in such scenarios, if the network cannot map the reported monitoring results to a given configuration. This can lead to unnecessary functionality reconfigurations and deactivation. Hence, it would be beneficial to also allow UEs to make management decisions based on the implemented AI/ML functionalities. The management decisions may be limited to, for example, a subset of AI/ML functionality configurations which are pre-configured to the UE by the network or switching to legacy-based measurement reporting procedure. RAN2 is requested to study the appropriate scenarios in the WI stage.

Observation 3: When model-based choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) are based on implementation, the network may not have a complete mapping between the monitoring performance reported by the UE and the prediction configurations provided to the UE.

Observation 4: When the network is not aware of the UE-sided conditions when the performance monitoring results are provided, it can lead to inefficiencies when the network makes decisions. This can lead to unnecessary functionality reconfigurations and deactivation.

Proposal 1: When flexibility is provided to the UE for measurement reduction scenarios, or when certain model related options are implementation dependent, the network may (pre) configure the UE to perform management decisions, for example, based on a set of pre-configured actions.

Proposal 2: RAN2 to study applicable scenarios and constraints for UE-decision in the WI stage.

# Conclusion

Observation 1: Although the UE-monitoring and network-decision option may reduce signalling when compared to network-sided monitoring, it may introduce latency and inefficiencies when the AI/ML functionality performance is insufficient.

Observation 2: For measurement reduction scenarios, the UE may be provided with some flexibility in the decision to perform or skip reference signal measurements based on the configurations provided by the network (skipping pattern/MRRT etc.).

Observation 3: When model-based choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) are based on implementation, the network may not have a complete mapping between the monitoring performance reported by the UE and the prediction configurations provided to the UE.

Observation 4: When the network is not aware of the UE-sided conditions when the performance monitoring results are provided, it can lead to inefficiencies when the network makes decisions. This can lead to unnecessary functionality reconfigurations and deactivation.

Proposal 1: When flexibility is provided to the UE for measurement reduction scenarios, or when certain model related options are implementation dependent, the network may (pre) configure the UE to perform management decisions, for example, based on a set of pre-configured actions.

Proposal 2: RAN2 to study applicable scenarios and constraints for UE-decision in the WI stage.

# Reference

- Chair’s notes RAN2#130

- R2-2506177