3GPP TSG-RAN WG2 #131	R2-2506184

Bangalore, India, 25-29 August 2025

Agenda Item:	8.9.3

Source:	Ericsson

Title:	UL capacity enhancements for IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss the remaining issues for UL capacity enhancements.

# 2	EDT definition, Koffset and OCC

## 2.1	EDT definition in MAC versus RRC

In running MAC CR we have:

Early Data Transmission: Allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure as specified in TS 36.300 [20]. The S1 connection is established or resumed upon reception of the uplink data and may be released or suspended along with the transmission of the downlink data. Early data transmission refers to both CP-EDT and UP-EDT.

While in RRC we have

Early Data Transmission: Allows one uplink data transmission optionally followed by one downlink data transmission during the random access procedure or during the CB-MSG3-EDT procedure as specified in TS 36.300 [9]. The S1 connection is established or resumed upon reception of the uplink data and may be released or suspended along with the transmission of the downlink data. Early data transmission refers to both CP-EDT and UP-EDT.

Copy the EDT definition in RRC to the MAC definition.

## 2.2	Koffset

The CB-Msg3-EDT functionality is supposed to work for UEs in IDLE or IDLE/suspended mode and eNB cannot know what UE that is accessing, therefore all UEs must use the cell specific Koffset for Msg3 transmission on contention based resources. However, if the UE has a UE specific Koffset it would be possible to use a UE specific Koffset after Msg3 – but that would be an unnecessary complication.

The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

The MAC entity shall discard any Differential Koffset at the start of a CB-Msg3-EDT procedure.

## 2.3	OCC for CB-Msg3-EDT

RAN1 has not finalized all details of OCC and there are issues that degrade OCC performance on a contention based channel (collisions may not be resolvable if the UE transmissions are received at eNB with different power and/or different time offset and/or different frequency offsets, multi tone transmissions may not be supported). Further, in RAN1 OCC is only for UEs in CONNECTED mode while CB-Msg3-EDT will access from IDLE mode.

Considering this RAN2 agreement from RAN2#130

If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

Based on this, if supported at all, it is obvious that OCC can be added later as a backwards compatible configuration without affecting non-OCC CB-Msg3-EDT resources, therefore to not cause delay to the closure of this work item we suggest to not support OCC for CB-Msg3-EDT now.

It is possible to add OCC support for CB-Msg3-EDT in a backwards compatible way in later releases.

RAN2 do work on OCC for CB-Msg3-EDT in release 19.

# 3	CB-Msg3 considerations

## 3.1	CB-Msg3 periodicity

RAN2 briefly discussed the periodicities of resources for CB-Msg3-EDT in RAN2#129bis:

For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration

We see no use for a feature where there is very long between opportunities for the UEs to send CB-Msg3.

If the load on the CB-Msg3-EDT resources is so low that an interval in the order of the H-SFN can be used, then there is no need to improve UL capacity – and regular EDT via the random access procedure can utilize the resources more efficiently without the delay to wait for the CB-Msg3 opportunity.

The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

# 4	CB-RNTI considerations

Considering the CB-RNTIs agreements in RAN2#129bis

For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

And the working assumption from last meeting

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

And the proposed text in the Running MAC CR:

For BL UEs and UEs in CE mode A, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = 2401 + (Msg3_W_id mod [FFS X]) + [FFS X]*CE_level

Where:

-	Msg3_W_id is floor((SFN_id - startSFN) / windowPeriodicity), where the SFN_id is the first SFN of the selected CB-Msg3 transmission window, startSFN is the CB-Msg3 transmission window start SFN defined by IE [FFS], the windowPeriodicity is the CB-Msg3 transmission window periodicity defined by IE [FFS].

-	[FFS X] is ceil(Msg4_WS/Msg3_WP), where the Msg4_WS is [FFS], the maximum CB-Msg3 response timer length, the Msg3_WP is [FFS], the minimum CB-Msg3 transmission window periodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 2).

For NB-IoT UEs, the CB-RNTI associated with the CB-Msg3 transmission window in which the CB-Msg3(s) are transmitted, is computed as:

CB-RNTI = 4097 + (Msg3_W_id mod [FFS X]) + [FFS X]*CE_level + 3*[FFS X]*carrier_id

Where:

-	Msg3_W_id is floor((SFN_id - startSFN) / windowPeriodicity), where the SFN_id is the first SFN of the selected CB-Msg3 transmission window, startSFN is the CB-Msg3 transmission window start SFN defined by IE [FFS], the windowPeriodicity is the CB-Msg3 transmission window periodicity defined by IE [FFS].

-	[FFS X] is ceil(Msg4_WS/Msg3_WP), where the Msg4_WS is [FFS], the maximum CB-Msg3 response timer length, the Msg3_WP is [FFS], the minimum CB-Msg3 transmission window periodicity.

-	CE_level is the selected enhanced coverage level (0 <= CE_level < 3).

-	carrier_id is the index of the UL carrier associated with the selected UL grants (0 <= carrier_id < 16). The carrier_id of the anchor carrier is 0.

Editor’s note: FFS whether to confirm the working assumption of CB-RNTI calculation.

This Working Assumption method of calculating RNTI is based on

- CB-Msg3 window periodicity in SIB

- start SFN in SIB

- SFN of start of current Msg3 window

- Maximum CB-Msg4 window length in SIB

- Minimum CB-Msg3 window periodicity in SIB

- Selected CE level

- carrier id of selected grant (only for NB-IoT)

- mod function

- ceil function

- floor function

This is complicated to calculate and it is complicated to get all the details correct (for example non overlapping RNTI ranges when different CE levels use different window periodicities etc.) as seen by the many diverging comments in the MAC Open Issue discussion [307].

Further, there are most likely gaps between the RNTIs associated with consecutive CB-Msg3 windows which may complicate the eNB possibility to use those RNTIs for other things. To fully utilize all the RNTI resources, the eNB and UE must calculate these gaps every time the CB-Msg3-EDT is reconfigured.

CB-RNTI formula in the working assumption uses seven parameters and three functions. This makes it difficult to understand and to get all the details correct to achieve non-overlapping RNTI ranges for different CE levels and there may be gaps between RNTIs for consecutive CB-Msg3 windows.

A simpler method is to decide on a start RNTI (startCB-RNTI) and the number of RNTIs (nrofCB-RNTI), then define that the first CB-Msg3 window as indicated by the SFN and H-SFN of the CB-Msg3-EDT configuration is associated with CB-RNTI = startCB-RNTI, the next CB-Msg3 window is associated with CB-RNTI = startCB-RNTI + 1, and so on. For example:

For CB-Msg3-EDT, a first CB-Msg3 window (M = 0), start in startH-SFN and startSFN, is associated with a CB-RNTI equal to startCB-RNTI. The Mth (1 ≤ M) CB-Msg3 window following the first CB-Msg3 window is associated with a CB-RNTI equal to:

CB-RNTI = startCB-RNTI + (M modulo (nrofCB-RNTI))

A first CB-Msg3 window (M = 0), starting in startH-SFN and startSFN, is associated with a CB-RNTI equal to startCB-RNTI.

For the Mth CB-Msg3 window before or after the first CB-Msg3 window, the associated CB-RNTI is equal to: 
CB-RNTI = startCB-RNTI + (M modulo (nrofCB-RNTI))

The proposed CB-RNTI formula is simple and configurable and supports having unique CB-RNTIs for CB-Msg4 monitoring even when CB-Msg4 windows overlap for different CB-Msg3 windows. The same formula can be used for eMTC and NB-IoT.

This simple method is based on

- startCB-RNTI in SIB

- nrofCB-RNTI in SIB

- startSFN in SIB

- startH-SFN in SIB

- mod function

# 5	CB-Msg4 considerations

## 5.1	Start of PDCCH monitoring for receiving Msg4

We discussed the start of Msg4 monitoring last meeting:

2.	We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

Here we discuss the NW processing time.

It is a waste of UE energy if the UEs start monitoring PDCCH without taking eNB processing into account as there will be nothing to receive.

Further, in legacy PDCCH monitoring there is always a part related to eNB processing time, therefore easiest is to follow legacy and take the eNB processing into account.

The NW processing time shall be considered for the start of Msg4 monitoring.

For eMTC after sending a RA preamble 36.321 5.1.4:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus 3 subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level;

For NB-IoT after sending a RA preamble 36.321 5.1.4:

-	RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions;

Where X is 4 or 41. However, preambles are special for NB-IoT and CB-Msg4 is more similar to a regular NPUSCH transmission for which we have (36.213, section 16.6):

- if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3.

The difference between counting DL and UL subframes and Kmac this is equivalent to UE-eNB RTT + 3 ms between the end of an NPUSCH transmission and the start of NPDCCH monitoring.

In legacy eMTC and NB-IoT, there is always 3 ms processing time for the eNB between a (N)PUSCH transmission and (N)PDCCH monitoring.

The UE shall start PDCCH monitoring for CB-Msg4 earliest UE-eNB RTT + 3 ms after the end of the CB-Msg3 window.

## 5.2	Contention control and Fallback

When the UE gets a C-RNTI and a matching UE CR ID MAC CE, then contention resolution of CB-Msg3-EDT was successful, but the network wants to send further messages to the UE. When shall the UE stop monitoring PDCCH in this case? Simple is for MAC to delay informing higher layers of CB-Msg3 success and reuse the mac-ContentionResolutionTimer for monitoring PDCCH:

If the UE receives a CB-Msg4, with a UE contention resolution ID MAC CE that match the CB-Msg3 that the UE sent, and a C-RNTI MAC CE (with or without an RRC message) the UE shall consider the contention resolution of the CB-Msg3-EDT successfully completed and start the TAT (implementing the TAC first, if received) then start monitor PDCCH. The UE shall start the mac-ContentionResolutionTimer and monitor PDCCH while it is running. If mac-ContentionResolutionTimer expires, MAC shall indicate successful CB-Msg3-EDT procedure to higher layers.

We need a way to determine when to give up further Msg3 transmissions, that is the unsuccessful CB-Msg3-EDT case. This is if the UE does not receive a Msg4 with a matching UE CR ID MAC CE for any of a configured number of reattempts of Msg3 transmission and possible stepping of CE level and/or power ramping.

For legacy EDT, there seems to be no failure case described in RRC. For PUR there is a NOTE:

NOTE:	For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation.

In running MAC CR, MAC informs higher layers of success/failure, but there is no handling in RRC. Therefore, we propose:

RAN2 to discuss the failure handling for CB-Msg3 in MAC and RRC. For example, shall we add a similar statement for CB-Msg3-EDT as for PUR?

## 5.3	Support for HARQ ACK of Msg4

Regarding HARQ we agreed

Proposal 7: HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how

the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Agreed

And

The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern.

Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK.

However, for User plane CIoT optimization, the UEs are in suspended mode, and may have received configuration of HARQ feedback disable, therefore we need to make clear when UE shall follow the signalled HARQ info in CB-Msg4 and when to follow a configured HARQ feedback disabled.

For the CB-Msg4, if the NW includes the HARQ ACK resources, the UE shall follow that info as the NW would not include the resources unless it wants the UE to send HARQ ACK feedback.

If the CB-Msg4 is successfully decoded and it contains a HARQ ACK resource allocation for a UE, the UE shall always send HARQ ACK.

However, when the CB-Msg4 contains a matching UE contention resolution identity and a C-RNTI and if it later is scheduled again, the DCI will contain resources for sending HARQ ACK but UE may have a configuration of disabled HARQ feedback.

If the CB-Msg4 is successfully decoded and it contains a matching UE contention resolution identity and a C-RNTI, when UE is later scheduled it follows the configured HARQ feedback enabled/disabled.

## 5.4	Size of the TAC in CMR

For Timing Advance in NTNs, the maximum remaining timing error is 97*Ts (36.133 section 7.20A and 7.24A).

The RRC_CONNECTED mode TAC of size six bits NTA,new = NTA,old + (TA 31)16 can adjust the TA in the range [-496:16:512]*Ts (36.213 section 4.2.3) which can compensate the remaining error of 97*Ts before applying the TAC to achieve performance that is on the same level as Rel-18 in RRC_CONNECTED mode.

However, using 4 bits for the TAC, signalling TA 0 to 15 in steps of 1, we get NTA,new = NTA,old + (TA 7)16 or a range of TA adjustments of [(-7*16):16:(8*16)] = [-112:16:128] which can fully compensate a maximum remaining error of 97*Ts.

To compensate the maximum remaining TA error after CB-Msg3 transmission, a TAC of 4 bits is sufficient.

The running MAC CR has these for CMR:

Figure 6.1.x-4a: MAC CMR for BL UEs and UEs in CE mode A

Figure 6.1.x-4b: MAC CMR for NB-IoT UEs

A TAC of 4 bits will save one octet in the CMR for NB-IoT UEs, which will increase, coverage, and simplify the description in MAC.

The timing advance command in the MAC CMR uses 4 bits. Inform RAN1 in an LS to add the new TAC of 4 bits indicating a timing advance adjustment of 16*(TACvalue – 7)*Ts.

With this agreement, the 7th octet in CMR has 4 bits of TAC for both NB-IoT and eMTC, 2 bits for HARQ ACK field plus 2 R bits for eMTC, and in NB-Iot 4 bits for HARQ ACK field plus no R bits.

## 5.5	MAC PDU for CB-Msg4

In Running MAC CR in “6.1.x MAC PDU (CB-Msg4)” we have:

If included, the CB Backoff Indicator subheader is only included once and is the first subheader included within the MAC PDU header.

Thus, the BI is always in the first subheader in the CMR.

Further, we have in “6.2.x	 MAC header for CB-Msg4”:

-	E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate at least another subheader follows. The subsequent subheader can be E/T/R/BI MAC subheader, E/T/R/R/H/TA/C MAC subheader or E/T/LCID/(F)/(L) MAC subheader. The E field is set to "0" to indicate that either a MAC CMR, a MAC SDU, or padding starts at the next byte;

Clearly E=1 cannot indicate that a BI subheader is following if BI subheader is always the first subheader.

However, we also have

Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.

When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

So, question is what is wanted? The Padding text comes from the DL-SCH MAC PDU. However, as an CB-Msg3-EDT implementation anyway need to conte every octet in all MAC subheaders, we propose to not treat one or two octet padding any different than larger padding. This will simplify the decoding of the MAC headers for all UEs when they are finding where the first CMR starts, and it has no drawback as there can already be padding at the end of a MAC PDU.

Revise text on padding:
Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed.
When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

Revise text on E field:
E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate at least another subheader follows. The subsequent subheader can be E/T/R/BI MAC subheader, E/T/R/R/H/TA/C MAC subheader or E/T/LCID/(F)/(L) MAC subheader. The E field is set to "0" to indicate that either a MAC CMR, a MAC SDU, or padding starts at the next byte;

Revise text on LCID field:
LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or padding as described in table 6.2.1-1 for the DL-SCH. There is one LCID field for each MAC SDU, or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. The LCID field size is 5 bits;

## 5.6	MAC header for CB-Msg4

In Running MAC CR “6.2.x MAC header for CB-Msg4” we have:

-	T: The Type field is a flag indicating the type of the MAC subheader. The T field is set to "00" to indicate the presence of a Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of the H field in the subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in the subheader. The value "11" is reserved for further use. The size of the Type field is 2 bits;

This can be clarified and aligned to other fields in MAC.

Adopt this text for the T Type field of CB-Msg4 MAC headers:
T: The Type field is a flag indicateing the type of the MAC subheader. The T field is set to "00" to indicate the presence of a Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of the H field in thea CMR subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in thea MAC SDU subheader. The value "11" is reserved for furtherfuture use. The size of the Type field is 2 bits;

All UEs, R19 and future ones, must be able to know the size of each MAC CMR and each MAC SDU to be able to search for a MAC CMR that is addressed to it.

This means each UE must count all octets in each subheader to find the first octet of the payload:

- is there zero, one or two padding subheader using 0, 1 or 2 octet in beginning of the MAC header

  - read T = 10

  - LCID but can be skipped if UEs assume there is always a CMR before a MAC SDU with data

  - read E field

  - no payload part

- is there a BI subheader using one octet

  - read T = 00

  - read E field

  - no payload part

- is there a MAC CMR subheader using one octet

  - read T = 01

  - read H, TA, C, E fields

  - payload 6, 7, 8 or 9 octet for eMTC and 6, 7, 8, 9 or 10 octet for NB-IoT

- is there padding or MAC SDU subheader with the previous CMR using one, two or three octets

  - read T = 10

  - read E

  - read LCID (aslo detect padding subheader if E = 0, then subheader is one octet)

  - read F field (if not padding), if F=0, subheader is two octets else subheader is three octet

  - read L field (if not padding)

  - payload is rest of TB if padding, else payload is indicated by L field

However, if the proposal above to only allow padding at the end of the MAC header/PDU we have

- is there a BI subheader using one octet

  - read T = 00

  - read E field

  - no payload part

- is there a MAC CMR subheader using one octet

  - read T = 01

  - read H, TA, C, E fields

  - payload 6, 7, 8 or 9 octet for eMTC and 6, 7, 8, 9 or 10 octet for NB-IoT

- is there padding or MAC SDU subheader with the previous CMR using one, two or three octets

  - read T = 10

  - read E

  - read LCID (aslo detect padding subheader if E = 0, then subheader is one octet)

  - read F field (if not padding), if F=0, subheader is two octets else subheader is three octet

  - read L field (if not padding)

  - payload is rest of TB if padding, else payload is indicated by L field

After reading all the subheaders, the UE knows the size of each CMR and if each CMR has an associated MAC SDU and the size of that MAC SDU, as well as where in the MAC payload the CMs/SDUs can be found.

Now, how shall a legacy UEs behave if the reserved T Type value “11” is received?

In our understanding, if 11 is used after R19, a R19 UE must still be able to count all octets in the full MAC PDU header to find the first MAC CMR, therefore it must know the size of all subheaders including future ones. While, for the payload part, legacy UEs do not need to know the sizes of future Type 11 CMRs/SDUs/etc. Therefore, to future proof the decoding of the MAC headers, we propose to specify that a Type 11 header has a given number of octets, for example two octets.

RAN2 to discuss future proofing T Type field value 11. For example, deciding that the T Type 11 subheader size is two octets.

We have in running CR:

-	F: The Format field indicates the size of the Length field. There is one F field per MAC PDU subheader except for the last subheader. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1.

-	L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC PDU subheader corresponding to the MAC SDU except for the last subheader. The size of the L field is indicated by the F field;

In legacy, the F field has a similar formulation about “last subheader” but then there are also separate subheader formats defined for the last subheader, which is not the case for CB-Msg3-EDT functionality now. We think it is not needed to define separate subheader formats for the last subheader, and therefore this part can be removed. However, there is the padding subheader that do not have an F field. Therefore, we propose:

Revise text on F field:
F: The Format field indicates the size of the Length field. There is one F field per MAC PDU subheader except for fixed sizethe last subheaders. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1.

Revise text on L field:
L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC PDU subheader corresponding to the MAC SDU except for the last subheader. The size of the L field is indicated by the F field;

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	It is possible to add OCC support for CB-Msg3-EDT in a backwards compatible way in later releases.

Observation 2	CB-RNTI formula in the working assumption uses seven parameters and three functions. This makes it difficult to understand and to get all the details correct to achieve non-overlapping RNTI ranges for different CE levels and there may be gaps between RNTIs for consecutive CB-Msg3 windows.

Observation 3	The proposed CB-RNTI formula is simple and configurable and supports having unique CB-RNTIs for CB-Msg4 monitoring even when CB-Msg4 windows overlap for different CB-Msg3 windows. The same formula can be used for eMTC and NB-IoT.

Observation 4	In legacy eMTC and NB-IoT, there is always 3 ms processing time for the eNB between a (N)PUSCH transmission and (N)PDCCH monitoring.

Observation 5	To compensate the maximum remaining TA error after CB-Msg3 transmission, a TAC of 4 bits is sufficient.

Observation 6	A TAC of 4 bits will save one octet in the CMR for NB-IoT UEs, which will increase, coverage, and simplify the description in MAC.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Copy the EDT definition in RRC to the MAC definition.

Proposal 2	The UE shall use the cell specific Koffset for the CB-Msg3-EDT procedure, regardless of if it has a UE specific Koffset or not.

Proposal 3	The MAC entity shall discard any Differential Koffset at the start of a CB-Msg3-EDT procedure.

Proposal 4	RAN2 do work on OCC for CB-Msg3-EDT in release 19.

Proposal 5	The periodicity of (N)PUSCH resources for CB-Msg3 is not larger than H-SFN duration.

Proposal 6	A first CB-Msg3 window (M = 0), starting in startH-SFN and startSFN, is associated with a CB-RNTI equal to startCB-RNTI.

Proposal 7	For the Mth CB-Msg3 window before or after the first CB-Msg3 window, the associated CB-RNTI is equal to:  CB-RNTI = startCB-RNTI + (M modulo (nrofCB-RNTI))

Proposal 8	The NW processing time shall be considered for the start of Msg4 monitoring.

Proposal 9	The UE shall start PDCCH monitoring for CB-Msg4 earliest UE-eNB RTT + 3 ms after the end of the CB-Msg3 window.

Proposal 10	If the UE receives a CB-Msg4, with a UE contention resolution ID MAC CE that match the CB-Msg3 that the UE sent, and a C-RNTI MAC CE (with or without an RRC message) the UE shall consider the contention resolution of the CB-Msg3-EDT successfully completed and start the TAT (implementing the TAC first, if received) then start monitor PDCCH. The UE shall start the mac-ContentionResolutionTimer and monitor PDCCH while it is running. If mac-ContentionResolutionTimer expires, MAC shall indicate successful CB-Msg3-EDT procedure to higher layers.

Proposal 11	RAN2 to discuss the failure handling for CB-Msg3 in MAC and RRC. For example, shall we add a similar statement for CB-Msg3-EDT as for PUR?

Proposal 12	If the CB-Msg4 is successfully decoded and it contains a HARQ ACK resource allocation for a UE, the UE shall always send HARQ ACK.

Proposal 13	If the CB-Msg4 is successfully decoded and it contains a matching UE contention resolution identity and a C-RNTI, when UE is later scheduled it follows the configured HARQ feedback enabled/disabled.

Proposal 14	The timing advance command in the MAC CMR uses 4 bits. Inform RAN1 in an LS to add the new TAC of 4 bits indicating a timing advance adjustment of 16*(TACvalue – 7)*Ts.

Proposal 15	Revise text on padding: Padding occurs at the end of the MAC PDU, except when single-byte or two-byte padding is required. Padding may have any value and the MAC entity shall ignore it. When padding is performed at the end of the MAC PDU, zero or more padding bytes are allowed. When single-byte or two-byte padding is required, one or two MAC PDU subheaders corresponding to padding are placed at the beginning of the MAC PDU before any other MAC PDU subheader.

Proposal 16	Revise text on E field: E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate at least another subheader follows. The subsequent subheader can be E/T/R/BI MAC subheader, E/T/R/R/H/TA/C MAC subheader or E/T/LCID/(F)/(L) MAC subheader. The E field is set to "0" to indicate that either a MAC CMR, a MAC SDU, or padding starts at the next byte;

Proposal 17	Revise text on LCID field: LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or padding as described in table 6.2.1-1 for the DL-SCH. There is one LCID field for each MAC SDU, or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. The LCID field size is 5 bits;

Proposal 18	Adopt this text for the T Type field of CB-Msg4 MAC headers: T: The Type field is a flag indicateing the type of the MAC subheader. The T field is set to "00" to indicate the presence of a Backoff Indicator field in the subheader (BI). The T field is set to "01" to indicate the presence of the H field in thea CMR subheader. The T field is set to "10" to indicate the presence of a Logical Channel ID (LCID) field in thea MAC SDU subheader. The value "11" is reserved for furtherfuture use. The size of the Type field is 2 bits;

Proposal 19	RAN2 to discuss future proofing T Type field value 11. For example, deciding that the T Type 11 subheader size is two octets.

Proposal 20	Revise text on F field: F: The Format field indicates the size of the Length field. There is one F field per MAC PDU subheader except for fixed sizethe last subheaders. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1.

Proposal 21	Revise text on L field: L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC PDU subheader corresponding to the MAC SDU except for the last subheader. The size of the L field is indicated by the F field;

# 7	References

RP-243278, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc., RAN#106, Dec. 2024