3GPP TSG-RAN WG2 Meeting #131bis	R2-2506776

# Prague, Czech Republic, 13 – 17 October 2025

Agenda item:		10.4

Source:	Xiaomi

Title:	Discussion on 6G mobility

Document for:		Discussion and Decision

# Introduction

SID of RAN WG study item on “Study on 6G Radio” [1] includes following objectives related to mobility:

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

In this contribution, we discuss intra 6G mobility in different RRC states, inter-RAT mobility and the support of the requirements on service level continuity upon mobility.

# Discussion

## Review of LTE and NR mobility solutions

In LTE and NR, many mobility solutions were standardized. In Table 1 below, a comparison of mobility solutions is shown. Note that DC related solutions (e.g. CPAC) are not shown according to SID [1] “RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC.”

Table 1: Comparison of LTE and NR mobility solutions

| Feature | LTE or NR | First release | Pros / Motivation | UE complexity | NW complexity | Commercialization |
| --- | --- | --- | --- | --- | --- | --- |
| Layer 3 HO | LTE, NR | - | Baseline | Low | Low | Yes |
| RACH-less | LTE | 14 | Reduce mobility interruption time | Low No RACH | Low Indicate the uplink timing and provide the pre-allocated uplink grant in the handover command | No |
| Make-Before-Break | LTE | 14 | Reduce mobility interruption time | Medium UE needs to be capable of two Rx | Low Keep the connection after HO command for source gNB | No |
| DAPS | LTE, NR | 16 | Reduce mobility interruption time (maintains the source gNB connection after reception of HO command, and release source cell after successful random access to the target gNB.) | High UE shall support the simultaneous DL reception of PDCCH and PDSCH from source and target cell, and single UL transmission for DAPS | High Keep the connection after HO command for source gNB, handle the data received from source/target | No |
| CHO | LTE, NR | 16 | Improve mobility robustness: UE-based Mobility can reduce the latency of handover decision. | Medium UE perform condition evaluation, and needs to evaluate up to 8 cells and 16 measId Need to store up to 8 candidate cell configurations | Medium Network needs to reserve resource up to 8 candidate cells） Note: resource efficiency is bad | Yes |
| LTM | NR | 18 19 | Reduce interruption, latency, and signalling overhead | Medium UE need to support L1 measurement and report UE may need to support Early TA acquisition or UE based TA measurement RACH-less can be achieved Need to store up to 8 candidate cell configurations | Medium Network needs to reserve resource up to 8 candidate cells） Network needs to indicate TA and beam Network needs to handle L1 measurement and report Note: resource efficiency is bad | No |
| Conditional LTM | NR | 19 | Reduce interruption, latency, and signalling overhead Improve mobility robustness | Medium UE need to support L1 measurement and report UE may need to support Early TA acquisition or UE based TA measurement RACH-less can be achieved Need to store up to 8 candidate cell configurations UE perform condition evaluation, and needs to evaluate up to 8 cells and 16 measId (L3) | Medium Network needs to reserve resource up to 8 candidate cells） Network needs to indicate TA Network needs to handle L1 measurement and report Note: resource efficiency is bad | No |

Note that LTE RACH-less mobility is only applicable for small cells or the scenario that UL Tx timing is not changed, while LTE Make-Before-Break mobility is only applicable for intra-frequency mobility scenario.

Following is the applicability of LTE / NR mobility solutions to 6G:

- The Rel-8/15 legacy HO of PCell change should be supported as baseline, due to low complexity at the UE and the network.

- RACH-less should be supported, especially for small/dense cell deployment.

- Make-Before-Break could be dropped, due to the dual Rx requirement for UE and there is no commercial deployment since Rel-14.

- DAPS can be dropped, due to the high complexity at the UE. In TR 38.914, requirements for mobility interruption time are as follows: “Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.” Since 0 ms interruption time is for mobility between TRxPs of a single base station while DAPS is for mobility between gNBs, there is no strong motivation to consider DAPS for 6G.

- CHO should be supported, due to the low complexity for the UE RF.

- LTM (including conditional LTM) should be supported for the low interruption and signalling overhead.

Note: We should simplify the mobility solution if multiple options are considered, i.e., unified mobility solution as discussed in section 2.2.

Lesson learnt from 5G (1): Too many mobility solutions were introduced in 5G which increased standards work load, UE/network complexity and also led to market fragment, especially some solutions are too complex to UE implementation which should be avoided.

Proposal 1: In 6G RAN2 study on mobility, compared to 5G, RAN2 should aim to reduce the options for 6G mobility design. Following NR mobility features can be used as the starting point for 6G study: L3 handover and LTM, including sub-features like RACH-less /early sync and conditional mobility (CHO and conditional LTM).

## Unified mobility

As analyzed in previous section, 5G supports two independent mobility solution directions: L3 mobility and LTM. This incurs complexity at both UE and network side.

Firstly, the supported features and the configuration restrictions for L3 mobility and LTM are different:

- LTM supports early UL/DL sync and subsequent mobility which can reduce handover delay and interruption, but L3 mobility does not support these features.

- Conditional LTM can be triggered based on L1 and L3 measurement, but CHO can only be triggered based on L3 measurement.

- In L3 RRM measurement, measurement configuration and L3 mobility configuration are decoupled, but L1 LTM measurement can only be configured for LTM candidate cell.

There is also large signalling overhead to support two set of mobility features in NR, e.g. CHO and LTM are configured by two independent RRC IEs. For the same candidate cell of CHO and LTM, duplicate configurations are provided to UE in CHO config and LTM config separately.

Lesson learnt from 5G (2): 5G supports two independent mobility solution directions: L3 mobility and LTM. This incurs complexity at both UE and network side.

Therefore in 6G, it is beneficial to unify L1 and L3 mobility to minimize signalling overhead and complexity from both UE and network side.

Following study areas can be considered:

- Unified Mobility Configuration for L3 handover and LTM

- Unified Trigger for L3 handover and LTM (e.g., NW triggered via L2 or L3 signaling, measurement report trigger event, etc.)

- Unified feature supported for L3 handover and LTM (e.g., early UL/DL sync, subsequent mobility, etc)

Figure 1: Unified L1 and L3 mobility

Proposal 2: In 6G RAN2 study on mobility, RAN2 to study unified mobility framework by considering following directions: L1/L3 resource configuration, report configuration, triggering of mobility, and unified feature set.

## Unified RRM

NR RRM model is shown in Figure 2 below. The design of RRM measurement is lack of system-level consideration in 5G life-cycle, and following issues are observed:

- Redundant UE measurement behaviors: there are many similarities between measurement tasks, e.g. L3 mobility measurement and L1 mobility measurement.

- Duplication between measurement tasks, e.g. L1-RSRP measurement for BM and L1-RSRP for mobility.

- Complicated UE measurement behaviors, e.g. sharing scheme between measurement tasks.

Figure 2: NR RRM model

Lesson learnt from 5G (3): The design of RRM measurement is lack of system-level consideration in 5G life-cycle as there are redundant UE measurement behaviors, duplication between measurement tasks, measurement configurations and resource configurations which make UE measurement behaviors complicated.

In 6G, it is beneficial to study how to support:

- Cross-Layer Integration for Same Function: L1-RSRP in LTM and L3-RSRP

- Cross-Function Integration for Same Layer: L1-RSRP in MIMO and L1-RSRP in LTM

- Cross-Layer Integration for Cross-Function: L1-RSRP in MIMO and L3-RSRP

Aim to archive:

- Unified Measurement Resource Configuration for L1 and L3 measurement

- Unified Measurement Report Configuration for L1 and L3 measurement (including unified measurement events, etc)

Proposal 3: In 6G RAN2 study on measurements, RAN2 to study simplified RRM architecture considering L1 and L3 measurement and multiple cells, with potential collaboration with RAN1 and RAN4 on

- Cross-Layer Integration for Same Function: L1-RSRP in LTM and L3-RSRP

- Cross-Function Integration for Same Layer: L1-RSRP in MIMO and L1-RSRP in LTM

- Cross-Layer Integration for Cross-Function: L1-RSRP in MIMO and L3-RSRP

## Idle mode mobility

TR 38.914 has the following definition about control plane latency: “Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state)”. Although the number of RRC states to be supported in 6G is to be discussed, it is assumed that 6G should at least support a “battery efficient” state (e.g. similar to RRC_IDLE / RRC_INACTIVE state in NR), and we refer to such state as “idle mode” in this section.

Mobility in idle mode is mainly related to cell (re)selection. As discussed in companion contribution [3], we assume a cell concept can be studied in 6G. The basic operation for cell reselection is that UE evaluates and ranks cells based on measurement results as well as network configuration (e.g. frequency priorities, offsets for frequencies / cells). Such basic principle can be the starting point for 6G study.

In 5G, some UE behaviors are specified for setting the highest / lowest priority according to some capabilities / features e.g. HSDN, MBS, NR sidelink, V2X sidelink. An excerpt from TS 38.304 is copied below:

When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform ranging/sidelink positioning, the UE may consider the frequency providing ranging/sidelink positioning configuration to be the highest priority.

Although feature specific frequency prioritization provides operator some tools to control the cell reselection, it is mainly introduced in 5G in an ad hoc manner and it is difficult for operator to set the preferred frequency when multiple features are supported in the network.

In 5G, Cell Individual Offset (CIO)Neighbor cell offset can be configured for neighbour cells and it will affect cell ranking. Depending on network’s configuration, such CIO can be bonus / punishment (value can be negative or positive) for cell ranking.

Lesson learnt from 5G (4): In 5G cell reselection, there is lack of unified control for feature specific frequency prioritization and cell individual offset which increased the complexity, and also difficult to use.

In 6G, multiple features (including new features like AI / ML and sensing) are expected to be introduced. Existing reselection priority handling is done at the UE side, and it handles features separately. For the case that a cell supporting multiple features, operator has no good control in weighting different features and directing UEs to the mostly wanted carriers. Study is needed on how to have a unified control for feature specific frequency prioritization and cell individual offset.

In addition, as discussed in companion contribution [3], RAN2 should study the concept of the 6G “Cell” additionally taking into account the following two aspects:

- Whether a 6GR Cell can comprise multiple carrier frequencies across one or multiple frequency bands;

- Whether a 6G can include the flexible combinations of the DL frequencies and UL frequencies that are not necessarily located on the same frequency band.

Consequently, cell reselection operation in a 6G cell with flexible spectrum utilization should be also studied.

Proposal 4: In 6G RAN2 study on “idle mode” mobility, basic operations of 5G cell reselection can be a starting point for idle mode mobility in 6G, e.g., frequency/cell/feature based selection priority. RAN2 to further study how to make cell (re)selection mechanism more efficient and simpler in 6G, especially, take into account flexible spectrum utilization and multiple features support.

## Inter-RAT mobility and migration

For migration, RAN WG 6G SID RP-252912 [1] has the following objectives:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Migration options supported in 5G NR includes inter-RAT mobility / cell reselection, DSS, and MRDC. In 5G study (as from TR 38.801 [1]), various migration options are considered: Option 2, 3/3A, 4/4A, 5, 7/7A. Typical migration options are shown in Table 2 below.

Table 2: Migration options in 5G

| Option | Architecture | Commercialized |
| --- | --- | --- |
| Standalone (Option 2) Note: Inter-RAT mobility and DSS are migration options on top of standalone option. |  | Yes |
| EN-DC (Option 3) |  | Yes |
| eLTE (Option 5) |  | No |
| NG-ENDC (Option 7) |  | No |

The motivation to specify non-standalone architectures in NR (e.g. EN-DC) is to allow early deployment of NR, e.g. to cater for the smooth upgrade of 4G network and inter-operability between 4G and 5G. Consequently, several drops (ASN.1 freeze) were introduced due to the high load in RAN WGs, e.g. for NSA and SA. This causes both specification/network/UE complexity and additional CAPEX for operators. For example, EN-DC was shortly deployed in China, but was quickly phased out for switching to SA, which not only delayed SA deployment but also incurs more CAPEX. This is a lesson to be learned for 6G standardization. It was already agreed in RAN plenary#102 meeting that (in RP-233985 [2]) Release 21 is expected to be delivered with a single drop (i.e., a single code freeze).

Lesson learnt from 5G (5): Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which should be avoided.

Baseline migration options include standalone, MRSS, and inter-RAT mobility between NR-6G. RAN WG 6G SID RP-252912 [1] states that “RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s)”. MRDC, as a migration option, has the issue of complexity of tight interworking at network side and UE side (as discussed above), and also increases the significant WG workload, market fragmentation, hence it is challenging to consider MRDC for migration from 5G NR to 6G.

On the other hand, considering the supported high data rate in 5G, to avoid the impact on user experience, NR has improved the mobility interruption a lot with LTM, etc. We expect that the similar mechanisms will be introduced in 6G as well to meet the mobility requirements. However, the interruption and the data loss during traditional inter-RAT handover, e.g., 4G-5G cannot be avoided. We do see the requirements to improve the inter-RAT handover as well in order to avoid downgrade of the use experience. To minimize interruption and data loss during mobility between 5G and 6G (which is one drawback of inter-RAT HO), but avoid to introduce tight network requirement (e.g. DU complexity) and also high UE complexity, dual stack / registration can be considered as an alternative migration option, where data can be aggregated at core network.

Proposal 5: In 6G RAN2 study on inter-RAT mobility, RAN2 to consider solutions for interworking between 5G NR and 6G with similar performance as intra-RAT mobility, e.g. high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G.

## Service continuity of new services (AI and sensing) upon mobility

6G RAN architecture should have efficient support of new services (AI/ML and sensing) from day 1. There is large amount of data (e.g. training data, sensing data) exchanged between UE, RAN, and CN, as shown in Table 3 and Figure 3 below.

Table 3: Data transfer between UE, RAN and CN

|  | UE and RAN | UE and CN |
| --- | --- | --- |
| AI / ML | UE collects / reports training data (e.g., measurement results) for NW-side model training. | For AI / ML UE-side data collection, UE transfer its collected data for UE-side model training to its own OTT server via application layer (i.e., transparent to 3GPP). |
| Sensing | Sensing results measured by UE is reported to RAN for communication optimization. | Sensing UE reports sensing results to either SF in CN or to sensing client directly |

Figure 3: Data transfer between UE, RAN and CN

Different types of data could have different QoS requirements. For example, as discussed in Rel-19 AI/ML, training data for offline training is delay tolerant while data for inference is delay sensitive.

One task of mobility work is to ensure service continuity of user plane traffic. In light of above data transfer and processing requirements of AI/ML and sensing, service continuity for new services should be also considered. Besides the service continuity from service data perspective, service level continuity also needs to be considered. For example, for sensing, e.g. UE continuously senses objects during mobility (as from use case on advanced modern city transportation system in clause 7.10 of SA1 TR 22.870 on 6G use cases and service requirements, with UE on a car to monitor the environment in front of the car).

Proposal 6: In 6G RAN2 study on mobility, RAN2 to study service level continuity during mobility for different service types (including AI, sensing).

## NTN mobility

In 5G, NTN-NTN mobility solutions have primarily focused on single-orbit satellite systems, meaning that handover and cell reselection procedures do not consider the orbital characteristics of the target cell. However, with the evolution of NTN, multi-orbit satellite deployments (e.g., GEO/LEO hybrid configurations) have emerged as key commercial scenarios.

In multi-orbit NTN-NTN mobility scenarios, the distinct service characteristics associated with different satellite orbital altitudes directly impact UE mobility behavior, including HO and cell reselection procedures. GEO satellites, due to their stationary orbit and wide coverage area, result in fewer mobility events and reduced UE power consumption. However, GEO systems typically suffer from higher transmission latency and lower data throughput, which may limit support for latency-sensitive and/or high-throughput services.

LEO satellites, on the other hand, benefit from reduced propagation delay and enhanced data rates, enabling support for high-performance services. However, their rapid movement and smaller footprint lead to more frequent HO and cell reselection events, increasing mobility management complexity and UE power consumption.

Therefore, the diversity in orbital characteristics across different satellite altitudes introduces varying mobility behavior and service performance impacts in NTN-NTN scenarios. To effectively support mobility across multi-orbit deployments, enhancements to mobility procedures are required. These enhancements should incorporate the orbital information of the target cell.

Moreover, mobility between NTN and TN has not been sufficiently addressed in 5G. Existing mobility mechanisms, such as location-based CHO and time-based CHO, which are designed for NTN-NTN scenarios, may not be directly applicable to NTN-TN mobility. This is primarily due to limitations in the TN, such as the inability to provide accurate reference location information or the expected service end time, which are essential for location-based and time-based CHO, respectively.

Additionally, cell reselection procedures face challenges such as inconsistent mobility state estimation between NTN and TN and unnecessary frequency measurements in TN or NTN may be triggered, resulting in increased UE power consumption.

Lesson learnt from 5G (6): 5G NTN-NTN lacks the support of the multi-orbit scenarios, and mobility between NTN and TN (including cell reselection procedure) has not been sufficiently addressed in 5G.

To address these gaps, 6G should introduce enhanced NTN-TN mobility solutions to support seamless service continuity and optimized mobility performance in integrated NTN-TN deployments. Furthermore, orbital information of the target cell should be considered in HO and cell reselection procedures.

Proposal 7: In 6G RAN2 study on mobility for NTN scenario, 5G NTN mobility solution can be used as starting point. RAN2 further study, including:

- Incorporation of orbital information of the target cell in HO and cell reselection procedures for multi-orbit scenarios.

- Development of optimized NTN-TN mobility solutions to support seamless service continuity and efficient mobility in integrated NTN-TN deployments.

# Conclusion

In this contribution, we discuss intra 6G mobility in different RRC states, inter-RAT mobility and the support of the requirements on service level continuity upon mobility, and propose the following based on lessons learnt from 5G:

Lesson learnt from 5G (1): Too many mobility solutions were introduced in 5G which increased standards work load, UE/network complexity and also led to market fragment, especially some solutions are too complex to UE implementation which should be avoided.

Proposal 1: In 6G RAN2 study on mobility, compared to 5G, RAN2 should aim to reduce the options for 6G mobility design. Following NR mobility features can be used as the starting point for 6G study: L3 handover and LTM, including sub-features like RACH-less /early sync and conditional mobility (CHO and conditional LTM).

Lesson learnt from 5G (2): 5G supports two independent mobility solution directions: L3 mobility and LTM. This incurs complexity at both UE and network side.

Proposal 2: In 6G RAN2 study on mobility, RAN2 to study unified mobility framework by considering following directions: L1/L3 resource configuration, report configuration, triggering of mobility, and unified feature set.

Lesson learnt from 5G (3): The design of RRM measurement is lack of system-level consideration in 5G life-cycle as there are redundant UE measurement behaviors, duplication between measurement tasks, measurement configurations and resource configurations which make UE measurement behaviors complicated.

Proposal 3: In 6G RAN2 study on measurements, RAN2 to study simplified RRM architecture considering L1 and L3 measurement and multiple cells, with potential collaboration with RAN1 and RAN4 on

- Cross-Layer Integration for Same Function: L1-RSRP in LTM and L3-RSRP

- Cross-Function Integration for Same Layer: L1-RSRP in MIMO and L1-RSRP in LTM

- Cross-Layer Integration for Cross-Function: L1-RSRP in MIMO and L3-RSRP

Lesson learnt from 5G (4): In 5G cell reselection, there is lack of unified control for feature specific frequency prioritization and cell individual offset which increased the complexity, and also difficult to use.

Proposal 4: In 6G RAN2 study on “idle mode” mobility, basic operations of 5G cell reselection can be a starting point for idle mode mobility in 6G, e.g., frequency/cell/feature based selection priority. RAN2 to further study how to make cell (re)selection mechanism more efficient and simpler in 6G, especially, take into account flexible spectrum utilization and multiple features support.

Lesson learnt from 5G (5): Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which should be avoided.

Proposal 5: In 6G RAN2 study on inter-RAT mobility, RAN2 to consider solutions for interworking between 5G NR and 6G with similar performance as intra-RAT mobility, e.g. high performance inter-RAT mobility and aggregation of data flows at core network between 5G NR and 6G.

Proposal 6: In 6G RAN2 study on mobility, RAN2 to study service level continuity during mobility for different service types (including AI, sensing).

Lesson learnt from 5G (6): 5G NTN-NTN lacks the support of the multi-orbit scenarios, and mobility between NTN and TN (including cell reselection procedure) has not been sufficiently addressed in 5G.

Proposal 7: In 6G RAN2 study on mobility for NTN scenario, 5G NTN mobility solution can be used as starting point. RAN2 further study, including:

- Incorporation of orbital information of the target cell in HO and cell reselection procedures for multi-orbit scenarios.

- Development of optimized NTN-TN mobility solutions to support seamless service continuity and efficient mobility in integrated NTN-TN deployments.

# References

[1] RP-252912, NTT DOCOMO (Moderator), “New SID: Study on 6G Radio”

[2] RP-233985, TSG Chairs, “High-level Considerations for 6G Timeline”

[3] R2-2506774, Xiaomi, “Discussion on 6GR control plane protocol design”