3GPP TSG RAN WG2 #131bis		                                                      R2-2506786

Prague, Czech Republic, Oct. 13th – 17th, 2025

Source:	CATT, CBN

Title:	Considerations on 6G data transfer and AI framework

Agenda Item:	10.3.3

Document for:	Discussion and Decision

- Introduction

The study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility was started in 3GPP Rel-19. Several AI/ML use cases such as RRM measurement prediction and measurement event prediction have demonstrated performance gains over non-AI solutions. It is expected that AI/ML will play an important role in more scenarios in 6G for tackling fundamental challenges and achieving improvements in mobility robustness, energy efficiency, and other critical KPIs.

A new study item for 6G Radio has been approved in RAN#108 [1]. The related detailed objectives of data transfer design and AI/ML are:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3]

(8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we share our views on the data transfer design, AI/ML framework and basic procedure study for 6G air-interface.

- Discussion

2.1 Data transfer design in 6G

Background

AI/ML function was introduced in the mid-stage of 5G evolution without structure modification. For the large size AI/ML involved data transfer in both UL and DL, multiple solutions have been provided by companies without convergence in 5G.

For data collection of network side model, MDT report like mechanism was used. But for data collection of UE side model, multiple solutions were provided:

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

RAN2 has analyzed that aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved for data collection, MNO controllability and visibility over the collected data, etc. But down selection among these solutions was still difficult.

For model transfer/delivery, the sharing of dataset/model parameters from NW-side to UE or UE-side training entity was discussed by RAN1 for two-sided models, and two alternatives for the sharing of dataset/model parameters are provided by RAN1. And how the data (including e.g. the dataset/model parameters) are transferred between gNB and NW dataset/model parameters collection entity (OAM/CN) in Alternative 1/2, if needed, is up to RAN3/SA2/SA5.

- Non-Over-The-Air (non-OTA) approach:

  - gNB -> NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)

- Over-The-Air (OTA) approach:

  - gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)

In 5G discussions, whether it is uplink AI/ML data collection solution or downlink AI model transfer/delivery solution, none of the schemes can converge. There are some limitations based on legacy CP/UP solution for large volumes of data in both UL and DL. Therefore it is necessary to study new mechanism to avoid the issues that arise in 5G.

Observation 1: There are some limitations based on legacy CP/UP solution for large volumes of data in both UL and DL of 5G AI/ML, to study new mechanism to avoid the issues that arise in 5G is necessary.

Coordination with SA2

SA2 also has work task on studying the potential need and framework of introducing a data plane for e.g. AI/ML data and sensing in 6G. In SA2#170 meeting, there is no progress achieved in technical aspect. The primary dispute regarding the ‌WT#5 data framework‌ revolves around whether it should be led by ‌SA2‌ or ‌SA5.‌ And in SA#109 meeting, it is agreed that for 6G data framework, SA2 and SA5 each carry out 6G data-related work and collaborate across multiple working groups, with progress checks in March and June 2026 to avoid redundant design [2]:

TSG SA asks SA2 and SA5 to progress and collaborate (including with RAN2 and RAN3 where applicable) on 6G data related work tasks and taking the 5G working scope as the starting point. SA2 and SA5 should coordinate with SA3 for the data security and privacy requirements. Regular coordination and collaboration among the WGs are expected, including checks on the progress of each group for 6G data related work tasks in March 2026 (TSG SA#111) and/or June 2026 (TSG#112), to avoid the duplicate design for the same functionality and to develop system wide consistent solutions.

RAN2 should have close coordination with SA2 for the data transfer requirement, framework‌ and mechanism, if any progresses are achieved by SA2 meeting in the future.

Observation 2: RAN2 should have close coordination with SA2 for the data transfer requirement, framework and mechanism, if any progresses are achieved by SA2 meeting in the future.

6G Data transfer

In 6G Day1, we propose to introduce a data transfer plane other than legacy CP and UP. Compared with 5G, 6G will introduce more abundant services/use cases, with more complex scenarios/requirements. Transmission characteristics: large data volume, frequent transmission, and diverse start/end nodes for transmission.

Below are some analysis of Advantages and Disadvantages of introducing DP (Data Plane) for AI/Sensing in 6G Network:

      - Qos requirement

5G QoS can be configured by mapping to different 5QI values over the air interface RB, with existing radio bearer types including SRB and DRB. 5G protocol explicitly defines high-priority settings only for SRB0, SRB1, and SRB3, while the settings for SRB2, SRB4, SRB5 and various DRB are not clearly defined. Their priority levels can be configured by the network based on actual bearer content.

Facing the numerous service requirements that 6G may introduce, data can be carried by the following three methods:

- Option 1: Add new SRBs;

- Option 2: Use existing DRBs, or expand the number of DRBs for bearing;

- Option 3: Add new types of RBs.

SRBs are dedicated to transmitting control signaling between the UE and the NB, which have strict latency and reliability requirements to ensure smooth network control. There are a limited number of standard SRB types with fixed functions. Adding new SRB should be based on plenty discussion on the fixed function, applied scenario, QoS parameters. As the number of 6G AI/ML services increases, such discussions seem redundant if new SRBs are used for new services.

DRBs are used to carry user plane data, such as voice, video, and application data, which have diverse QoS profiles tailored to user services. DRBs are more flexible in quantity—multiple DRBs can be established for different services of a single UE, each with unique QoS configurations.

For AI/ML data in different scenarios/applications, there may be significant differences in QoS requirements, e.g., training data is not sensitive to latency and allows packet loss; model data does not allow packet loss and has a large data volume, etc. The introduction of a new plane in 6G can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

Observation 3: From QoS requirement aspect, introduction of a new plane can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

      - Routing and delay

The CP tunnel can only be established between UE and NB, and the UP tunnel can only be established between UE and UPF, the fixed start/terminate node may not fulfill the data transmission requirements of diverse AI/ML services. By introducing data plane with flexible start/terminate nodes, this issue can be resolved, settings can be made by flexibly configuring different start and terminate points of data transmission on the network side, e.g. data between UE and RAN node / CN / OAM, data between RAN nodes / relay(s), and data routing via RAN nodes. Take BM data collection as an example, UEs need to first report the data to gNBs, which then forward it to training nodes. In 6G, using the potential data plane for transmission can avoid multi-node forwarding. The gNB can directly perform transparent transmission of data, saving the time required for repackaging.

Especially, the AS data carried in AS bearer without intervention of NAS or back-and-forth delivery between RAN and CN become possible. This is benefit for the data transmission only involving air-interface, e.g. BM data collection.

Observation 4: From data generation and routing aspect, compared with legacy SRB/DRB, introduction of a new plane can more flexibly configure different start and terminate points of data transmission on the network side, and the delay due to data forwarding via intermediate node can be avoided.

      - Data size

Currently, for air interface content related to AI, most signaling can reuse existing procedures such as RRC/NAS signaling. The potentially added data plane only has effective solutions for large-volume data content, e.g. training data collection/report, model transfer.

From the perspective of data volume, regardless of which plane a large amount of data is transmitted through, it occupies air interface resources. Therefore, the total amount of data transmitted over the air interface will not change due to the introduction of potential data plane. Moreover, limited by the size of PDCP SDUs, the upper limit of individual PDCP SDUs is fixed irrespective of the RB type. This value is 9000 bytes in 5G, but may be expanded in 6G.

For data collection in existing standardized use cases, only the BM use case involves UEs collecting large amounts of data. In the Positioning use case, the existing LPP/NRPPa protocols are reused, and the reporting to LMFs is basically enhanced using existing signaling. For the unified processing of multiple use cases in the future, the potential data plane can terminate at RAN nodes or other nodes, e.g. NWDAF, LMF, and OAM, etc. Unified design for data transmission of multiple use cases, and future-proof for use cases added in later releases.

Observation 5: From use case scalability aspect, introduction of a new plane can unify design for large size data transmission of multiple use cases, and future-proof for use cases added in later releases.

Proposal 1: The 6G data transfer design should consider QoS requirement, routing and use case scalability.

Based on the analyses above, we propose to introduce a RAN data plane to support the large volumes of AI data collection and transmission, especially for model training, and AI/ML model transfer/delivery to enables flexible configuration of QoS and flexible start/end points for data transmission, and reduces the forwarding latency of data at intermediate nodes.

Proposal 2: RAN2 to support the data transfer via a RAN data plane for 6G large amount of new service data.

2.2 AI/ML framework for 6G

5G New Radio (NR) has pioneered the integration of AI/ML in wireless access network. In 5G NR AI/ML study, an AI/ML lifecycle management (LCM) framework—covering data collection, model training, management, and inference was introduced [3]:

Figure 1: Functional framework for AI/ML for NR Air Interface

Since this framework is improved from NG-RAN AI/ML framework figure and could be used for both AI/ML air-interface and AI/ML for mobility use cases, it could be considered as a starting point for 6G AI/ML framework study.

Proposal 3: Consider the 5G functional framework in TR38.843 as a starting point of study for 6G AI/ML framework.

Building upon this foundation, the 6G radio is expected to evolve an enhanced AI/ML framework with expanded capabilities to satisfy increasingly demands. The designed AI/ML framework should be future-proof and applicable to diverse emerging use cases with minimal use case specific modifications. Furthermore, the enhanced framework is anticipated to provide the following improvements:

- Advanced model training methodologies

The 5G AI/ML primarily focused on offline training, wherein the training dataset is generated or collected in advance, and the AI/ML model is trained using the pre-existing dataset. This training approach inherently limited the generalization capabilities in ‌time-varying wireless channel conditions. To mitigate performance degradation, performance monitoring mechanism was introduced in 5G AI/ML to detect the mismatches between AI functionalities/models and environments. When such mismatches occur, the system may fall back to the corresponding conventional non-AI/ML solutions.

In contrast, 6G can embrace online training to enable continuous model adaptation. Such adaptive learning will improve both generalization capabilities and continuity of AI/ML-driven functionalities. To fulfill these requirements, the enhanced AI/ML framework should incorporate consideration of latency-sensitive data transfer demands of online training operations.

- Continuity of AI/ML features

In 5G-A, for UE-sided AI/ML features, the procedure of applicable functionality reporting is used for determining whether UE-side AI/ML features can be used in the current cell. For mobility scenario, the target cell requests applicability reporting during or after handover. If the UE does not have a suitable AI/ML feature for target cell, non-AI/ML feature will be used. In the 6G AI/ML framework, how to enhance applicable functionality reporting can be considered to improve the continuity of UE-side AI/ML features. For example, by considering the application scope of associated ID, or by completing the availability judgment of AI/ML features in the target cell before handover and using fine-tuning, etc., to improve the adaptability of AI/ML features to the target cell.

Proposal 4: For 6G AI/ML framework, develop an enhanced LCM framework to enable future-proof framework applicable to diverse emerging use cases, incorporating:

- Advanced training techniques, e.g. online training

- Continuity of AI/ML features

2.3 Basic procedure of AI/ML

The current NR air interface procedures for AI/ML have been discussed. The NR LCM procedures are built on existing NR procedures and tailored to address the initial use cases.  In 6G it is important that AI procedures for 6G are developed taking the progress of 5G NR into account from the beginning and utilized to embrace the new uses of 6G.

In NR, the procedure of AI/ML consists of the following part including model training, applicability reporting, inference, monitoring and management.

Model training

For model training in NR, offline model training was adopted to minimize the spec impacts. Data collection for offline model training was discussed and specified in NR. For NW-side model, the data collection can be initiated by OAM or gNB. The UE can log the data in UE-side buffer and report to the NW and indicate data availability to NW when particular conditions are satisfied, e.g., logging buffer is full. For UE-side model, data collection configuration can be provided to the UE or released with or without UE request if configured by NW.

In 6G, offline model training can be studied in Day1 and data collection for NW-side and UE-side model in NR can be studied as starting point.

Online model training can be studied in 6G Day1 to facilitate both generalization capabilities and continuity of AI/ML-driven functionalities. It may bring potential impacts on signaling procedure for fine-tuning which can also be investigated in 6G Day1.

Applicability reporting

For UE-side model in NR, the NW can enquire the UE to report its supported functionalities via UE capability signaling. Afterwards, the network provides inference configuration based on UE supported functionalities. The UE can report its applicable/ inapplicable functionalities and subsequent reporting for applicability status changing of functionalities to network. Consequently, NW and UE get aligned on the applicability status of functionalities. The common procedure is depicted in the Figure below.

Figure 2: AI applicability reporting procedure in NR

In NR, NW is in control of the activation/deactivation of the functionalities. This main reason is that for use cases specified in NR, like BM and CSI prediction, the low accuracy of the AI/ML mainly brings deterioration on network performance, e.g., throughput in BM, which is more concerned by the network. In 6G, it may happen that the various use cases investigated are mainly concerned by UE.

Inference

In NR, NW is responsible to configure the inference configuration for UE-side model. For instance, NW configures CSI resources and CSI reporting for BM. The inference configuration may be different depending on various use cases. Moreover, in order to reduce the compute resources, it is beneficial to study the inference conditions for the potential diverse use cases in 6G.

Monitoring

Regarding performance monitoring for NW-side model, the NW plays the role of performance monitoring. And UE can provide the label data to NW. NW can make management decision without informing UE.

While for UE-side model, performance monitoring can be conducted by both UE and NW. For NW-side monitoring, the NW sends monitoring configuration to UE and UE can report measurement and inference output to NW afterwards. For UE-side monitoring, UE performs measurements based on network configuration and generates monitoring results by comparing inference output and monitoring data. Then, UE reports the monitoring results to NW.

Management

In NR, NW controls model (de-)activation is mainly for the reason that inaccuracy AI/ML brings deterioration on network performance, e.g., throughput for BM, which is more concerned by the network. However, in 6G, considering more diverse AI/ML use cases may be studied. It is reasonable to reconsider the UE-decision management on AI/ML functionality.

Proposal 5: Consider applicability reporting and inference configuration/reporting procedure in NR as a starting point and make necessary enhancements considering online training and UE-decision management in 6G Day1.

- Conclusion

In this contribution, we present our views on data transfer design, AI/ML framework and basic procedure study for 6G air interface. The following observations and proposals are provided:

Observation 1: There are some limitations based on legacy CP/UP solution for large volumes of data in both UL and DL of 5G AI/ML, to study new mechanism to avoid the issues that arise in 5G is necessary.

Observation 2: RAN2 should have close coordination with SA2 for the data transfer requirement, framework and mechanism, if any progresses are achieved by SA2 meeting in the future.

Observation 3: From QoS requirement aspect, introduction of a new plane can more flexibly address diverse QoS requirements for large amount data transfer and avoid the repetitive discussion process caused by defining e.g. new SRBs.

Observation 4: From data generation and routing aspect, compared with legacy SRB/DRB, introduction of a new plane can more flexibly configure different start and terminate points of data transmission on the network side, and the delay due to data forwarding via intermediate node can be avoided.

Observation 5: From use case scalability aspect, introduction of a new plane can unify design for large size data transmission of multiple use cases, and future-proof for use cases added in later releases.

Proposal 1: The 6G data transfer design should consider QoS requirement, routing and use case scalability.

Proposal 2: RAN2 to support the data transfer via a RAN data plane for 6G large amount of new service data.

Proposal 3: Consider the 5G functional framework in TR 38.843 as a starting point of study for 6G AI/ML framework.

Proposal 4: For 6G AI/ML framework, develop an enhanced LCM framework to enable future-proof framework applicable to diverse emerging use cases, incorporating:

- Advanced training techniques, e.g. online training

- Continuity of AI/ML features

Proposal 5: Consider applicability reporting and inference configuration/reporting procedure in NR as a starting point and make necessary enhancements considering online training and UE-decision management in 6G Day1.

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, Prague, Czech Republic, June 9 th -13 rd, 2025.

- SP-251261 LS on Guidance on 6G data related work tasks TSG SA To: SA2, SA5; CC: SA3, SA6, TSG RAN, RAN2, RAN3

- 3GPP TR 38.843 v18.0.0, Study on AI/ML for NR air interface.