3GPP TSG RAN WG2 #131bis          	                                                             R2-2506811

Prague, CZ, Oct 13th – 17th, 2025

Source:	CATT

Title:	Discussion on 6GR mobility

Agenda Item:	10.4

Document for:	Discussion and Decision

- Introduction

For 6G radio, the mobility aspects should be discussed, according to 6G_WG_SID [1].

- Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

The contribution is organized as the following. In section 2, we discuss the following aspects,

- General mobility principles

- Mobility Types

- Measurements

Our observations and proposals are summarized in Section 3.

- Discussion

  - General mobility principles

Based on SID [1], the following objectives are outlined:

- Mobility between 5G NR and 6GR

- a harmonized 6G Radio design for TN and NTN, including their integration

The design of mobility functions (including measurement mechanisms) shall align with these objectives. Accordingly, a unified 6G mobility framework must support both Terrestrial Network (TN) and NTN scenarios.

Notably, NTN has distinct characteristics compared to TN—such as cell mobility in NTN deployments. RAN2 should therefore conduct dedicated studies to identify mobility-related technical aspects affected by NTN-specific attributes. The goal of this analysis is to leverage NTN’s unique traits for improved mobility management, for instance by supporting location-based measurement reporting or CHO.

Therefore, it is proposed that,

Proposal 1:  6GR mobility (including measurement) supports the following scenarios:

- Mobility in TN, NTN(including NTN specific functions) and between them.

- Mobility between 6GR and 5G NR

According to SID [1], the following objective is included:

- Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

To prevent excessive configurations, a unified framework represents a viable solution. Within such a framework, a single configuration could be utilized by the network to provide different indications, enabling the UE to perform various functions. For instance, currently in 5G mobility it often duplicate the same configuration across separate functional settings—such as candidate cell configurations being specified independently in both CHO and LTM configurations, or RS configurations appearing separately in RRM measurement settings and L1 LTM configurations. This duplication increases configuration redundancy and elevates signaling overhead.

Observation 1: Key 5G mobility design lessons: Redundant design across 3GPP releases (for different handover types), distinct frameworks for measurement types (e.g., RRM, L1-level LTM), and Solutions to improve HO performance are often limited to specific handover types.

A unified framework would enable the shared use of identical configurations across multiple functions, thereby addressing these inefficiencies.

Proposal 2: Design a unified framework for handover and measurement mechanisms in 6GR.

LTM in NR already supports functionalities such as early DL synchronization, early TA (Timing Advance), and early CSI (Channel State Information) acquisition to reduce interruption time and improve throughput during handover. To further enhance the handover performance to improve the user experience and support delay-sensitive services, 6GR mobility should explore performance enhancements based on mechanism in LTM in NR as the baseline. These enhancements should aim to achieve further reductions in data interruption and improvements in throughput during handover procedures.

Proposal 3: Enhance mobility performance further in 6GR, with LTM solutions (e.g., early TA acquisition, early CSI acquisition) in NR as the baseline. Specific targets include:

- Minimizing mobility interruption time to the greatest extent possible

- Mitigating throughput fluctuations during the handover phase

  - Mobility Types

Scenarios

With the evolution from 5G to 6G, mobile communication networks are required to support more diverse application scenarios and user demands—unlike 5G, which primarily focused on balancing coverage, capacity, and basic low-latency needs, 6GR aims to achieve "ubiquitous, intelligent, and high-reliability" mobility management. However, existing 5G mobility solutions have shown limitations in adapting to scenario-specific characteristics: for example, dense user gatherings often lead to signaling congestion and handover delays, high-speed predictable movement (such as rail transit) struggles with frequent cell switching and latency fluctuations, fixed-area mobility lacks optimized resource allocation strategies, and delay-sensitive services (e.g., industrial control, remote surgery) have increasingly stringent requirements for handover stability.​

Against this backdrop, 6GR mobility design cannot rely on a "one-size-fits-all" framework. Instead, it must fully consider the differentiated needs of typical user scenarios and service types. These scenarios not only represent high-frequency real-world usage scenarios but also cover core challenges that 6GR needs to address—including dense user access, high-speed mobility, fixed-area resource optimization, and extreme low-latency services. Therefore, identifying and analyzing these key scenarios is a prerequisite for formulating targeted 6GR mobility solutions, ensuring that the network can provide stable, efficient, and tailored mobility support across different usage contexts.​

Figure 1

Observation 2: The following scenarios shall be considered as key use cases for 6GR mobility design,

- Case 1: High-UE-density public venues

- Case 2: High-speed scenarios with predictable trajectories

- Case 3: Fixed mobility range scenarios

- Case 4: Delay-sensitive service scenarios

CONNECTED state

In 5G NR, a diverse set of handover mechanisms has been introduced to address the complex mobility requirements of emerging use cases. These handover types are designed to optimize connectivity across varying network conditions, device speeds, and service demands.

To provide a comprehensive understanding of their practical implications, we analyze and compare the advantages and limitations of each handover type from multiple critical perspectives—including latency performance, signaling overhead, network resource consumption, robustness against mobility speed, and implementation complexity. The following table presents this comparative analysis:

|  | Resource efficiency | Mobility interruption time | HO Robustness | Implementation Complexity |
| --- | --- | --- | --- | --- |
| L3 handover | No pre-handover resource reservation | Long interruption time |  | Low Complexity |
| CHO | Need pre-handover resource reservation | Long interruption time | Improved Robustness | Low Complexity |
| DAPS | No pre-handover resource reservation | 0ms interruption time |  | High Complexity |
| LTM | Need pre-handover resource reservation | near 0ms interruption time |  | Medium Complexity |
| CLTM | Need pre-handover resource reservation | near 0ms interruption time | Improved Robustness | Medium Complexity |

Table 1

In 6GR, selecting appropriate handover mechanisms tailored to diverse scenarios is critical to meeting both network and UE requirements. Below is an analysis of optimal handover strategies for each key scenario:

|  | Key handover performance requirement | Applicable Solution |
| --- | --- | --- |
| Case 1: High-UE-Density Environments | Dense UE deployments bring constrained radio resources; handover schemes needing RAN pre-reserve resources for individual UEs worsen inefficiency. | NW controlled handover (e.g., L3 HO-like) |
| Case 2: High-Speed Mobility with Predictable Trajectories | Ensuring ultra-high-speed UE (up to 1000km/h) HO robustness is key and fixed trajectories need no subsequent HOs. | UE controlled handover (e.g., CHO-like) |
| Case 3: Fixed Mobility Range Scenarios | UEs with fixed mobility (e.g., commute, workplace) have predictable patterns; | UE controlled handover (e.g., CLTM-like) |
| Case 4: Delay-Sensitive Services | Delay-sensitive services need near-zero data interruption; (C)LTM (intra-DU ~0ms) is better than DAPS for 6GR. | NW controlled handover (LTM-like), UE controlled handover (e.g., CLTM-like) |

Table 2

Based on the above analysis, the following conclusions can be drawn:

Proposal 4: 6GR mobility supports the following handover types,

- NW controlled handover(e.g., L3 HO-like, LTM-like), with/without resource reservation

- UE controlled handover(e.g., CHO-like, CLTM-like), with/without subsequent handover

RRC IDLE/INACTIVE state

While the need to support RRC inactive state in 6GR remains a topic for further discussion, cell (re)selection behavior exhibits no fundamental distinction between RRC idle and RRC inactive states.

In both 4G and 5G, cell reselection relies on Radio Resource Management (RRM) measurements, which directly reflect channel quality—an objective and real-time indicator of link suitability. Consequently, the RRM-based mobility framework (e.g., R criteria, frequency based priority) for RRC idle/inactive states, as established in 5G, should be retained as the baseline for 6GR. This principle applies uniformly regardless of whether the network is terrestrial (TN) or non-terrestrial (NTN).

Proposal 5: The 5G cell reselection mechanism (e.g., R criteria, frequency based priority) is the baseline for 6G mobility in RRC idle state and RRC inactive state (if adopted).

RLM/RLF

RLM is utilized to monitor the quality of the serving link. In 5G NR, RLM encompasses the detection of physical layer issues, RACH problems, and RLC problem. Upon detecting an RLF, if AS security is activated and the UE is configured with at least one DRB, the UE shall trigger RRC reestablishment. otherwise, the UE transitions to the idle state.

For 6G NR, as the protocol specifications will undergo re-discussion, determinations regarding which problem detections should be included in RLM may be postponed. Similarly, the RRC procedures related to this mechanism require re-evaluation, and thus decisions on UE behavior upon RLF detection may also be deferred.

Proposal 6: Discussions on RLM/RLF may be initiated once substantive progress has been achieved in protocol design and RRC procedure development.

  - Measurements

5G NR includes three measurement types: RRM measurements, LTM L1 measurements, and L1 measurements for beam management/channel state information (CSI) reporting.

For 6G NR measurements, first clarify the required types—all three above are recommended to be retained, with justifications as follows:

- RRM measurements: Provide filtered results to avoid network misjudgments from occasional measurement fluctuations. While network-side filtering is proposed, it would add high complexity due to large neighbor cell counts; they also support legacy handovers (preferably via L3 measurements) to prevent ping-pong effects.

- LTM L1 measurements: Help select optimal neighbor beams when serving cell beam quality drops. Combined with L2 commands, they reduce LTM latency and avoid delayed handovers, with minimal data interruption during switching (making ping-pong effects less critical). The network can configure TTT to filter anomalies, and filtering via periodic reports (for limited candidate cells) can be left to network implementation.

- L1 measurements (beam management/CSI reporting): Enable optimal UE scheduling, boosting throughput and data transmission efficiency.

In 5G NR, these measurements use separate frameworks. To clarify these existing frameworks, we compare them across key aspects: Purpose, RS Configuration, Report Type, Configuration Signaling, and Report Signaling. The comparison is shown in the table below.

|  | Purpose | RS configuration | Report type | Configuration Signaling | Report signaling |
| --- | --- | --- | --- | --- | --- |
| RRM measurement | - For RRC connected/idle/inactive sate cell level mobility - For CA configuration | - SSB, - CSI-RS(Cell level configuration) | - event trigger, - periodic | MeasConfig in RRC message RRCReconfiguration | RRC |
| L1 measurement for beam management/channel state information reporting | - Beam management, - CSI acquisition - tracking | - SSB, - CSI-RS(BWP level configuration) | - event trigger, - periodic, - semi-Persistent, - aperiodic | CSI-MeasConfig in RRC message RRCReconfiguration | UCI |
| LTM L1 measurement | - For cell level mobility (LTM), - CSI acquisition (LTM) | - SSB, - CSI-RS(Cell level configuration) | - event trigger, - periodic, - semi-Persistent, | In LTM-Config-r18 RRC message RRCReconfiguration | - UCI - MAC CE |

Table 3

As the table shows, in 5G NR the UE needs separate measurement configurations, independent measurements (on different or same RS), and separate signaling to report results to the network—creating notable redundancy between measurement types. Thus, a unified measurement framework for 6G NR is critical to boost UE power efficiency, cut implementation complexity, and improve signaling efficiency.

In this framework, standardize configurations, triggering mechanisms, and reporting signaling where possible. At minimum, it should cover RRM and LTM L1 measurements (both for mobility, within RAN2’s scope).

L1 measurements (beam management/CSI reporting) fall mainly to RAN1 (discussions decided by RAN1). Integrating them into the framework may be challenging, as they differ greatly from the other two types in RS configuration, report types, and reporting signaling (per the table).

Observation 3: The feasibility of integrating L1 measurements (for beam management/channel state information reporting) into the unified measurement framework shall be determined by RAN1.

RRM measurements and LTM L1 measurements can share at minimum RS configurations and certain report type configurations. RAN2 may consider designing unified RS and report type configurations for these two measurement types. Regarding reporting signaling:

- Direct reporting to the CU should be supported, given the CU-DU split architecture.

- Security is critical: Since RRM measurements include cell-level data with PCI information, they must be reported with security protection to prevent location exposure—making RRC signaling-based reporting necessary.

- For neighboring cell L1 measurements, MAC CE support is essential to ensure scheduling flexibility, especially for event-triggered reporting.

- UCI-based reporting is unnecessary for neighboring cell L1 measurements, as MAC CE already meets latency requirements.

Therefore, it is proposed that,

Proposal 7: RAN2 design a unified framework for RRM measurements and LTM L1 measurements, encompassing the following aspects:

- Unified configuration of measurement resources (e.g., SSB, CSI-RS)

- Unified types of measurement reports (e.g., event-triggered, periodic reports)

- Support for measurement report transmission via both MAC CE and RRC signalling

- Conclusion

Observation 1: Key 5G mobility design lessons: Redundant design across 3GPP releases (for different handover types), distinct frameworks for measurement types (e.g., RRM, L1-level LTM), and Solutions to improve HO performance are often limited to specific handover types.

Observation 2: The following scenarios shall be considered as key use cases for 6GR mobility design,

- Case 1: High-UE-density public venues

- Case 2: High-speed scenarios with predictable trajectories

- Case 3: Fixed mobility range scenarios

- Case 4: Delay-sensitive service scenarios

Observation 3: The feasibility of integrating L1 measurements (for beam management/channel state information reporting) into the unified measurement framework shall be determined by RAN1.

General mobility principles

Proposal 1:  6GR mobility (including measurement) supports the following scenarios:

- Mobility in TN, NTN(including NTN specific functions) and between them.

- Mobility between 6GR and 5G NR

Proposal 2: Design a unified framework for handover and measurement mechanisms in 6GR.

Proposal 3: Enhance mobility performance further in 6GR, with LTM solutions (e.g., early TA acquisition, early CSI acquisition) in NR as the baseline. Specific targets include:

- Minimizing mobility interruption time to the greatest extent possible

- Mitigating throughput fluctuations during the handover phase

Mobility Types

Proposal 4: 6GR mobility supports the following handover types,

- NW controlled handover(e.g., L3 HO-like, LTM-like), with/without resource reservation

- UE controlled handover(e.g., CHO-like, CLTM-like), with/without subsequent handover

Proposal 5: The 5G cell reselection mechanism (e.g., R criteria, frequency based priority) is the baseline for 6G mobility in RRC idle state and RRC inactive state (if adopted).

Proposal 6: Discussions on RLM/RLF may be initiated once substantive progress has been achieved in protocol design and RRC procedure development.

Measurements

Proposal 7: RAN2 design a unified framework for RRM measurements and LTM L1 measurements, encompassing the following aspects:

- Unified configuration of measurement resources (e.g., SSB, CSI-RS)

- Unified types of measurement reports (e.g., event-triggered, periodic reports)

- Support for measurement report transmission via both MAC CE and RRC signaling

- References

- RP- 252912 New SID: Study on 6G Radio, RAN#108