3GPP TSG-RAN WG2 Meeting #131bis 	 R2-2506835

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.8.3

Source:	vivo

Title:	Discussion on 304 Open Issue 2 regarding PDCCH Repetition

Document for:	Decision

# Introduction

To improve NR downlink coverage in FR1 band (including both FR1 band and DR1-NTN band), repetition of PDCCH common search spaces (CSS) is supported by RAN1, For both CSS for SI repetition and CSS for paging, the UE is allowed to receive PDCCH wihtin the same slot on two CSSs that are associated with the same search space linking ID (i.e. searchSpaceLinkingId-CE-r19).

In this contribution, we would like to provide our consideration of 304 Open Issue 2 regarding PDCCH repetition.

# Discussion

Currently, the descriptions of Paging and System Information (SI) reception in relevant 3GPP protocols—specifically 3GPP TS 38.304 and 3GPP TS 38.331—only reference a single search space for Paging and SI reception. For example, the corresponding text for paging is cited as below,

## TS 38.304 7.1 Discontinuous Reception for paging The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4]. When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF. When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

This specification constraint creates a critical misalignment with the PDCCH repetition technology introduced by RAN1: The RAN1 design enables the UE to use two linked search spaces for PDCCH soft combining, but the existing RAN2 specs do not account for the second associated search space in paging reception scenario.

Observation: The RAN1 design enables the UE to use two linked search spaces for PDCCH reception for paging reception with soft combining, but the existing RAN2 specs do not account for the second associated search space in paging reception scenario.

Without explicit specification support for the two linked search spaces, we believe that the UE cannot fully leverage the PDCCH repetition mechanism for paging reception.

Therefore, we propose revising the relevant clauses in RAN2 specifications. For example as shown in Figure 1, for paging reception, RAN2 captures that the UE can receive PDCCH for paging based on the paging common search space and its corresponding linked search space (with the same search space link ID), if configured.

Figure 1: PDCCH repetition for paging

During the open issue discussion phase, we noted that the 304 spec rapporteur held a view that the current generic text in 304 spec can cover the PDCCH repetition case. This is because, by nature, the UE will consider the search space linked with pagingSearchSpace as pagingSearchSpace. However, we remain unconvinced. In the 304 spec, the field pagingSearchSpace is a specific SS#ID, not a generic term. In our understanding, as per NR design, the pagingSearchSpace is identified by a unique SS#ID, and a single SS#ID cannot correspond to two search spaces. We believe linked SS cannot be classified as pagingSearchSpace. One more thing, the rapporteur thinks UE should monitor two SSs with the same linked ID. We remember that RAN1/RAN2 never agreed that UEs must listen to both linked SS for paging. It is totally up to the UE implementation. For example, if the UE can successfully decode the DCI via the pagingSearchSpace, it does not need to conduct soft combining at all based on the copy sent on the linked SS.

We believe a simple revision will ensure consistency between RAN1’s PDCCH repetition design and RAN2’s protocol specifications, enabling the intended robustness improvement for paging reception in NR R19.

Proposal: (304-2) For paging reception, RAN2 captures that PDCCH monitoring occasion(s) are determined according to pagingSearchSpace and its associated common search space (if configured).

For the Proposal, a corresponding TP is given in the Annex for reference.

# 3 Conclusion

In this contribution, we have discussed the remaining issues regarding PDCCH repetition. And we make the following observation and proposal,

Observation: The RAN1 design enables the UE to use two linked search spaces for PDCCH reception for paging reception with soft combining, but the existing RAN2 specs do not account for the second associated search space in paging reception scenario.

Proposal: (304-2) For paging reception, RAN2 captures that PDCCH monitoring occasion(s) are determined according to pagingSearchSpace and its associated common search space (if configured).

# 4 Annex

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. A L2 U2N Relay UE monitors the paging occasions of its PC5-RRC connected L2 U2N Remote UEs. In this case, the DRX cycle and UE ID mentioned in this clause refer to those of the L2 U2N Remote UE.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS. However, if a L2 U2N Relay UE in RRC_INACTIVE state receives a CN initiated paging for a L2 U2N Remote UE, the L2 U2N Relay UE does not move to RRC_IDLE state.

NOTE 0a:	The L2 U2N Remote UE does not need to monitor the PO in order to receive the paging message.

NOTE 0b:	While the SDT procedure is ongoing in RRC_INACTIVE state, the UE monitors the PO in order to receive only the Short Message as specified in TS 38.331 [3].

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace and its linked searchSpace (if configured), as specified in TS 38.213 [4] and TS 38.331 [3], and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, or when SearchSpaceId other than 0 is configured for SearchSpace linked with pagingSearchSpace, if configured, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

……… the unrelated part is omitted ………