3GPP TSG-RAN WG2 Meeting #131bis                            R2-2506886

Prague, Czech Republic, Oct. 13th-17th, 2025

Source:	China Telecom

Title:	Discussion on Topology 2 for Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- 1.Introduction

In accordance with the work for Ambient IoT (Internet of Things) in NR in Rel-19, the specification of Device 1 for use in Deployment Scenario 1 with Topology 1 has been completed. In Rel-20, RAN2 should start the discussion on Topology 2 where the UE acts as an intermediate node under NW control. Actually, we have already completed basic discussions on Topology 2 during the SI phase, and the specification can be found in TR 38.769. At the last RAN#109 meeting, RAN updated the following constraints related to Topology 2 for RAN2 work [1].

RAN2 - The objective includes specification of necessary Uu procedures to enable the A-IoT operations. - RRC CONNECTED UE reader only, including RLF and handover, without specific enhancement for other temporary out-of-connection scenarios. - A-IoT radio resource of the UE reader is only valid within the serving cell. - Resource allocation to the UE Reader shall be via Uu RRC signaling.

In this contribution, we would like to share our opinions on the related aspects of D2T2 for Ambient IoT, based on the above WID.

- 2.Discussion

For the architecture/protocol stack options of Topology 2, according to the progress of SA, the work task for supporting A-IoT services in Topology 2 is specified by a new SID as follows [2].

WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity. NOTE 3: Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

Correspondingly, to align with SA’s decision, the RAN plenary has also updated the WID.

- Specify support for Deployment Scenario 2 with Topology 2 with intermediate UE as Reader under the following conditions [RAN2-led]: - Specify RRC-based architecture, in coordination with SA2. - The following are not supported: - Coordination between UEs operating as UE readers - New Uu protocol layer(s) - Simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band This objective begins after RAN#109 (September 2025), and after RAN1#111 (March 2026) for RAN1.

Therefore, RAN2 only needs to discuss the RRC-based solution and define the relevant RRC procedures to enable A-IoT operations in the Rel-20 phase.

Proposal 1: RAN2 should focus on the RRC-based design to enable the A-IoT Topology 2 operations.

During the SI stage, the following alternatives were studied regarding the radio resources allocation request.

- Alternative 1: BS configures/allocates a set A-IoT radio resource to the UE reader, in response to a request from the UE reader; and/or - Alternative 2: BS configures/allocates a set A-IoT radio resource to the UE reader, based on the service request from CN.

The CN indication-based approach is efficient and direct for allocating A-IoT radio resources. Considering the support for the DO-A traffic type in the Rel-20 phase, A-IoT device may actively send a D2R message to the UE reader for data transmission. In this case, the occupied radio resources can be indicated by the CN or in response to a UE reader request. Therefore, RAN2 should support the above two radio resources allocation mechanisms. The UE request-based approach has certain impacts on RAN2, and a new Uu signalling should be defined accordingly.

Proposal 2: RAN2 to confirm support for both the CN indication-based and UE request-based mechanisms regarding the radio resources allocation request.

For the issue related to UE reader temporary out-of-connection, WID has restricted to include RLF and handover (HO), with no other specific enhancements. Regarding the Uu RLF handling of UE reader, the UE reader can follow the legacy behaviour to initiate RRC re-establishment procedure and suspend the transmission of all RBs in the Uu interface. For the A-IoT air interface, the ongoing A-IoT service shall be suspended as well. After the re-establishment procedure, the UE reader can follow the configuration of the network to resume/re-initiate/stop the A-IoT service.

As for HO, it is quite likely that the potential HO of the UE reader is under the control of the gNB, since there is no clear need to support the coexistence of A-IoT and UE triggered HO such as CHO. Thus, a simple way might be that the UE reader suspends the A-IoT service during the HO. After HO to the target cell, the UE reader can just follow the configuration of the network, which is similar to the above RLF case.

Proposal 3: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 4: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure. After HO to the target cell, the UE reader follows the configuration of the network.

For the UE reader selection, it is up to RAN3 and SA2 on whether BS or CN selects the UE reader(s) and what information is shared between BS and CN. From RAN2’s perspective, before selecting a UE reader and sending the A-IoT radio configuration to a UE reader, the gNB shall get the UE radio capability of A-IoT and decide the configuration accordingly. Regarding the A-IoT UE radio capability as a UE reader, RAN2 can further discuss the details in the stage-3.

Proposal 5: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the stage-3.

- 2.Conclusion

Based on the above analysis, the following proposals are given for Ambient IoT Topology 2.

Proposal 1: RAN2 should focus on the RRC-based design to enable the A-IoT Topology 2 operations.

Proposal 2: RAN2 to confirm support for both the CN indication-based and UE request-based mechanisms regarding the radio resources allocation request.

Proposal 3: For Uu RLF handling of UE reader, the UE reader follows the legacy behaviour to initiate RRC re-establishment procedure in the Uu interface.

Proposal 4: For HO of the UE reader, the UE reader suspends the ongoing A-IoT service during the HO procedure. After HO to the target cell, the UE reader follows the configuration of the network.

Proposal 5: For A-IoT Topology 2, the gNB shall get the UE radio capability of A-IoT before sending the A-IoT related configuration. Further details could be discussed in the stage-3.

# References

- RP-252894 Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2.

- SP-250834, NEW SID on Architecture support of Ambient power-enabled Internet of Things - Phase 2.