3GPP TSG RAN WG2 Meeting #131bis	R2-2506907

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda Item:	8.8.2

Source:	CMCC

Title:	Remaining issues on broadcast service for NR NTN

Document for:	Discussion, Discussion

Introduction

In last RAN2 meeting, there were some discussions on support of broadcast service and some agreements were achieved as follows [1]:

Agreements:

2.	We don’t introduce new AS mechanisms to associate the ISA with relevant neighbour frequencies to perform MBS frequency prioritization

3.	It is up to UE implementation how to use the ISA definition in SIB/USD to skip MCCH acquisition when it is outside the ISA of the MBS broadcast service(s) it is interested in (can further check the wording for the change in 38.300 and 38.331)

4.	The Intended Service Area in the Service Announcement can be used for frequency prioritization in MBS NTN. Introduce a corresponding statement in TS 38.304 (FFS on the actual wording and whether it’s normative text or a note)

Agreements:

1.	RAN2 understands that for MII reporting when an ‘intended service area’ is signalled the term ‘broadcast service area’ refers to the ‘intended service area’ (no spec impact)

2.	RAN2 understands that a UE may initiate MII upon entering or leaving the ISA (no spec impact)

3.	RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is applicable for reception within the entire cell area, with legacy behavior applicable (FFS whether we capture this in the spec)

Therefore, in this contribution, we would like to provide some further considerations on the remaining issues for broadcast service in NTN system.

Discussion

In last meeting, whether provide neighbouring cell information for service continuity is raised but postponed to this meeting.

R2-2505822 Support for broadcast services in NR NTN Ericsson discussion Rel-19 NR_NTN_Ph3-Core Proposal 1 Broadcast future neighbour cells providing the same MBS service(s) as the serving cell. Proposal 2 In MBS NTN, a UE is informed whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters. Can be discussed in the next meeting

For proposal 1, per the current specification, a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell can already be provided in the MBSBroadcastConfiguration message for TN MBS. Therefore, we could just reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

Proposal 1: Reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

For Proposal 2, a new indication (e.g. mtch-StaticConfigSwitch in [2]) is introduced to indicated the UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters to minimize service interruptions and lower UE power consumption due to frequent re-acquisition of information upon each cell change. From our perspective, such optimization is based on the per geographical area MTCH settings mentioned in [2]. However, if the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Observation 1: If the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Proposal 2: The optimization that inform UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters is not needed.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on MBS broadcast enhancements for NTN and have the following observations and proposals:

Proposal 1: Reuse the existing mbs-NeighbourCellList-r17 in MBSBroadcastConfiguration to provide the future neighbour cells providing the same MBS service(s) as the serving cell for NTN MBS.

Observation 1: If the MTCH settings are per geographical area, then the new indication is not needed. And if the MTCH settings varies in different areas/cells, the significance of introducing the new indication information is not obvious.

Proposal 2: The optimization that inform UE whether the cell replacing the serving cell provides an MBS service with identical MTCH configuration parameters is not needed.

Reference

- R2-2506203 Report from Break-out session on NR-NTN and IoT-NTN

- R2-2505822	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core