3GPP TSG-RAN WG2 Meeting #131bis 	R2-2506915

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda Item:	8.2.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT remaining issues

Document for:	Discussion and Decision

Introduction

In this contribution, we analyze the remaining issues and provide our opinions.

Discussion

2.1 Issue 1-3

| Subgroup: Paging message content |  |  |
| --- | --- | --- |
| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., how many bits, taking into account of CT4 and SA2 inputs. - Relevant agreements: - Note: SA2 already agreed the filtering information and captured it in clause 5.8 in 23.369, this may enable some extent of RAN2 discussion before their feedback.8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion. - However, in Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Device Permanent ID Length, indicating SA2 identified requirements for longer device ID, and the rapporteur understand this will impact the length of paging ID as well, e.g., 600 bits are needed instead of 256 bits. Therefore, in Oct meeting, RAN2 needs to discuss how to reply the SA2 LS, and how to handle the impact to paging ID length field. - Status in running CR: the field name is captured in 6.2.1.1 with the detailed format. | To be discussed by company contributions |

The current paging message size (excluding the paging ID) is fewer than 200 bits, which includes more than 20 bits for physical layer parameters from RAN1 and more than 30 bits for RAN2 overhead. Although the maximum message size is 1000 bits and segmentation for the paging message is not required.

From RAN2 signalling design perspective, it is feasible to extend the permanent device ID to 496 bits if required by SA2. According to SA2 estimation, the filtering information may be extended to 600 bits if the permanent device ID is extended to 496 bits. Therefore, the paging ID length field should be capable of indicating 600 bits. The paging ID length field also should be extended to 10 bits.

However, this extension would increase the overall the paging message size, which will reduce the inventory efficiency and decrease the coverage performance of A-IoT.

RAN2 #131 agreements:

For forward compatibility:

-	Paging message can be extended to add more fields at the end of the message in further releases, and Rel-19 devices ignore the extension parts added in future release instead of dropping the whole message, without extension indication. Future extension using the same message type is not supported for R2D messages other than paging message.

-	No version bit will be introduced

-	Remove the R-field in paging message from the running CR

-	Use 3-bit R2D message type.

For forward compatibility, RAN2 has achieved the above agreements [2]. It is noted that if the paging ID is extended to 600bits, the scalable space for the R20 paging message would be only about 200 bits, significantly limiting the future scalability.

Therefore, from RAN2 signalling perspective, it is feasible to extend the paging ID to 496 bits. However, from RAN2 perspective the less overhead the better.

Proposal 1: RAN2 thinks it is feasible to extend the paging ID to 496 bits from signalling perspective. However, from RAN2 perspective, the less overhead the better.

Proposal 2: The paging ID length field should be extended to 10 bits.

2.2 Issue 1-7

| Issue 1-7: Security parameter | How to include the security parameters in Paging message. - Based on SA3 LS in S3-252392, it’s understood that security is a mandatory feature to be supported by the Rel-19 devices. But companies also raised question during online discussion whether the security parameters can be optional, for instance the feature can be activated/controlled by network. And companies also mentioned that the signaling overhead of the current paging message design is an important aspect from RAN2 point of view. Then how to capture the security parameter has been postponed and expected more discussion in Oct meeting. In this case, this issue is classified as “to be discussed by contribution”, and the rapporteur would like to suggest companies also to consider more from RAN2 perspective when designing signaling in their contributions, in terms of e.g., signaling compatibility, flexibility, signaling overhead, and scalability. - SA3 provided more information about the security design in LS S3-252933 in Aug meeting, which should be take into account. - Relevant agreements: - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not. - Reply to SA3 (The LS is approved in R2-2506465) - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - Indicate space pressure from all the WG - Status in running CR: not captured yet | To be discussed by company contributions |
| --- | --- | --- |

According to SA3 requirements, the authentication is a mandatory feature for A-IoT devices. Furthermore, as agreed in TS 33.369 and TR 33.713, we understand that the AIoTF shall authenticate the device for each inventory procedure, inventory and command procedure. So the security parameter should be mandatory field. So there is no need to add one bit to indicate the presence of the security parameter.

TR 33.713 7 Conclusions …… 7.5 Conclusion on KI #5: Authentication in Ambient IoT service …… 2. The following principles are taken as the conclusion for KI#5: Inventory-only Procedure: - Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported. NOTE 1: Details of the authentication procedure (e.g., one-way, parameter(s) used, etc.) are to be resolved during normative phase. Inventory and Command Procedure: - Mutual authentication between the AIoT device and the 5G network shall be supported. - The authentication procedure is based on a challenge-response mechanism. NOTE 2: Details of the mutual authentication procedure and details of the challenge-response mechanism(e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

Proposal 3：The 128 bits security parameter should be mandatory in paging message according to SA3 agreements.

The security parameter are self-byte aligned and R bits is not needed. The specific format is described in Figure 1 and 2.

Figure 1: MAC PDU of A-IoT Paging message indicating CBRA

Figure 2: MAC PDU of A-IoT Paging message indicating CFA

Proposal 4: The 128 bits security parameter should be added after the paging ID within MAC PDU of A-IoT Paging message.

2.3 Issue 4-6

| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. - Status in running CR: captured. | To be discussed by company contributions |
| --- | --- | --- |

Regarding integrity check failure case, we don’t believe the device needs to respond with a 0 SDU to the reader. If the device cannot successfully decode the command request due to an integrity check failure, replying with a 0 SDU would be ineffective, as the reader would not be schedule another D2R transmission. In this scenario, the device should not send any response. Instead, the reader should be responsible for retransmitting the command request to obtain the D2R response from the device.

Proposal 5: Confirm that there are no other no data available cases except for the case of data arrival later.

Conclusions

In this contribution, we discuss the Rel-19 remaining issues. And the proposals are given as follows:

Proposal 1: RAN2 thinks it is feasible to extend the paging ID to 496 bits from signalling perspective. However, from RAN2 perspective, the less overhead the better.

Proposal 2: The paging ID length field should be extended to 10 bits.

Proposal 3: The 128 bits security parameter should be mandatory in paging message according to SA3 agreements.

Proposal 4: The 128 bits security parameter should be added after the paging ID within MAC PDU of A-IoT Paging message.

Proposal 5: Confirm that there are no other no data available cases except for the case of data arrival later.

References

[1] Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC Running CR).

[2] RAN2 #131 meeting report, ETSI MCC, Bangalore, India, 25-29, Aug. 2025.