3GPP TSG-RAN WG2 Meeting #131bis		R2-2506920

Prague, Czech Republic, Oct. 13th-17th

Title:	[Draft] Reply LS on the maximum supported AIoT NAS container length

Response to:	C1-255679 from CT1

Release:	Release 19

Work Item:	Ambient_IoT_solutions

Source:	Lenovo [To be RAN2]

To:	CT1, RAN3

Cc:	None

Contact person:	Jing HAN

hanjing8@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

RAN2 thanks CT1 for the LS on the maximum supported AIoT NAS container length.

RAN2 further discussed this issue with below analyses:

The R2D message relates to the command message is R2D Upper Layer Data Transfer message, and the maximum size is 125 bytes. In this message, the MAC header(s) occupies 6 bytes. And RAN2 would like reserve additional 3 bytes for future extension. Thus, the maximum supported length for upper layer data is 116 bytes. This message does not support segmentation. Thus, the maximum size of R2D NAS container is 116 bytes.

The D2R message relates to the command response message is D2R Upper Layer Data Transfer message, and the maximum size is 125 bytes. In this message, the MAC header(s) occupies 2 bytes. And RAN2 would like reserve additional 3 bytes for future extension. Thus, the maximum supported length for upper layer data is 120 bytes. This message support segmentation, i.e. the maximum supported length for upper layer data is 120 bytes in one segment, and there is no limitation on the number of segments. RAN2 understand the maximum size of whole command response message that includes all segments can be larger than 120 bytes. But the total size of the command response may be limited by the Received Data Size field which is used to schedule followed segments by the reader. The field has 7 bits and could indicate maximum 127 bytes. Thus, the maximum size of one command response that can be received by the reader is 127 bytes, and thus the maximum size of D2R NAS container is 127 bytes.

In conclusion, as to the maximum supported AIoT NAS container length mentioned in the LS:

- Maximum size of one R2D NAS container is 116 bytes, which does not support segmentation in Rel-19. If segmentation is introduced in future release, the maximum size of one R2D NAS container may be larger than 116 bytes.

- Maximum size of one D2R NAS container is 127 bytes, which is the maximum size of command response the reader can be received and also considering the segmentation case.

The above conclusion is from RAN2 signaling capability point of view. RAN2 also note that from SA1 requirement, the maximum message size is 1000 bits.

2	Actions

To CT1, RAN3

ACTION: 	RAN2 kindly asks CT1 and RAN3 to take above into account.

3	Dates of next RAN2 meetings

TSG-RAN2 Meeting #132		17 - 21 Nov 2025		Dallas, US

TSG-RAN2 Meeting #133		09 - 13 Feb 2026		Stor-Göteborg, SE