3GPP TSG-RAN WG2 Meeting #131bis					      R2-2506955

Prague, Czechia, 13 - 17 October, 2025

Title: 	Discussion on data transfer and AI/ML framework in 6G

Source: 	Huawei, HiSilicon

Agenda item:	10.3.3

Document for:	Discussion/Decision

- Introduction

In 6GR SID [1], there are objectives related to data transfer design and AI/ML framework:

(3) Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] (...) (8) AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g. performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g. MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In the RAN2#131bis agenda [2], data transfer and AI/ML framework have been listed.

10.3.3 Common User plane and Control plane Access stratum security aspects, in alignment with requirements from SA3. Transfer of various type of data (including AI/ML data, sensing, etc. ) and general AI/ML framework considerations. NOTEs: Detailed AI/ML use case specific proposals are not expected in this meeting. Specific technical details/procedures related to sensing are not expected until RAN1 starts 6G sensing work.

In this contribution, we provide our considerations on the data transfer and AI/ML framework in 6G.

- Consideration on Data Transfer in 6G

Motivation of supporting an E2E and unified Data Management framework

As mentioned in the related objective, data collection and transfer solution in 6G is expected to support various types of data. This is because many 3GPP 6G use cases related to network operational aspects, AI, ISAC and immersive communication are introduced in TR 22.870 [3] which require access to different kinds of data, e.g.:

- Several use cases describe the collection of sensing data. One example described in section 7.13 is related to a new service about collaborative robots. In this case the collaborative robots will utilize the Digital Twin information to fully control actions in the physical world without human interaction, and such information will be obtained via combination of sensing measurements and AI capabilities. There are also other cases for sensing data collection and environment object reconstruction described in section 5.9.4, 6.7, 7.5 etc. to facilitate utilization of sensing information for different services. According to the above use cases, constructing Digital Twin or environment information from the collected sensing data may not only serve new applications, e.g. collaborative robots, AI agents etc., but also can be used to support enhanced network performance, improved sensing deployment utilization and spectral efficiencies.

- The collection of AI-related data, such as training data, monitoring data, etc. is described in section 6, to support different kinds of services, such as AI agents, intelligent vehicles, or network optimization.

- The collection of QoE measurements is also discussed to improve user experience. For example, optimizing user experience for GenAI applications is addressed section 6.25, and improving user experience in multimedia-based applications is addressed in section 9.14. In these use cases, QoE measurements can be collected and used by the RAN to optimize network and improve user service experience. And based on QoE, the network can also adjust the QoS configurations dynamically when the payload type changes.

- Some use cases discuss the collection of various types of data. For instance, Section 6.7 discusses how 6G systems can effectively provide environmental information to AI agents to assist in high-precision reasoning. This information includes sensing information, real time QoS change information, etc. In addition, section 5.9.2 explores the collection and processing of data from various data types and sources to support heterogeneous use cases. These data include MDT data, sensing data, AI/ML data, etc. The 6G system is capable of gathering massive amounts of multi-source, diverse types of data to address demands across heterogeneous use cases such as AI/ML, sensing, XR service and digital twin network.

- …

According to the aforementioned use cases, various types of data need to be collected in 6G, including AI/ML data, sensing data, MDT data, QoE/QoS data, etc. The collected data can be used to support new services and applications of 6G, while simultaneously enhancing network performance. One service or application should also be able to utilize different types of data. In addition, participating nodes in data collection may include UE, RAN, CN, etc. This leads to the question on how to design the data collection mechanism for different types of data and which can support these variety of services.

In 5G, there are multiple mechanisms for collecting data for different purposes. However, these different types of data are collected and utilized independently using different mechanisms, and cannot be well integrated or combined. There might be overlapping, duplicated, mixed classification of data, which is not efficient to be utilized by the network, and also difficult to be provided as a service to the data consumers.  Looking ahead to future 6G systems, the collection of different types of data from various sources for different services and applications should not lead to unnecessary design of multiple options. Instead, there is an opportunity to establish an end-to-end, unified data management framework, to enable joint collection, management and distribution of different kinds of data within 3GPP system, i.e. between RAN and CN nodes, as shown in Figure 1.

Figure 1: Example of end-to-end and unified data management

Observation 1: Different types of data from different sources need to be collected in 6G.

Proposal 1: To support various services in 6G, there is a need to establish an E2E and unified data management framework within 3GPP system, including the study of the functions of data transfer, data utilization and data monitoring.

Functions of Data Management framework

To facilitate discussions of data management framework, the following functions can be studied in RAN2:

Data Transfer: A process of transferring data from a data provider to a data consumer. A single data item may have one or multiple consumers. In RAN2, the data transfer between UE and RAN can be studied.

Data Utilization: The collected data can be used to support new services and applications, such as AI/ML model training and inference, digital twin construction, etc.

Data Monitoring: Monitoring operations performed on data. 6G involves more types of data, requiring data monitoring to ensure data quality and guarantee performance of different tasks or services utilizing this data.

In the following sub-sections, we provide more consideration on these functions.

Data Transfer

As outlined in the 6GR SID [1], the data transfer design aims to support various types of data (e.g. AI/ML, Sensing, etc). In Table 1, we provide some examples of data types in 6G. It is noted that some data types are based on 5G-A definitions (e.g. AI for communication performance enhancement), while some data types may be totally new in 6G (e.g. digital twin). The data types discussed here are not limited to specific usages, i.e., data types may serve one or more usages, which also helps to enhance the efficiency of data transfer.

Table 1: Examples of Data Types

| Data Types | Motivation | Data Transfer Path | Examples of data | Data volume/latency |
| --- | --- | --- | --- | --- |
| (1) AI for communication performance enhancement (e.g. AI for PHY, AI for Mobility, AI for NG-RAN) | Utilize AI/ML to enhance BM, CSI reporting, Mobility, energy saving features | UE->NW, NW->UE, NW->NW | L1/L3 measurements, predictions, training dataset, etc. | For training: Potentially large data volume, relaxed latency [4] For inference: Rather small data volume, near real-time latency [4] |
| (2) Network for AI (e.g. digital twin) | Unlock new service opportunities, improve AI service performance or user experience | UE->NW, NW->UE, NW->NW | Environmental information/sensing information | Environmental information/sensing information: Depends on the use case |
| (3) Network Operation and Maintenance Data (e.g. SON, MDT) | Support RAN automation functionalities | UE->NW | RLF reports, SHR, and etc. MDT data (logged/immediate MDT) | For most of use cases: Small/Moderate data volume, relaxed latency [6] |
| (4) User experience measurement data (e.g. QoE) | Support network optimization by knowing user experience level | UE->NW | Container based QoE, RAN visible QoE metrics | Container based QoE: Large/Moderate data volume, relaxed latency [5] RAN visible QoE: Small data volume, stringent latency requirement |
| (5) Sensing data (e.g. sensing measurement result) | Support sensing based services such as digital twin, optimize network performance | UE->NW, NW->UE, NW->NW | The potential sensing measurement can be channel estimation results, delay, Doppler, angle, signal strength, etc. [3] | Small/Moderate data volume |

For data types shown in Table 1, data volume emerges as a key factor. From RAN2’s perspective, the data volume size will impact the data transfer mechanism, especially in the case the size becomes quite large. Therefore, we suggest to study how to support potentially large data transfer between UE and RAN. The data transmission latency should also be an evaluation criterion, because it will have an impact on the designed solution.

During 5G-A R19 AI for PHY discussions, RAN2 addressed UE-side data collection, resulting in an endorsed TP [7]. It is commonly recognized that maintaining controllability and visibility over collected data is essential, with preliminary analysis already conducted. We suggest to study how to ensure data visibility and controllability by the MNO during the data transfer.

5G-A utilizes RRC-based option for transferring SON/MDT data, QoE data, and other data types, incorporating QoS differentiation mechanisms. For instance:

- SRB2 has a lower priority than SRB1 and UEInformationResponse shall use SRB2 when logged MDT is to be transferred

- SRB4 is dedicated to QoE data transfer with a configurable priority (typically set as the lowest among SRBs/DRBs)

- A new SRB has been defined in Rel-19 for NW-side data collection for AI based beam management. This SRB also has a configurable priority which should be configured lower than SRB1 not to impact real-time network operation.

For 6G, we think it is important to study how to differentiate the data for data transfer and other data (such as control plane or user plane data).

In summary, the study on data transfer should take the following requirements into account: data volume, transmission latency, data visibility and controllability by the MNO, QoS differentiation between different types of collected data and between the collected data and other control plan or user plane data.

For data transfer between UE and RAN in RAN2, 5G-A defines mechanisms which as shown in Table 2. It can be seen there is room to improve the current mechanism.

Table 2: Limitations of 5G-A to support data transfer

| Data transfer mechanisms | The currently supported data types | Potential improvement directions to support diverse data types |
| --- | --- | --- |
| CP-based (Control Plane): RRC-based, NAS-based | RRC-based: 1) Network-side data collection for AI for communication performance enhancement 2) SON/MDT data collection 3) QoE data collection NAS-based: Currently NAS is used for transferring NAS messages between UE and AMF, but not for data collection. | Scalability: For large-volume data types with relaxed latency requirements, CP-based option lacks scalability. QoS flexibility: Current SRB design shows insufficient QoS adaptability, and it requires definition of new SRB types for emerging data types. |
| UP-based (User Plane): PDU session-based | Currently PDU session is mainly for transferring user application data, so it may be irrelevant to data transfer. | Path inefficiency: UE-to-CN path may lead to redundant data transfer when the RAN is data consumer. |

In RAN2, we think CP-based/UP-based options, as well as potentially new options (e.g. a dedicated data plane), can be studied in order to support data transfer between UE and RAN, taking the requirements mentioned above into account.

Proposal 2: For data transfer, RAN2 to study the following requirements, considering various data types: data volume, transmission latency requirement, data visibility and controllability, QoS differentiation between different data types.

- RAN2 to study CP-based, UP-based and potentially new options (e.g. a dedicated data plane) for data transfer according to these requirements.

Data Utilization

For Data Utilization, there are two aspects:

(1) Communication performance enhancement. The collected data can be used to improve network operation, e.g. CSI feedback enhancement, beam prediction. As mentioned in section 2.1, constructing Digital Twin or environment information from the collected sensing data can be used to support enhanced network performance, improved sensing deployment utilization and spectral efficiencies.

(2) New 6G service enablement. As mentioned in section 7.5 Use case on environment object reconstruction in TR 22.870 [3], the use case of environmental object reconstruction in 3GPP can facilitate Smart Transportation, Smart City, Smart Home and etc.

We think RAN2 could study the relations between the collected data and its usages, including the above two aspects, and different data provider(s)/consumer(s).

Proposal 3: For data utilization, RAN2 to study the relations between different data provider(s)/consumer(s) and how the collected data can be utilized for multiple purposes, including AI-based enhancements, sensing, SON/MDT.

Data Monitoring

Data Monitoring is mainly used to ensure data quality. For instance, for sensing data, there may be some monitoring parameters for sensing data quality, such as the accuracy of the sensing results which refers to how closely the measured or estimated parameters match their true values. Metrics like Root Mean Square Error (RMSE) can be tracked.

The benefits of data quality monitoring include:

- Maintains system performance: early detection of quality issues allows for corrective actions (like calibration) before performance significantly degrades

- Optimizes resource allocation: knowing the quality of data from different sources can help the network allocate the resources more rationally

For mechanisms for monitoring, a feedback loop may be considered, and the assessment of data utilization performance (e.g. how well the sensing data performs in an environment reconstruction or digital twin) can be considered. If the data quality degrades, an adjustment of the monitoring and data collection process may be triggered.

Proposal 4: For data monitoring, RAN2 to study how to ensure quality of the collected data.

- Consideration on AI/ML framework in 6G

AI/ML LCM functions in 6G

As noted in the 6G SID, 6G system is expected to provide extensible AI/ML enablers to support various AI/ML use cases. At the same time, it is noted that “6GR and RAN design shall ensure that the 6G System can also operate without AI/ML.” These requirements require a study on a flexible AI/ML framework, including the following LCM functions:

Functionality management

As a principle, in 6G it should be the network which is in control of whether a certain functionality is enabled or not, e.g. depending on the current needs, preferences, network conditions etc. UE fallback or switching between AI/ML-based and non-AI/ML based functions does not have to be precluded, but its necessity should be carefully studied and its usage should always be under network control.

Model management

Offline model training is usually assumed for a given model location setting in 5G-A discussion. Furthermore, trained models were assumed to be applicable only in certain cells sharing common radio characteristics which was denoted with a so-called associated ID. However, considering the number of different network configurations and deployments one may come across in the real life, it is inefficient to train a model dedicated to, e.g. each network cell configuration separately. Therefore, techniques like online model training/adjustment will have to be investigated closer, as discussed in more detail in section 3.2.

Model performance monitoring

In 6G, similar principles can be followed for performance monitoring as those developed in 5G-A. In particular, as a baseline a common performance monitoring framework should be utilized while allowing a certain degree of flexibility to cater for use case specific requirements and metrics. In addition, with more introduced use cases, AI/ML functionalities may be coupled or dependent on each other (e.g. output of one AI/ML functionality may be the input for another AI/ML functionality). This calls for consideration of more efficient and low overhead/complexity/power consuming monitoring mechanisms.

Observation 2: To ensure 6GR is able to exploit the most powerful AI/ML methods and support the latest AI/ML services, the study of 6G AI/ML LCM framework should consider more flexible functionality/model management, efficient applicability reporting procedures and performance monitoring.

Consideration of Network for AI within 6G AI/ML LCM framework

Considering the rapid progress of AI/ML technologies, forward compatibility to exploit the most powerful AI/ML methods and support the latest AI/ML services is important factor to consider for framework design. According to [8], many AI/ML use cases have been proposed in RAN1 for 6G. Meanwhile, SA1 also achieved agreements on many AI use cases and their service requirements [3]. It is then expected that 6GR should support use cases related to both AI/ML-enabled network and Network for AI (e.g. digital twin etc.).

Observation 3: Based on RAN1 and SA1 discussions, it is expected that 6GR should support use cases related to both AI/ML-enabled network and Network for AI (e.g. digital twin).

As a typical AI/ML service, digital twin based on sensing has been discussed in SA1 [3]. To facilitate digital twin construction, sensing data needs to be collected and AI/ML methods can be utilized to facilitate this process. For example, to minimize overhead and energy consumption, each sensing node may only collect sparse samples of the environment, then distributed training and inference can be used to improve the precision and completeness of the environment information. The digital twin can provide environment information which can be used for both AI services and optimizing the air interface communication itself.

Figure 2: Distributed AI/ML enabled digital twin construction

As shown in Figure 2, each single UE can only generate sparse sensing data which reflects the local environmental information around the UE. Based on the inference with the sparse environmental information as input, the AI/ML model can output more precise and complete environmental information. The UE-part model input is in forms of sparse sensing data, e.g., multi-path information, angle information, point cloud information, etc.; UE-part model is adopted to convert the raw data to latent space and outputs latent space information for compressing and privacy protection; the NW-part model takes the latent space information reported from multiple UEs as input for aggregation and builds the precise and complete global environment information, e.g. point cloud information of global environment, as inference output. The NW-part model works with multiple-on-one manner to various UE-part models, which is denoted as a distributed model solution with NW and UE collaboration targeting at joint AI/ML operation.

With the above-mentioned distributed training and inference introduced, multiple UEs will be involved. Hence, management of multiple UEs should be studied. For example, the NW should select the UEs which can collect the needed data to report. The UE selection should also take UE state and capability as well as channel conditions between the UE and NW into consideration.

As the environment changes, for example caused by UE moving, the data distribution may be shifted, which leads to performance degradation. Online training should be considered to achieve more flexible model update. As defined in [6], online training is an AI/ML training process where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. With the consideration of complexity, online training is usually conducted based on a previous (potentially outdated) version of the model rather than re-training it from scratch, i.e., low overhead online finetuning is more desirable.

- One choice of online training is to transmit the newly collected data back to the model training server, where the model is updated and sent back to the device (UE or BS) for further usage. This method is suitable for the case when the computing capability of the device is limited and some model update delay is acceptable.

- For the case when the timeliness of the model is important, the online training procedure can be conducted on a device. Processing capability and power consumption are required for the device, however, advanced training methods such as Low-Rank Adaptation (LoRA) can help with alleviating this issue.

The online training procedure can be triggered by events such as performance degradation, and it can be conducted along with the changing of data distribution. Continuous learning is important in this case to allow the model to stay up to date.

Distributed learning and online training are described above based on digital twin example which, once constructed, can be used for both Network for AI services as well as AI enhanced communications. However, it is important to note that these techniques are also applicable to other AI enhanced communications use cases:

- Pre-trained AI/ML models with fixed parameters (e.g. weights and biases), after being deployed in the system, can only work with guaranteed performance in the scenario within the range of its generalization capability. Model switch or even fallback may be needed when it comes to a new scenario. Model switch consumes high resources for preparing and storing a large number of models for different scenarios, which is inconvenient in some scenarios. Frequent fallback to non-AI/ML operation due to model mismatch/underfitting does not fully exploit the benefits of AI/ML and leads to performance loss. This issue can be addressed, e.g. with online training.

- To train a robust model, data from multiple sources needs to be gathered and processed at the training entity. Furthermore, this data should have a proper distribution, e.g. the training entity may require data only from certain parts of the cell which are underrepresented in the collected data. Therefore, it is also important in these situations to allow the network to control the UE selection and type/range of data that needs to be collected.

Observation 4: Enhanced model training and data collection/reporting techniques bring benefits for both AI for Network and Network for AI use cases.

Based on the discussion above, we can draw the following conclusions with respect to the requirements of AI/ML framework design in 6G:

Proposal 5: RAN2 should support Network for AI (e.g. digital twin etc.) and corresponding AI/ML LCM framework in 6G.

Proposal 6: RAN2 should study how to support distributed training and inference in 6G AI/ML LCM, e.g., data collection and reporting, UE selection.

Proposal 7: RAN2 should study how to support online training and finetuning in 6G AI/ML LCM, e.g., data collection and reporting for real-time training/finetuning.

- Conclusion

In this contribution, we provide our initial views on data transfer and AI/ML framework in 6G. We make the following observations and proposals:

Data Transfer in 6G

Observation 1: Different types of data from different sources need to be collected in 6G.

Proposal 1: To support various services in 6G, there is a need to establish an E2E and unified data management framework within 3GPP system, including the study of the functions of data transfer, data utilization and data monitoring.

Proposal 2: For data transfer, RAN2 to study the following requirements, considering various data types: data volume, transmission latency requirement, data visibility and controllability, QoS differentiation between different data types.

- RAN2 to study CP-based, UP-based and potentially new options (e.g. a dedicated data plane) for data transfer according to these requirements.

Proposal 3: For data utilization, RAN2 to study the relations between different data provider(s)/consumer(s) and how the collected data can be utilized for multiple purposes, including AI-based enhancements, sensing, SON/MDT.

Proposal 4: For data monitoring, RAN2 to study how to ensure data quality of the collected data.

AI/ML framework in 6G

Observation 2: To ensure 6GR is able to exploit the most powerful AI/ML methods and support the latest AI/ML services, the study of 6G AI/ML LCM framework should consider more flexible functionality/model management, efficient applicability reporting procedures and performance monitoring.

Observation 3: Based on RAN1 and SA1 discussions, it is expected that 6GR should support use cases related to both AI/ML-enabled network and Network for AI (e.g. digital twin).

Observation 4: Enhanced model training and data collection/reporting techniques bring benefits for both AI for Network and Network for AI use cases.

Proposal 5: RAN2 should support Network for AI (e.g. digital twin etc.) and corresponding AI/ML LCM framework in 6G.

Proposal 6: RAN2 should study how to support distributed training and inference in 6G AI/ML LCM, e.g., data collection and reporting, UE selection.

Proposal 7: RAN2 should study how to support online training and finetuning in 6G AI/ML LCM, e.g., data collection and reporting for real-time training/finetuning.

- References

RP-252912, Revised SID: Study on 6G Radio

Draft RAN2#131bis agenda

TR 22.870, Study on 6G Use Cases and Service Requirements

R2-2311720, Reply LS on Data Collection Requirements and Assumptions, Source: RAN1, To: RAN2

R2-2109386, Reply LS on QoE configuration and reporting related issues, Source: SA4, To: RAN2, Cc: RAN3, SA5

TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

R2-2506457, RAN2 inputs to TR 38.843 - first part, Ericsson

R1-2506456, Moderator summary #2 on AI/ML for 6GR, Samsung (Moderator)