3GPP TSG RAN WG 2 # 131-bis											R2-2506991

Prague, CZ, October 13th-17th, 2025

Agenda Item:	9.7.2

Source:	CSCN

Title:	Discussion on IMS voice over GEO

Document for:	Discussion

- Introduction

RAN2 has received 3 liaisons on IMS voice over GEO from SA4 (R2-2505068/S4-251584) [1] and SA2 (R2-2506746 /S2-2507578 [2], R2-2506747/S2-2507636 [3]).

In LS R2-2505068/S4-251584, SA4 asked RAN2 whether there is a preference among the UP/CP and IP/Non-IP solution, and whether the 1 byte of MAC header is feasible.

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In LS R2-25xxxx/S2-2507578, SA2 answered they has not yet concluded on the solution and believes RAN2 should answer the size of protocol header.

[SA2 answer]: SA2 has documented a number of alternative solutions in TR 23.700-19 for “Key Issue #1: Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC” that cover various options related to a) using User Plane or Control Plane CIoT EPS Optimisation and b) IP or non-IP PDN type. SA2 has not yet concluded on the solutions for Key Issue #1. The final protocol overhead depends on the selected solutions. The conclusion on all these issues and the estimated overhead is expected to be provided later. The completion date of the FS_5GSAT_Ph4_ARC SID is December 2025 (SA#110). - specifically, whether a packet overhead of 1 byte of MAC header is realistic. [SA2 answer]: SA2 believes RAN2 should answer this question and also provide feedback on the PDCP, RLC and MAC overhead expected.

In the last SA2 meeting, SA2 also discussed about CP and UP solution but they didn’t have consensus on the issue. So, in LS R2-25xxxx/S2-2507636, SA2 raised several questions for RAN2 in order to further estimate the solution for IMS voice over GEO.

Under the study item FS_5GSAT_Ph4_ARC SA2 is studying the support for IMS voice over NB-IoT NTN connected to EPC. In the context of this study item SA2 is investigating solutions in TR 23.700-019 that could potentially use IP or non-IP type (i.e. with and without IP/UDP header) for the (voice) traffic and would like to ask some questions in order to help SA2 to evaluate the alternative solutions. When IP PDN connection is used, in order to achieve the target KPIs for support of IMS voice over NB-IoT NTN connected to EPC, it is expected that the support for Robust Header Compression (RoHC) will likely be required. RoHC is supported between UE and eNB when "user plane" DRBs are used to transport the data or between the UE and MME when "control plane CIoT" and SRBs are used to transport the data. In order for SA2 to be able to evaluate the viability of using RoHC for support of IMS voice over NB-IoT NTN connected to EPC, SA2 has the following questions: During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state). Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC? Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically? Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20? Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media? Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

In this contribution, we will discuss on the questions in LS and provide our views.

- Discussion

  - CP and UP down-selection

All aforementioned questions in the LS are related to CP and UP downselection in a way. SA2 has conducted analysis on this but has no consensus for the downselection so far. Since the first item in the objective section of WID [4] is about the selection of the CP/UP solution, this is also the most important aspect of this topic. Considering the down-select needs to be figure out at the RAN#110 meeting and if there is still no conclusion from SA2 at the October meeting, RAN2 can provide some constructive preference at the WG level.

- Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection

Before RAN#110, RAN2 can have constructive preference on CU and UP solution down selection from WG level.

  - The size of MAC header

According to the TS 36.321 spec [5], the two types of minimal MAC subheader are defined as follows:

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader with 7-bits and 15-bits L field Figure 6.1.2-2: R/F2/E/LCID/ MAC subheader - L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field and F2 field; - R: Except when this field is used for short DCQR, reserved bit, set to "0". For short DCQR, the mapping of R field to short DCQR value is described in table 6.2.1-5.

Currently the L field can be used to indicates the length of MAC SDU corresponding to the transmission of voice data. If we also consider the dynamic change possibility for the data value during the GEO IMS voice call, the L field is also necessary. If the dynamic change of data is not considered and only the MAC subheader indicates whether the MAC transmits data as talk or silence, then maybe 1 byte MAC header is feasible. However, this R field can not be used to indicate short DCQR as legacy. Meanwhile, 1 byte MAC header will also lead to multiplexing MAC SDU(s) and MAC CE(s) to be invalid either. In summary, the 1 byte MAC header limits many protocol functions, and the MAC subheader should be at least 2 bytes.

1 byte MAC header feasibility requires a lot of assumptions and limits the functionality of existing protocols.

RAN2 responds to SA4 and SA2 that a packet overhead of 1 byte of MAC header for voice packets is not realistic.

  - Fall back of RoHC (Q1)

In our understanding, the fall back of RoHC is strong relative to the implementation, and the mechanism of RoHC is defined in IETF. So how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC is out of RAN2 scope.

It is out of RAN2 Scope to decide how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back.

RAN2 responds to SA2 RoHC state fall back issue is out of RAN2 scope.

  - Dynamically changed voice data and SPS issue (Q3)

In the current specification, only UL SPS for BSR is supported for NB-IoT [5].

Except for NB-IoT, multiple UL Semi-Persistent Scheduling configurations are supported per Serving Cell. For NB-IoT, UL Semi-Persistent Scheduling configuration is only supported for BSR per Serving Cell. On one Serving Cell, multiple UL configurations can be active simultaneously only for the same TTI length. Multiple UL/DL configurations can also be active simultaneously on different Serving Cells.

Typically, there are two different size data in GEO IMS voice, one for talk and another for silence. RAN2 should consider how to avoid resource waste and achieve accurate resource allocation when scheduling these two kinds of data sizes. According to the current specification, only UL SPS for BSR is supported. So, multiple separate SPS configuration can be further enhanced for scheduling different TB sizes for voice data.

RAN2 responds to SA2 that RAN2 confirm to support scheduling methods for IMS voice over NB-IoT NTN with voice packets of different sizes.

  - More than 2 DRBs and additional new SRB (Q6, 9, 10)

The 2-DRB limitation for UEs is from the commercial consideration of buffer cost and hardware complexity. From technical perspective, it is feasible to support more than 2 DRBs (e.g., 3 DRBs) for UE accessing NB-IoT in Rel-20. For example, the UP solution will utilize two DRBs, one for control signalling and the other for voice data transmitting. Considering the existing default DRB, additional DRB may needed for legacy IoT service if UP solution introduced. Then the new DRB will transmit data, while the existing SRB still transmit signalling for other services as legacy. If the CP solution is adopted, at least a new SRB with RLC UM mode needs to be added for voice-data transmission. Existing SRB cannot be transmitted using the RLC-UM mode due to the security concern. So from technical perspective, more than 2 DRBs and additional new SRB are reasonable, and the RB introduction is related to the CP and UP downselection.

From technical perspective, more than 2 DRBs and additional new SRB can be support in RAN2, and the RB introduction is related to the CP and UP downselection.

- Conclusion

Observation 1: 1 byte MAC header feasibility requires a lot of assumptions and limits the functionality of existing protocols.

Observation 2: It is out of RAN2 Scope to decide how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back.

Proposal 1: Before RAN#110, RAN2 can have constructive preference on CU and UP solution down selection from WG level.

Proposal 2: RAN2 responds to SA4 and SA2 that a packet overhead of 1 byte of MAC header for voice packets is not realistic.

Proposal 3: RAN2 responds to SA2 RoHC state fall back issue is out of RAN2 scope.

Proposal 4: RAN2 responds to SA2 that RAN2 confirm to support scheduling methods for IMS voice over NB-IoT NTN with voice packets of different sizes.

Proposal 5: From technical perspective, more than 2 DRBs and additional new SRB can be support in RAN2, and the RB introduction is related to the CP and UP downselection.

- Reference

- R2-2505068/S4-251584, LS on the RAN simulation assumptions for ULBC, SA4

- R2-2506746/S2-2507578, Reply LS on the RAN simulation assumptions for ULBC, SA2

- R2-2506747/S2-2507636, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, SA2,

- RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4

- TS 36.321, V18.4.0 (2025-03), Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification(Release 18)