3GPP TSG RAN WG 2 #131-bis										R2-2506992

Prague, Czech Republic, October 13th-17th, 2025

Agenda Item:	10.2

Source:	CSCN

Title:	Discussion on the general aspects of 6G NTN

Document for:	Discussion

- Introduction

In RAN#109 meeting, RP-252882 [1] was approved on the section of requirements for architecture and migration, as shown below, and mentioned that “The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.” There is also detailed object captured in RP-252912[2] that “Aim at a harmonized 6G Radio design for TN and NTN, including their integration.” Meanwhile, RP-2873[3] endorsed 6G should study further whether and how to support NTN-specific device types.

## Requirements for architecture and migration Editor note: 6G RAN architecture, 5G-6G migration The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements: ... - The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network. ...

- 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] … J) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. … Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

Proposal 3: To investigate further: … - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. … Note: the terminology “device type” is subject to further discussion and possible refinement.

We think that 6G needs to natively support NTN from Day1, and here we present our views on the general aspects including deployment scenarios and design principle.

- Discussion

6G NTN deployment scenarios are defined to allow for the provision of services for those areas including vertical space (e.g., up to several kilometers altitude) where terrestrial network service is not available (on a permanent or temporary basis), and also for those services that can be more efficiently supported by Non-Terrestrial Networks.

Non-Terrestrial Networks can overcome coverage limitations of terrestrial networks especially in low-density populated areas and maritime areas.

As such, NTN will mainly contribute to:

- Achieve ubiquitous connectivity and availability of services in remote and sparsely populated areas.

- Improve overall 6G network resiliency, especially in situations where disasters disrupt terrestrial networks or damage undersea communication cables by supporting in priority critical communications and emergency communications (messaging, voice calls).

6G NTN should support both regenerative and transparent payload as in 5G. and also support any orbits range from VLEO (300 to 600 km altitude) to GSO (~35786 km altitude), including LEO (600 to 2000 km altitude) and MEO (2000 to ~35000 km altitude). NTN may be based on a combination of orbit types (e.g. GSO and LEO), multi-orbit collaboration between LEO, MEO and GSO satellites can be considered in 6G. With the goal of seamless global coverage with ubiquitous connectivity, 6G NTN should also consider the scaling evolution from sparse to dense satellite constellations with resilience and service flexibility.

NTN deployment

- Cell deployment

As studied and described in R16 TR38.821[4], “Similar to TN, one or several SSB index can be used per PCI to separate SSB transmission on different beams.” We believe that 6G NTN can reuse the designs studied in 5G TN and NTN, whether it is SSB beam or other logical beams newly defined in 6G. As a result, 6G cell deployment can include one-beam cell and multi-beam cell, as shown below.

- One-beam cell: The cell consists of only one logical beam as in Figure 2.

Figure 2: One-beam cell

- Multi-beam cell: The cell consists of multiple logical beams with beam sweeping within a cell as in Figure 3. Logical beams may be SSB beams inherited from 5G or newly defined synchronization signals in 6G. To achieve refined management, supporting more than 8 logical beams in FR1 can be considered.

Figure 3: Multi-beam cell

In multi-beam cell, it is worth noted that the shape of the cell may have irregular shape (i.e. non-circular) due to the inclusion of multiple circular beams within it. As illustrated in the Figure 4 below, taking multiple beams as an example, the cell is difficult to deploy as a perfect circle and is more likely to be shaped as an approximate polygon. Furthermore, in practical NTN deployment, the NTN cell coverage area should take into account for national boundaries constraints. Therefore, 6G NTN should study on the impact with the irregular shape of NTN cells.

Figure 4: Irregularly cell deployment with multiple beams

In the scenario of multi-beam cell, the cell may have irregular shape (i.e. non-circular) due to the inclusion of multiple circular beams within it.

RAN2 to study the impacts on the irregular shape of NTN cells, for example, mobility.

- Cell type

NTN can be deployed to provide coverage with earth-moving cell, quasi-earth-fixed cell and earth-fixed cell which are supported respectively by the following three types of service link [5]:

- Earth-fixed: provisioned by beam(s) continuously covering the same geographical areas all the time (e.g., the case of GSO satellites); - Quasi-Earth-fixed: provisioned by beam(s) covering one geographic area for a limited period and a different geographic area during another period (e.g., the case of NGSO satellites generating steerable beams); - Earth-moving: provisioned by beam(s) whose coverage area slides over the Earth surface (e.g., the case of NGSO satellites generating fixed or non-steerable beams).

The Quasi-Earth-Fixed deployment can be categorized into two types in the scenario of multi-beam cell, the current serving cell is denoted by a yellow circle, with its beams denoted by blue circles, as shown in Figure 5:

- Case 1: The areas of cell coverage in two consecutive periods are completely different.

- Case 2: The areas of cell coverage in two consecutive periods partly overlap.

Figure 5: Quasi-Earth-Fixed case 1 and case 2

For Quasi-Earth-Fixed case 1, service for UE1 and UE2 starts at time T1 and stops simultaneously at time t-Service, after which the satellite starts to serve the next geographic area (i.e. another cell). The geographical area of both the serving cell and its beams remains unchanged during time T1 to time t-Service. The continuous service duration for each UE is given by t-Service – T₁.

For Quasi-Earth-Fixed case 2, with the satellite moves, the cell coverage area hops over the Earth surface with the earth-fixed beams. Different from all beams within Quasi-Earth-Fixed case 1, one or several beams covering for one geographic area switch to another geographic area after a limited period. The service for UEs in corresponding beams starts and stops simultaneously while the UEs in the other beams of this cell have the different start and stop time. From our perspective, case2 is beneficial for the link budget with ubiquitous coverage.

RAN2 to study a new case 2 with cell coverage in two consecutive periods partly overlap in multi-beam deployment.

  - Design principle for NTN

Compared to TN networks, NTN networks are characterized by limited air interface resources, restricted power consumption of base stations and terminals, and frequent handovers for terminals in regenerative mode. Therefore, it is necessary to reasonably optimize and reduce the signaling size and procedure complexity on the NTN air interface to efficiently utilize air interface resources, improve user experience, and reduce system power consumption.

In multi-beam cell scenarios, due to the wide-area coverage of NTN networks, the network conditions, geographical environment, and user distribution vary across different sub-areas within a cell. Therefore, applying uniform configurations across the entire cell results in coarse granularity, single and ambiguous configurations. This not only wastes network resources but also causes UEs to execute configurations unrelated to their actual location, leading to unnecessary power consumption. It is recommended to adopt a fine-grained signaling design in multi-beam cell scenarios.

RAN2 to study a fine-grained signaling design for 6G NTN.

  - Multi-orbit collaboration

Multi-orbit collaboration means that satellites in different orbits are capable of offering services to UEs. However, a prerequisite for this is NGSO compliance with an ITU regulation as defined in TR38.821 [4].

In the ITU Radio Regulations (ITU, 2016) the interference to any GEO network caused by a NGSO system is strictly constrained. One restriction to the system is that a NGSO system shall not cause unacceptable interference into any GEO network in the fixed satellite service for uplink and downlink in the same frequency (in-line interference), shown in Figure C-1 (i.e. Figure 6). For that reason, (ITU, 2016) limits the maximum equivalent power flux densities (EPFD) of the NGSO system to values which are not sufficient to enable satellite communication. ... So the downlink signal from a NGSO satellite is interfering a GEO receive station on earth. The satellite beam pointing towards the GEO ground station has either to be steered to another direction or has to be switched off completely.

Figure 6: In-line interference caused by NGSO systems (MEO or LEO)

To fulfill the aforementioned interference avoidance principles for GEO networks, the coverage of a single NGSO satellite may become incomplete. That is, partial service blind zones may exist within a serving cell. However, the geographical position of these blind zones on the ground changes as the NGSO satellite moves, and the UEs are unaware of when they will be affected by such blind zones. In order to decrease the impact for UE, it is recommended to study the impact of interference avoidance for GEO.

RAN2 to study the impact of in-line interference between GEO and NGSO, for example, the blind zones.

  - Energy efficiency

In the NTN network, satellites have very limited energy sources, including solar panels and batteries. Once launched, the satellite's energy system is fixed and cannot be recharged in the shadow zone or have its batteries replaced. Given the energy constraints of satellites, improving energy efficiency becomes an important consideration.

Specifically, 6G NTN can consider extending the SSB periodicity (e.g., 320ms) to improve satellite energy efficiency. Such extension may have impact on average latency during initial user access as well as delays in cell handover and reselection, which may decrease user experience. Consequently, it is necessary to evaluate these trade-offs in the system design.

6G NTN may consider implementing on-demand signaling to improve satellite energy efficiency. On-demand SSB has been introduced in 5G and is also applicable to 6G systems. Both base station and UE should enter an inactive state when there is no data transmission, activating signaling only when initiating a service or responding to the requests. A joint energy-saving strategy between xNB and UEs should be prioritized in the design.

RAN2 to study energy-efficient design for 6G NTN (e.g., SSB periodicity extension, on-demand SSB).

- Conclusion

Observation 1:	In the scenario of multi-beam cell, the cell may have irregular shape (i.e. non-circular) due to the inclusion of multiple circular beams within it.

Proposal 1:	RAN2 to study the impacts on the irregular shape of NTN cells, for example, mobility.

Proposal 2:	RAN2 to study a new case 2 with cell coverage in two consecutive periods partly overlap in multi-beam deployment.

Proposal 3:	RAN2 to study a fine-grained signaling design for 6G NTN.

Proposal 4:	RAN2 to study the impact of in-line interference between GEO and NGSO, for example, the blind zones.

Proposal 5:	RAN2 to study energy-efficient design for 6G NTN (e.g., SSB periodicity extension, on-demand SSB).

- Reference

- RP-252882, TR 38.914 v0.2.0 for Study on 6G Scenarios and requirements, CMCC, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- RP-252873, Moderator's summary on 6G diverse device types (incl. minimum UE bandwidth), Apple, 3GPP TSG#109, Beijing, China, September 15-18, 2025

- TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), v16.2.0

- TS 38.300, NR and NG-RAN Overall Description, v18.6.0