3GPP TSG-RAN WG2 Meeting #131bis		R2-2507014

Prague, Czech Republic, 13th – 17th Oct., 2025

Source:	vivo

Title:	Discussion on MAC open issues for LTM

Agenda Item:	8.6.3

Document for:	Discussion and Decision

- Introduction

In this paper, we will discuss the remaining MAC open issues LTM.

- Discussion

  - [MAC-V01] RACH based LTM with MIMO 2TA

In RAN2#129bis meeting, the following agreements were made for the co-existence of inter-CU LTM with Rel-18 MIMO 2TA,

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

However, whether RACH-based inter-CU LTM can coexist with MIMO 2TA has not been discussed.

As for RACH-based LTM and MIMO two TA, the same TAG mismatch issue exists as we discussed in Rel-18 [1]. Specifically, if a TCI state ID included in the LTM Cell Switch Command MAC CE is associated with TRP 1, while the TAG of TA in the RAR for RACH-based LTM is associated with TRP 2, after completing the LTM cell switch, UE shall use the TCI state in the LTM Cell Switch Command MAC CE for data transmission without having any available TA value associated to the TCI state.

Observation 1: [MAC-V01] Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and MIMO two TA is supported.

In [1], four options were proposed to resolve the problem:

- Option 1: After the completion of LTM, the target DU initiates a PDCCH order triggered RACH procedure via the indicated TCI state in LTM Cell Switch Command. The RACH procedure is performed to obtain the TA value associated with the indicated TCI state.

- Option 2: The target DU sends a new TCI state in RACH Msg4. The new TCI state is associated with the TAG indicated in the RACH RAR.

- Option 3: If the mismatch issue occurs, after the completion of RACH procedure, UE follows the TCI state associated to the RACH procedure, i.e. the DL TCI state is associated to the SSB for RACH procedure and the UL TCI state is associated with the TCI state for Msg3 transmission. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the TAG ID same as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM.

Due to time constraints, it was agreed that network implementation (e.g., option 1) would handle this issue in Rel-18.

RAN2#127 discussion for RACH based LTM and 2 TAs

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Actually, initiating a PDCCH order-triggered RACH procedure via the indicated TCI state after LTM cell switch completion would result in additional latency. To ensure the UE can perform data transmission in the target cell immediately, other options need to be considered.

Proposal 1: [MAC-V01] RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 2: [MAC-V01] if Option 3 in Proposal 1 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

  - [MAC-V02] CFRA resource in LTM Cell Switch Command MAC CE

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, RAN3 reached the following agreement in the last meeting:

RAN3#128 Agreement

In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation.

However, whether the CFRA resource provided by the candidate gNB can be shared by multiple UEs or is reserved for a delicated UE has not yet been discussed.

In our view, there are two potential approaches:

Approach 1: Allocated by Target DU

- During the handover preparation phase, S-DU obtains dedicated CFRA resources per UE from T-DU;

- S-DU then allocates specific CFRA resource to the UE to perform LTM cell Switch.

Approach 2: Allocated by Source DU

- During the handover preparation phase, S-DU obtains a shared CFRA resource pool from T-DU for multiple UEs within a single DU;

- S-DU then allocates specific CFRA resource from the pool to the UE to perform LTM Cell Switch.

For approach 1 (CFRA resource allocated by Target DU), specific CFRA resources need to be reserved for each UE. This could lead to significant resource wastage when subsequent LTM is supported since CFRA resources would have to remain reserved over extended periods of time for LTM candidate cells. Additionally, if CFRA resources are allocated by Target DU, there would be no practical difference between including them in an LTM Cell Switch Command MAC CE versus including them in preconfigured candidate RRC configurations. This goes against our original motivation behind including CFRA resources within an LTM Cell Switch Command MAC CE.

For approach 2 (CFRA resource allocated by source DU), CFRA resources are shared among UEs within a single DU, which is similar to the early-RACH resources sharing mechanism supported in Rel-18 LTM. The S-DU is responsible for allocating specific CFRA resources for each UE to perform RACH-based LTM cell switch while simultaneously avoiding the CFRA resource conflicts. As a result, the resource wastage can be largely reduced. Therefore, we prefer using Approach 2.

Proposal 3: [MAC-V02] For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Since the CFRA resources for RACH-based LTM are assigned to UE by source DU, the source DU needs to inform the target DU about the mapping relationship between the CFRA resource information (e.g., preamble index and RA-RNTI) and the UE to perform LTM. This ensures that the target cell knows which UE is performing RACH-based LTM via the CFRA resource.

Observation 2: [MAC-V02] For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 4: [MAC-V02] Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

  - [MAC-V03] Impact of removing or modifying the L1 measurement reporting configuration

For legacy L3 measurement reporting, when a measId, or a MO/reportConfig associated with that measId is removed from or modified in the current UE configuration, the UE will remove the measurement reporting entry for this measId from the VarMeasReportList, stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId. This behavior is described in TS 38.331, Section 5.5.2, as follows:

5.5.2 Measurement configuration 5.5.2.2 Measurement identity removal The UE shall: 1> for each measId included in the received measIdToRemoveList that is part of the current UE configuration in VarMeasConfig: 2> remove the entry with the matching measId from the measIdList within the VarMeasConfig; 2> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 2> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId. […] 5.5.2.3 Measurement identity addition/modification […] The UE shall: - for each measId included in the received measIdToAddModList: […] 2> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 2> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId; NOTE 1: If the measId associated with reportConfig for conditional reconfiguration is modified, the conditions are considered to be not fulfilled as specified in 5.3.5.13.4. […] 5.5.2.4 Measurement object removal The UE shall: 1> for each measObjectId included in the received measObjectToRemoveList that is part of measObjectList in VarMeasConfig: 2> remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig; 2> remove all measId associated with this measObjectId from the measIdList within the VarMeasConfig, if any; 2> if a measId is removed from the measIdList: 3> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 3> stop the periodical reporting timer or timer T321 or timer T322, whichever is running, and reset the associated information (e.g. timeToTrigger) for this measId. NOTE: The UE does not consider the message as erroneous if the measObjectToRemoveList includes any measObjectId value that is not part of the current UE configuration. 5.5.2.5 Measurement object addition/modification The UE shall: 1> for each measObjectId included in the received measObjectToAddModList: 2> if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry: […] 3> for each measId associated with this measObjectId in the measIdList within the VarMeasConfig, if any: 4> if the UE does not support Uplink PDCP delay measurements upon MO update; or 4> if the UE supports Uplink PDCP delay measurements upon MO update and if ul-DelayValueConfig or ul-ExcessDelayConfig is not configured for the associated reportConfig: 5> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 5> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId; 5.5.2.6 Reporting configuration removal The UE shall: 1> for each reportConfigId included in the received reportConfigToRemoveList that is part of the current UE configuration in VarMeasConfig: 2> remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig; 2> remove all measId associated with the reportConfigId from the measIdList within the VarMeasConfig, if any; 2> if a measId is removed from the measIdList: 3> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 3> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId. NOTE: The UE does not consider the message as erroneous if the reportConfigToRemoveList includes any reportConfigId value that is not part of the current UE configuration. […] 5.5.2.7 Reporting configuration addition/modification The UE shall: 1> for each reportConfigId included in the received reportConfigToAddModList: 2> if an entry with the matching reportConfigId exists in the reportConfigList within the VarMeasConfig, for this entry: 3> reconfigure the entry with the value received for this reportConfig; 3> for each measId associated with this reportConfigId included in the measIdList within the VarMeasConfig, if any: 4> remove the measurement reporting entry for this measId from the VarMeasReportList, if included; 4> stop the periodical reporting timer or timer T321 or timer T322, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId; 2> else: 3> add a new entry for the received reportConfig to the reportConfigList within the VarMeasConfig.

Observation 3: [MAC-V03] According to the current RRC specification, if a measurement identity or a reporting configuration/measurement object associated with that measurement identity is removed or modified, the UE will remove the measurement reporting entry for this measId from the VarMeasReportList, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this measId.

For the L1 event-triggered measurement reporting or L1 event based C-LTM, the same issue as in L3 measurement reporting exists. Specifically, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity should remove the measurement reporting entry for this ltm-CSI-ReportConfigId from the MR_LIST, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId. This prevents unnecessary event evaluation and reporting, as well as incorrect C‑LTM execution.

Proposal 5: [MAC-V03] Similar as L3 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall remove the measurement reporting entry for this ltm-CSI-ReportConfigId from the MR_LIST, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

If proposal 1 is agreed, the following TP based on the latest MAC running CR [2] can be adopted.

Proposal 6: If proposal 1 is agreed, adop the TP in Annex for MAC specification.

  - [MAC-V04/V05] TA update of CLTM TAT

- TA update of CLTM TAT when receiving an LTM cell switch command

In RAN2#129bis meeting [3], it was agreed that the C-LTM TAT timer for the target cell will not stop upon C-LTM cell switch execution (if running).

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

In our view, the C-LTM TAT for the target cell should also not be stopped upon LTM cell switch execution triggered by the LTM Cell Switch Command MAC CE while it is running. In this case, it needs to be discussed whether the TA maintained by the UE for the target cell should be updated according to the TA indicated in the LTM Cell Switch Command MAC CE.

One scenario involves the network triggering the UE to perform early-RACH to the target cell. The network receives the TA value and directly triggers the LTM cell switch, including the TA value in the LTM Cell switch Command MAC CE, without transmitting the LTM Candidate Timing Advance Command MAC CE to update the maintained TA on the UE side for the target cell. At this point, the TA sent by the network is more up-to-date than the TA maintained on the UE side (or the UE has no stored TA for the target cell). Therefore, UE should store the TA value indicated in the LTM Cell switch Command MAC CE for the target cell, and the C-LTM TAT for the target cell should be started or restarted. This ensures that the target cell maintains an up-to-date TA value for the subsequent CLTM procedures.

Proposal 7: [MAC-V04] Upon receiving an LTM Cell switch Command MAC CE with valid TA, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

- TA update of CLTM TAT when receiving an TA command of serving cell

Similar to the logic discussed above, upon receiving an (Absolute) Timing Advance Command MAC CE or a RAR from the serving cell, the UE should also update the TA value of any candidate cell(s) that share the same PCI as the serving cell(s) in the TAG, and start or restart the CLTM TAT to ensure the stored TA value for the CLTM candidate cell remains up to date.

Proposal 8: [MAC-V05] Upon receiving an (Absolute) Timing Advance Command MAC CE or a RAR, UE shall store the TA value and start or restart the CLTM TAT of the candidate cell(s) that share the same PCI as the serving cell(s) of the TAG.

  - [MAC-O02] Valid PUCCH resource for SR of L1 MR

The dedicated SR configuration for L1 MR was introduced in Rel-19, as specified in following running CR:

5.4.4 Scheduling Request The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission. The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel or for SCell beam failure recovery (see clause 5.17) and for consistent LBT failure recovery (see clause 5.21), at most one PUCCH resource for SR is configured per BWP. For a logical channel serving a radio bearer configured with SDT, PUCCH resource for SR is not configured for SDT. For beam failure recovery of BFD-RS set(s) of Serving Cell, up to two PUCCH resources for SR are configured per BWP. For positioning measurement gap activation/deactivation request, a dedicated SR configuration is configured. For event triggered L1 measurement report, a dedicated SR configuration may be configured. Each SR configuration corresponds to one or more logical channels and/or to SCell beam failure recovery and/or to consistent LBT failure recovery and/or to beam failure recovery of a BFD-RS set and/or to positioning measurement gap activation/deactivation request and/or to event triggered L1 measurement report. Each logical channel, SCell beam failure recovery, beam failure recovery of a BFD-RS set and consistent LBT failure recovery, and event triggered L1 measurement report, may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered a BSR (clause 5.4.5) or a DSR (clause 5.4.9) or the SCell beam failure recovery or the beam failure recovery of a BFD-RS set or the consistent LBT failure recovery (clause 5.21) (if such a configuration exists) or positioning measurement gap activation/deactivation request (clause 5.25) or event triggered L1 measurement report (clause 5.x) is considered as corresponding SR configuration for the triggered SR. Any SR configuration may be used for an SR triggered by Pre-emptive BSR (clause 5.4.7) or Timing Advance reporting (clause 5.4.8).

For legacy SR transmission, the selection of which valid PUCCH resource for the SR to signal SR, was discussed and specified in the MAC specification as in following notes:

NOTE 1: Except for the cases specified in NOTE 3 below, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation. NOTE 2: If more than one individual SR triggers an instruction from the MAC entity to the PHY layer to signal the SR on the same valid PUCCH resource, the SR_COUNTER for the relevant SR configuration is incremented only once. NOTE 3: When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid. NOTE 4: For a UE operating in a semi-static channel access mode as described in TS 37.213 [18], PUCCH resources overlapping with the set of consecutive symbols where the UE does not transmit before the start of a next channel occupancy time are not considered valid. NOTE 5: If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the valid PUCCH resource for the SR transmission can be signalled by the physical layer and the SR transmission occasion overlaps with the PUSCH duration of an uplink grant of a MSGA payload. NOTE 6: When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set. NOTE 7: If an SL-SCH resource overlaps with the PUCCH resource for the SR transmission occasion for the pending SR triggered by Uu MAC CEs except BSR/SL-BSR MAC CE, and the MAC entity is not able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource, it is left to UE implementation to determine whether this SR transmission is prioritized over the SL transmission.

The general principle is that the PUCCH resource for pending SR for SCell beam failure recovery, or for pending SR for beam failure recovery of a BFD-RS set, should be considered as valid as the highest priority when overlapping with other PUCCH resources (as in NOTE 3); the selection of these two is left to UE implementation (as in NOTE 6).

Observation 4: [MAC-O02] The PUCCH resource for pending SR for SCell beam failure recovery, or for pending SR for beam failure recovery of a BFD-RS set, is prioritized over other PUCCH resources.

we think it is also better to clarify the case when pending SR for event triggered L1 measurement report, is overlapping with other PUCCH resources.

Considering the priority of MAC CE for (Enhanced) BFR and the priority of L1 measurement report MAC CE, is the same, as follows:

5.4.3.1.3 Allocation of resources <omitted…> Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation, or MAC CE for Event Triggered L1 Measurement Report; <omitted…>

Observation 5: [MAC-O02] The priority of MAC CE for (Enhanced) BFR and the priority of L1 measurement report MAC CE is the same.

We think the handling of pending SR for event triggered L1 measurement report, can be the same as pending SR for SCell beam failure recovery and for pending SR for beam failure recovery of a BFD-RS set. Therefore, we propose:

Proposal 9: [MAC-O02] The PUCCH resource for event triggered L1 measurement report, is considered as valid as the highest priority when overlapping with other PUCCH resources (other than PUCCH resources of pending SR for SCell beam failure recovery, or for beam failure recovery of a BFD-RS set).

Proposal 10: [MAC-O02] When PUCCH resource for pending SR for SCell beam failure recovery, for beam failure recovery of a BFD-RS set, and for event triggered L1 measurement report is overlapped, it is up to UE implementation to select one of them.

- Conclusion

In this paper, we discuss the remaining MAC open issues for LTM. We have the following observations and proposals:

[MAC-V01] RACH based LTM with MIMO 2TA

Observation 1: [MAC-V01] Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and MIMO two TA is supported.

Proposal 1: [MAC-V01] RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 2: [MAC-V01] if Option 3 in Proposal 1 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

[MAC-V02] CFRA resource in LTM Cell Switch Command MAC CE

Observation 2: [MAC-V02] For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 3: [MAC-V02] For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Proposal 4: [MAC-V02] Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

[MAC-V03] Impact of removing or modifying the L1 measurement reporting configuration

Observation 3: [MAC-V03] According to the current RRC specification, if a measurement identity or a reporting configuration/measurement object associated with that measurement identity is removed or modified, the UE will remove the measurement reporting entry for this measId from the VarMeasReportList, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this measId.

Proposal 5: [MAC-V03] Similar as L3 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed from or modified in the current UE configuration, the MAC entity shall remove the measurement reporting entry for this ltm-CSI-ReportConfigId from the MR_LIST, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

Proposal 6: If proposal 1 is agreed, adop the TP in Annex for MAC specification.

[MAC-V04/V05] TA update of CLTM TAT

Proposal 7: [MAC-V04] Upon receiving an LTM Cell switch Command MAC CE with valid TA, UE shall store the TA value for the target cell and start or restart the CLTM TAT of the target cell.

Proposal 8: [MAC-V05] Upon receiving an (Absolute) Timing Advance Command MAC CE or a RAR, UE shall store the TA value and start or restart the CLTM TAT of the candidate cell(s) that share the same PCI as the serving cell(s) of the TAG.

[MAC-O02] Valid PUCCH resource for SR of L1 MR

Observation 4: [MAC-O02] The PUCCH resource for pending SR for SCell beam failure recovery, or for pending SR for beam failure recovery of a BFD-RS set, is prioritized over other PUCCH resources.

Observation 5: [MAC-O02] The priority of MAC CE for (Enhanced) BFR and the priority of L1 measurement report MAC CE is the same.

Proposal 9: [MAC-O02] The PUCCH resource for event triggered L1 measurement report, is considered as valid as the highest priority when overlapping with other PUCCH resources (other than PUCCH resources of pending SR for SCell beam failure recovery, or for beam failure recovery of a BFD-RS set).

Proposal 10: [MAC-O02] When PUCCH resource for pending SR for SCell beam failure recovery, for beam failure recovery of a BFD-RS set, and for event triggered L1 measurement report is overlapped, it is up to UE implementation to select one of them.

- References

- R2-2406439, Discussion on MAC open issue for LTM, vivo.

- R2-2505397, Introduction of NR mobility enhancements Phase 4 in MAC

- RAN2#129bis_ChairNotes.

- R2-2507013, List of MAC open issues for R19 mobility, vivo.

Annex: TP for TS 38.321

5.x L1 measurement and event triggered report 5.x.1 Introduction The network may configure an RRC_CONNECTED UE to perform L1 beam level measurements for LTM candidate cell(s) and/or serving cell. The network may configure the UE to report them in accordance with the event triggered L1 measurement configuration. The measurement report is used for indicating to serving gNB of the L1 measurement results from the serving cell and/or candidate cell(s). The measurement configuration is provided by means of RRC dedicated signalling. The network may configure the UE to report the following measurement information based on SS/PBCH block(s): - Measurement results per SS/PBCH block; - SS/PBCH block(s) resource indicator (SSBRI). The network may configure the UE to report the following measurement information based on CSI-RS resources: - Measurement results per CSI-RS resource; - CSI-RS resource indicator (CRI). The RRC configures the following parameters in the LTM-CSI-ReportConfig for event triggered L1 measurement and corresponding reporting procedure: - LTM-CSI-ReportConfig for the event-triggered measurement report; - eventTriggered for the event-triggered measurement report; - eventLTM2, eventLTM3, eventLTM4, eventLTM5: events for the event-triggered measurement report; - timeToTrigger: time during which an entering/leaving condition needs to be consistently satisfied for reporting event triggered L1 measurement report or for cell switch execution to be met; - ltm-CandidateReportConfigList: List of report configurations for LTM candidate IDs; - ltm-EventTriggeredPeriodicReport: whether the event triggered L1 measurement report is sent periodically if an LTM event is triggered; - reportOnLeave: whether the event triggered L1 measurement report shall be triggered when the leaving condition for an event is satisfied; - ltm-EventTriggeredReportReportContent: the content of the event triggered L1 measurement report. If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, the MAC entity shall: 1> remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included: - stop the periodical reporting timer, if running, and reset the associated information (e.g. timeToTrigger) for the corresponding ltm-CSI-ReportConfigId. 5.y Conditional LTM 5.y.1 Introduction The UE may perform conditional LTM by using the L1 or L3 measurement for LTM cell switch conditions evaluation in accordance with the reconfiguration for conditional LTM. The RRC configures the following parameters for L1 trigger condition for CLTM procedure: - ltm-ExecutionCondition and ltm-ServingCellExecutionCondition for conditional LTM cell switch execution condition. If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, the MAC entity shall reset the associated information (e.g. timeToTrigger) for the corresponding ltm-CSI-ReportConfigId.