3GPP TSG-RAN WG2 Meeting #131bis	R2-2507064

Prague, Czech Republic, 13th ~17th Oct, 2025

Agenda Item:	8.17

Source: 	Huawei, HiSilicon

Title: 	Neighbour cell measurement in IoT NTN TDD

Document for:	Discussion and Decision

# 1	Introduction

In RAN#105, a new work item was approved to introduce NB-IoT TDD mode exclusively for 1616-1626.5 MHz Mobile Satellite Services MSS band [1]. During the RAN1#119 meeting, some agreements were made for the TDD pattern and the corresponding frame structure [2], as listed below.

Agreement： Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

In this paper, we further discuss the remaining issue of neighbour cell measurement in IoT NTN TDD.

# 2	Discussion

In the RAN2#131 meeting, the following agreements were reached for neighbour cell measurement.

Agreements: TDD radio frame offset between serving cell and neighbor cell(s) can be provided via SIB33 to the UE for neighbor cell measurement.

However, based on the agreement, it is not crystal clear about how the neighbour measurement is done with this offset. Now it is clear that the offset is to help UE detect the Frame boundary of the neighbour cell. Based on the Frame Structure specified in RAN1, there will be a set of consecutive valid downlink frames (8.28ms) within one 90ms frame structure. Therefore, in order to measure the neighbour cell, UE should monitor the neighbour downlink reference signal from the start of the consecutive valid downlink frames which lasts 8.28ms, based on the TDD radio frame offset.

Proposal 1: RAN2 clarifies that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration.

Furthermore, in legacy, for inter-frequency measurements in RRC_CONNECTED to reduce RRC re-establishment time, the UE can perform neighbour cell measurements during DL/UL idle periods that are provided by DRX or packet scheduling based on existing TS 36.300.

TS 36.300: For NB-IoT, measurements in RRC_CONNECTED are optionally supported to reduce the time taken for RRC reestablishment. The following principles are applied: - The "current cell" above refers to the configured carrier in the serving cell. The "target cell" above refers to the anchor carrier in the target cell. For instance, for the definition of intra and inter frequency measurements, this means: - Intra-frequency neighbour (carrier) measurements: Neighbour carrier measurements performed by the UE are intra-frequency measurements when the configured carrier in the serving cell and the anchor carrier in the target cell operates on the same carrier frequency. The UE shall be able to carry out such measurements without measurement gaps. - Inter-frequency neighbour (carrier) measurements: Neighbour cell measurements performed by the UE are inter-frequency measurements when the configured carrier in the serving cell and the anchor carrier in the target cell operates on a different carrier frequency. The UE may not be able to perform such measurements without measurement gaps. - The eNB configures the criteria to perform measurements via broadcast signalling; - Dedicated measurements gaps are not supported. The UE may need to perform neighbour cell measurements during DL/UL idle periods that are provided by DRX or packet scheduling; - Measurement reporting is not supported.

Since this is a legacy feature, it should also be supported in IoT NTN TDD. Specifically in IoT NTN TDD, due to the new frame structure, most of time UE is in non-valid subframes. To reduce the interruption of service due to measurement, RAN2 should clarify UE should be able to perform the inter-frequency neighbour cell measurement during the invalid UL and DL periods.

Proposal 2: RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD.

# 3	Conclusion

The observations and proposals made in this contribution are summarized below:

Proposal 1: RAN2 clarifies that UE should perform neighbour cell measurement within the 8.28ms duration from the start of the valid downlink duration.

Proposal 2: RAN2 clarifies that inter-frequency neighbour cell measurement for RRC reestablishment is supported in invalid UL and DL periods in IoT NTN TDD.

# 4	References

- RP-242415, New WID on introduction of IoT-NTN TDD mode

- Chair notes RAN1#119 eom1