3GPP TSG-RAN WG2 Meeting #131bis	R2-2507086

Prague, Czech Oct 13th – 17th, 2025

Source:	ZTE Corporation, Sanechips

Title:	RRC corrections (RILZ001, RILZ002) for R19 IoT NTN

Agenda Item:	8.9.2

Document for:	Discussion and Decision

Introduction

RAN2 has completed the basic functions of R19 IoT NTN in RAN2#131 meeting.

In this contribution, we will elaborate our RIL comments for RRC spec, e.g., RILZ001 and RILZ002.

Discussion

  - Elaboration of RILZ001 for IoTNTN

The current definition for periodicity of (N)PUSCH resources and CB-Msg3 Tx window are as below:

eMTC: CB-Msg3-ConfigList-r19 ::= SEQUENCE (SIZE (1.. maxCE-Level-CB-Msg3-r19)) OF CB-Msg3-Config-r19 CB-Msg3-Config-r19 ::= SEQUENCE { cb-Msg3-TBS-r19 ENUMERATED {b144, b328, b408, b504, b600, b712, b808, b936}, cb-Msg3-NumOfReplicas-r19 INTEGER(1..4), cb-Msg3-TimeResource-r19 SEQUENCE { pusch-Periodicity-r19 ENUMERATED {sf2, sf4, sf8, sf16, sf32, sf64, sf128, sf256}, pusch-StartSFN-r19 INTEGER (0..1023), pusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-MPDCCH-Config-r19 CB-Msg3-MPDCCH-Config-r19, cb-Msg3-PUCCH-Config-r19 CB-Msg3-PUCCH-Config-r19, cb-Msg3-PUSCH-Config-r19 CB-Msg3-PUSCH-Config-r19, cb-Msg3-TxWindow-r19 SEQUENCE { windowSize-r19 ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128}, windowPeriodicity-r19 ENUMERATED {n1, n2, n4, n8, n16, n32, n64, n128} OPTIONAL --Need OP } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-r19 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240}, cb-Msg3-MaxAttemptNum-r19 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} OPTIONAL, --Need OP ... } CB-Msg3-ConfigSIB field descriptions. <skip>. cb-Msg3-TxWindow CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. When windowPeriodicity is absent, the window periodicity uses the same value as windowSize. Value n1 corresponds to 10ms, n2 corresponds to 20ms and so on.. NB-IoT: CB-Msg3-Config-NB-r19 ::= SEQUENCE { cb-Msg3-TBS-NB-r19 ENUMERATED {b144, b328, b408, b504, b584, b680, b808, b936}, cb-Msg3-NumOfReplicas-NB-r19 INTEGER (1..4), cb-Msg3-TimeResource-NB-r19 SEQUENCE { npusch-Periodicity-r19 ENUMERATED {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}, npusch-StartSFN-r19 INTEGER (0..1023), npusch-StartSubframe-r19 INTEGER (0..9) }, cb-Msg3-PhysicalConfig-r19 SEQUENCE { npusch-NumRUsIndex-r19 INTEGER (0..7), npusch-NumRepetitionsIndex-r19 INTEGER (0..7), npusch-SubCarrierSetList-r19 SEQUENCE (SIZE(1..48)) OF NPUSCH-SubCarrierSet-r19, npusch-MCS-r19 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13, p0-UE-NPUSCH-r19 INTEGER (-8..7), alpha-NB-r19 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npdcch-CarrierIndex-r19 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP npdcch-NumRepetitions-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-CSS-r19 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} }, cb-Msg3-TxWindow-NB-r19 SEQUENCE { windowSize-NB-r19 ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}, windowPeriodicity-NB-r19 ENUMERATED {n8, n16, n32, n64, n128, n256, n512, n1024}, } OPTIONAL, --Need OP cb-Msg3-ResponseWindow-NB-r19 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}, cb-Msg3-MaxAttemptNum-NB-r19 ENUMERATED {n2, n3, n4, n5, n6, n7, n8, n10} OPTIONAL, --Need OP ... } CB-Msg3-ConfigSIB-NB field descriptions. <skip>. cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. When windowPeriodicity-NB is absent, the window periodicity uses the same value as windowSize-NB. Value n8 corresponds to 80ms, n16 corresponds to 160ms and so on.. -----

Per our understanding, the CB-Msg3 Tx window is primarily used in the scenario of multiple replicas, aiming to provide as sufficient PUSCH resource opportunities as possible for the random distribution of multiple replicas transmission, while trying to minimize collisions between different UEs' selections and also ensuring that the uplink transmission for a UE does not last very long. Additionally, besides the size of CB-Msg3 Tx window, the system also allows defining a periodicity of CB-Msg3 Tx window. The Tx window periodicity is continuous in the time domain, and the Tx window size can be configured shorter than the Tx window periodicity. This means that within each Tx window periodicity, the system may configure only a portion of time for PUSCH (CB-Msg3) transmission, leaving the remaining time available for other type transmissions. In other words, a certain gap can be left between two Tx windows to accommodate other transmissions.

In previous meeting, there was some discussion on how to define the CB-Msg3 Tx window size and periodicity, e.g., what the unit is, but no exact agreement. As a compromise, during [post-RAN2#131-306] email discussion, both CB-Msg3 Tx window size and periodicity are defined with unit of absolute time duration, e.g., 10ms. Moreover, the value range of windowPeriodicity is the same as that of windowSize.

For the current definitions of windowSize and windowPeriodicity, we at least see the following issues:

- Firstly, according to the definition of windowPeriodicity, if its value needs to be identical to windowSize, then this parameter windowPeriodicity could be absent (that means no gap between two Tx windows). Therefore, when windowPeriodicity is explicitly configured, its value needs to be different from the configured value of windowSize. Based on the current value range, the smallest difference between a value in windowPeriodicity and its nearest counterpart in windowSize is two times. For example for eMTC, if the PUSCH resource periodicity is configured as 2ms and windowSize is set to n1, e.g., 10ms (meaning 5 PUSCH periodicities/5 PUSCH resources are provided within a Tx window), then windowPeriodicity either is absent—indicating the same value as windowSize—or, if explicitly configured, must be set to at least n2, e.g., 20ms or even larger. This implies that between two Tx windows, there will be a gap of at least one Tx window length. The larger the windowSize, the larger this gap becomes. We think such gap of at least one Tx window length is unnecessary and would result in long intervals between PUSCH transmission opportunities.

Observation 1: With current value range of windowPeriodicity and windowSize, if windowPeriodicity needs to be explicitly configured (e.g., different from windowSize), it at least needs to be two times the configured value of windowSize. This may cause the gap between two CB-Msg3 Tx window unnecessary large.

- Secondly, it can be observed that windowSize's values are also not so suitable. For example, for some small value pusch-Periodicity, e.g., 2ms, typically windowSize has to be set to either 5 (e.g., windowSize=n1=10ms) or 10 (e.g., windowSize=n2=20ms) times the pusch-Periodicity. In a scenario where 2 replicas are configured, it may wish to provide PUSCH resources that are 2 to 3 times the number of replicas to achieve suitable dispersion. That means it expects 4 or 6 PUSCH resources within a CB-Msg3 Tx window. If 5 PUSCH resources within a Tx window is considered insufficient, the another alternative is to configure 10 PUSCH resources, which is too coarse a granularity. This could lead to either insufficient or excessive PUSCH transmission opportunities within a CB-Msg3 Tx window that is difficult to adapt to the number of replicas. In another example, e.g., pusch-Periodicity=sf16 while windowSize=n4=40ms, that means the value of windowSize couldn’t guarantee multiple complete PUSCH transmission opportunities and cause unnecessary delay.

Observation 2: With current value range of windowSize, it can only ensure that the PUSCH resources available within the CB-Msg3 Tx window are integer multiples of 5 (e.g., 5 PUSCH resources, 10 PUSCH resources, and so on), or are not integer multiples at all. The granularity of multiples of 5 is too coarse, making the number of PUSCH resources within a Tx window difficult to adapt to the number of replicas, e.g., resulting in either insufficient or excessive PUSCH resources relative to the number of replicas.

Based on the above observations, we think the most suitable way to define windowSize and windowPeriodicity is with the unit of (N)PUSCH resources periodicity. We suggest to define a 4-bit field for consecutive integers, allowing up to 16 PUSCH resources. This ensures sufficient resources are available even when the number of replicas is configured as 4, e.g., providing up to 4 times the PUSCH resources. Furthermore, windowSize and windowPeriodicity would not to have a multiple relationship and can be configured flexibly. For example, windowSize can be configured with 6 PUSCH resources while windowPeriodicity can be configured with 8 PUSCH resources, meaning that a gap between the two Tx windows is only 2 PUSCH resources.

Proposal 1: It’s suggested to define windowSize and windowPeriodicity with the unit of (N)PUSCH resources periodicity as below:

For eMTC:

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19

INTEGER (1..16),

windowPeriodicity-r19

INTEGER (1..16)    OPTIONAL	--Need OP

}                                                                           OPTIONAL,	--Need OP

cb-Msg3-TxWindow CB-Msg3 transmission window configuration. Both windowSize and windowPeriodicity are configured in multiples of the CB-Msg3 PUSCH periodicity. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. When windowPeriodicity is absent, the window periodicity uses the same value as windowSize.

For NB-IoT:

cb-Msg3-TxWindow-NB-r19			SEQUENCE {

windowSize-NB-r19

INTEGER (1..16),

windowPeriodicity-NB-r19

INTEGER (1..16),

}                                                                          OPTIONAL,	--Need OP

cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. Both windowSize-NB and windowPeriodicity-NB are configured in multiples of the CB-Msg3 NPUSCH periodicity. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. When windowPeriodicity-NB is absent, the window periodicity uses the same value as windowSize-NB.

  - Elaboration of RILZ002 for IoTNTN

For NB-IoT, RAN1 has indicated to define the frequency domain resource npusch-SubCarrierSetIndex as a set and all the sets can be shared by the UEs. Currently it’s defined in RRC spec as below:

….. CB-Msg3-Config-NB-r19 ::= SEQUENCE { <skip> cb-Msg3-PhysicalConfig-r19 SEQUENCE { npusch-NumRUsIndex-r19 INTEGER (0..7), npusch-NumRepetitionsIndex-r19 INTEGER (0..7), npusch-SubCarrierSetList-r19 SEQUENCE (SIZE(1..48)) OF NPUSCH-SubCarrierSet-r19, npusch-MCS-r19 CHOICE { singleTone INTEGER (0..10), multiTone INTEGER (0..13) }, ack-NumRepetitions-NB-r19 ACK-NACK-NumRepetitions-NB-r13, p0-UE-NPUSCH-r19 INTEGER (-8..7), alpha-NB-r19 ENUMERATED {al0, al04, al05, al06, al07, al08, al09, al1}, npdcch-CarrierIndex-r19 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP npdcch-NumRepetitions-r19 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1}, npdcch-StartSF-CSS-r19 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64}, npdcch-Offset-CSS-r19 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} }, <skip> ... } <skip> NPUSCH-SubCarrierSet-r19 ::= CHOICE { khz15 INTEGER (0..18), khz3dot75 INTEGER (0..47) } <skip> -- ASN1STOP

As reference, the definition of subCarrierSet for PUR in RAN1 spec is cited in below:

TS 36.213 16.5.1.1 Resource allocation The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE - a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB - a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB - a repetition number () determined by the repetition number field according to Table 16.5.1.1-3, and for a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers; except for NPUSCH with 16QAM where NRep=1. For NPUSCH transmission with subcarrier spacing, where is the subcarrier indication field and is reserved, or nsc is configured by higher layers parameter npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources. For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI or npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1. Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with . Subcarrier indication field (); Set of Allocated subcarriers (). 0 – 11. 12-15. 16-17. 18. 19-63; Reserved. -----

For the definition in RAN2 spec, we at least see the following issues and give suggestions for correction:

- For current npusch-SubCarrierSetList, it firstly define a number of set and for each set, it can make choice between configuration for subcarrier spacing of 3.75kHz and subcarrier spacing of 15kHz. However, we understand the subcarrier spacing should be consistent among all the frequency resource set. So the correct way should be to define two separate lists for subcarrier spacing of 3.75kHz and subcarrier spacing of 15kHz respectively. And in each list, there are several sets.

- For current npusch-SubCarrierSetList, the maximum number of sets is 48, we see no clear justification for defining such number and doubt this number may be at least incorrect for the case of subcarrier spacing of 15kHz. Based on the observation in first bullet, we assume the number of sets to be different for the separate lists for subcarrier spacing of 3.75kHz and subcarrier spacing of 15kHz, according to the available set resources.

- Taking subcarrier spacing of 15kHz as example, we understand different subcarrier indications correspond to different set of allocated subcarriers. In other word, the subcarrier indication is segmented (e.g., the first segment is 0~11, the second segment is 12~15 etc.), with different segments corresponding to subcarrier configurations of different scales. In order to guarantee the UEs select different sets within the list of subcarrier spacing of 15kHz can have similar scale of subCarriers resources, we tend to think for a set configured with a value in a subcarriers indication segment, the other sets need also to be configured with the value in the same subcarriers indication segment. Based on this understand, we think the maximum number of sets can be set to 12, corresponding to the case that the subcarriers indication configured for each set is from the subcarriers indication segment of 0~11. For other subcarriers indication segment, we assume the resulted number of possible sets would be smaller.

Based on above observations, the following proposal is given:

Proposal 2: It’s suggested to use the following way to define npusch-SubCarrierSetList-r19:

npusch-SubCarrierSetList-r19 ::=		CHOICE {

npusch-SubCarrierSetList-khz15			SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75		SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

}

Conclusion

Based on the above discussion, the following proposals are given:

Proposal 1: It’s suggested to define windowSize and windowPeriodicity with the unit of (N)PUSCH resources periodicity as below:

For eMTC:

cb-Msg3-TxWindow-r19			SEQUENCE {

windowSize-r19

INTEGER (1..16),

windowPeriodicity-r19

INTEGER (1..16)    OPTIONAL	--Need OP

}                                                                           OPTIONAL,	--Need OP

cb-Msg3-TxWindow CB-Msg3 transmission window configuration. Both windowSize and windowPeriodicity are configured in multiples of the CB-Msg3 PUSCH periodicity. The start time of the CB-Msg3 transmission window is aligned with the PUSCH start time indicated by pusch-StartSFN-r19 and pusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-r19 equals 1, this field is absent. When windowPeriodicity is absent, the window periodicity uses the same value as windowSize.

For NB-IoT:

cb-Msg3-TxWindow-NB-r19			SEQUENCE {

windowSize-NB-r19

INTEGER (1..16),

windowPeriodicity-NB-r19

INTEGER (1..16),

}                                                                          OPTIONAL,	--Need OP

cb-Msg3-TxWindow-NB CB-Msg3 transmission window configuration. Both windowSize-NB and windowPeriodicity-NB are configured in multiples of the CB-Msg3 NPUSCH periodicity. The start time of the CB-Msg3 transmission window is aligned with the NPUSCH start time indicated by npusch-StartSFN-r19 and npusch-StartSubframe-r19. When cb-Msg3-NumOfReplicas-NB-r19 equals 1, this field is absent. When windowPeriodicity-NB is absent, the window periodicity uses the same value as windowSize-NB.

Proposal 2: It’s suggested to use the following way to define npusch-SubCarrierSetList-r19:

npusch-SubCarrierSetList-r19 ::=		CHOICE {

npusch-SubCarrierSetList-khz15			SEQUENCE (SIZE(1..12)) OF INTEGER (0..18),

npusch-SubCarrierSetList-khz3dot75		SEQUENCE (SIZE(1..48)) OF INTEGER (0..47)

}