# 3GPP TSG RAN WG2 Meeting #131b      	                        R2-2507117
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.1.2.2

Source:	Apple, ZTE Corporation, Sanechips

Title:	Discussion on RIL issues related to predictionConfiguration-r19 (A105/N021/H003)

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the latest RRC CR of Rel-19 AI/ML PHY [1], the IE predictionConfiguration-r19 implements the following configurations:

- Inference configuration of CSI prediction (csi-InferencePrediction-r19)

- Inference configuration of BM (configurationForChannelPrediction-r19)

- Data collection configuration of CSI prediction (share csi-InferencePrediction-r19)

- Data collection configuration of BM (share configurationForChannelPrediction-r19)

- Monitoring configuration of CSI prediction (configurationForChannelMonitoring-r19 )

- Monitoring configuration of BM (share configurationForChannelMonitoring-r19 )

]], [[ nrofReportedRS-v19xy ENUMERATED {n6, n8} OPTIONAL, -- Need R reportQuantity-r19 ReportQuantity-r19 OPTIONAL, -- Need R predictionConfiguration-r19 CHOICE { csi-InferencePrediction-r19 ENUMERATED {true}, configurationForChannelPrediction-r19 SEQUENCE { resourcesForChannelPrediction-r19 CSI-ResourceConfigId OPTIONAL, -- Need R associatedIdForChannelPrediction-r19 AssociatedId-r19 OPTIONAL, -- Need R associatedIdForChannelMeasurement-r19 AssociatedId-r19 OPTIONAL, -- Need R nrofReportedPredicted-RS-r19 ENUMERATED {n1, n2, n3, n4} OPTIONAL, -- Need R nrofTimeInstance-r19 ENUMERATED {n1, n2, n4, n8} OPTIONAL, -- Need R timeGap-r19 ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL, -- Need R ... }, configurationForChannelMonitoring-r19 SEQUENCE { refToPredictionConfig-r19 CSI-ReportConfigId, nrofBestBeamForMonitoring-r19 ENUMERATED {n1, n2} OPTIONAL, -- Need R nrofTransmissionOccasion-r19 ENUMERATED {n1, n3, n7, n15} OPTIONAL, -- Need R timeInstanceFor-RS-PAI-r19 ENUMERATED {n1, n2, n8, spare1} OPTIONAL, -- Need R mappingToResourcesForChannelPrediction-r19 BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OPTIONAL, -- Need R timeInstanceFor-SGCS-r19 ENUMERATED {n1, spare3, spare2, spare1} OPTIONAL, -- Need R ... } } OPTIONAL -- Need R ]] ReportQuantity-r19 ::= CHOICE { none-BM-r19 NULL, none-CSI-r19 NULL, p-CRI-r19 NULL, p-SSB-Index-r19 NULL, p-CRI-RSRP-r19 NULL, p-SSB-Index-RSRP-r19 NULL, rs-PAI-r19 NULL, sgcs-r19 NULL } Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases.

There are some RILs proposed to fix/improve this IE, including A105, N021, H003. In this contribution, we discuss how to address these RILs.

# 2 Discussion

## 2.1 Description of A105/N021/H003

A105, N021, H003 can be described as follows:

A105: [Description]: In existing implementation, BM and CSI prediction share the same monitoring configuration (configurationForChannelMonitoring-r19). However, we understand that separate parameters are needed for them: - Only refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 are used for CSI prediction - refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are used for beam management. [Description]: The existing mixed structure between BM and CSI are difficulty to read. To improve the readiness, we propose to have separate monitoring configuration for beam management and CSI prediction.

N021: [Description]: The extra hierarchy introduced by forcing a choice between predictionConfiguration-r19 and configurationForChannelMonitoring-r19 is unnecessary and does not have the advantage of grouping many parameters together. It should be obvious to the configuring entity that one or the other should be configured. [Proposed Change]: Eliminate one level of hierarchy by removing the choice element predictionConfiguration from CSI-ReportConfig and bring predictionConfiguration-r19 and configurationForChannelMonitoring-r19 to the main level of CSI-ReportConfig.

H003: [Description]: This is related to the following EN captured ubnder CSI-ReportConfig information element code: Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases. Currently “configurationForChannelPrediction-r19” is used for both inference and UE data collection. However, the notation “ForChannelPrediction” and “predictionConfiguration-r19” suggests that this parameter is only for inference. We think it is better to add a new parameter for data collection (e.g. configurationForUE-DataCollection-r19) and make “configurationForChannelPrediction-r19” applicable only for inference [Comments]: [WI CR rapporteur-v022]: We agree that the suggested change is a possibility to group the parameters and thus make the type of configuration clear. In the past we received input from some companies that we should avoid redundancy (the parameters for inference and those for data collection are the same), so we removed the separate CHOICE branch for UE-side data collection. Thus, we propose to discuss this issue in Tdocs, as for RIL N021, which is related.

## Discussion

First, we disagree to remove the CHOICE structure. We believe the existing CHOICE structure is useful for readiness and avoid misconfiguration. Thus, we propose:

Proposal 1: Reject N201, i.e. keep the CHOICE structure of predictionConfiguration-r19.

Next, we discuss the details of inference configuration of CSI prediction. In existing implementation, when csi-InferencePrediction-r19 is choosen, it indicates that it can be either inference configuariton of CSI prediction or data collection configuration of CSI prediction (i.e. the legacy CSI-ReportConfig simialr to Rel-18 CSI prediciton is used).

csi-InferencePrediction-r19         ENUMERATED {true}.

Observation 1: In existing implementation, when csi-InferencePrediction-r19 is choosen, it indicates that it can be either inference configuariton of CSI prediction or data collection configuration of CSI prediction (i.e. the legacy CSI-ReportConfig simialr to Rel-18 CSI prediciton is used).

We think this implementation can work. But we have similar concern as H003, and we prefer NW to explicitly indicate whether it is for inference or UE data collection. Thus, we propose to revise the type of csi-InferencePrediction-r19 to ENUMERATED {trueInference, dataCollection}.

Proposal 2: To differentiate whether it is inference configuration of CSI prediction or UE data collection configuration of CSI prediction, change the type of csi-InferencePrediction-r19 to ENUMERATED {trueInference, dataCollection}.

Then, we discuss the details of inference configuration of BM. We share the similar view as H003 to differentiate whether it is for inference or UE data collection of BM. But regarding to specification changes, we think it is not necessary to introduce a separate configurationForUE-DataCollection-r19. Instead, we can just add one explicit indication on whether it is for inference or UE data collection. Meanwhile, since configurationForChannelPrediction-r19 is now only used for BM, we tend to think it is better to rename it to configurationForBMPrediction-r19. Thus, we propose:

Proposal 3: Adopt H003 with a variant of specification change:

- Rename configurationForChannelPrediction-r19 to configurationForBMPrediction-r19.

- Introduce the following one explicit indication on whether it is for inference or UE data in configurationForBMPrediction-r19: BM-Inference-r19  ENUMERATED {true}

Finally, we discuss the details of performance monitoring configuration for BM and CSI prediction. In existing implementation, BM and CSI prediction share the same monitoring configuration (configurationForChannelMonitoring-r19). However, we understand that non-overlapping parameters are needed for them:

- Only refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 are used for CSI prediction.

- refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are used for beam management.

Observation 2: In existing implementation, BM and CSI prediction share the same monitoring configuration (configurationForChannelMonitoring-r19). However, non-overlapping parameters are needed for them:

- Only refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 are used for CSI prediction.

- refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are used for beam management.

To improve the readiness, we propose to have separate monitoring configuration for beam management and CSI prediction, i.e. adopt A105:

Proposal 4: Adopt A105, i.e. separate monitoring configuration for beam management and CSI prediction:

- Introduce configurationForCSIChannelMonitoring-r19 including refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 only for CSI prediction.

- configurationForBMChannelMonitoring-r19 including refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are only for beam management.

Finally, we provide a TP in Appendix to implementation Proposal 2/3/4. And we also provide Table 1 to summarize how to configure different cases.

Proposal 5: Adopt the TP in Appendix.

| Case | RRC configuration |
| --- | --- |
| Inference configuration of CSI prediction | Select “Inference” of csi-InferencePrediction-r19 |
| Data collection configuration of CSI prediction | Select “dataCollection” of csi-InferencePrediction-r19 |
| Inference configuration of Beam management of UE-side model | Select configurationForBMPrediction-r19 and its BM-Inference-r19 is absent. |
| UE-side data collection configuration of Beam management | Select configurationForBMPrediction-r19 and its BM-Inference-r19 is present. |
| Performance monitoring configuration of CSI prediction | Select configurationForCSIChannelMonitoring-r19 |
| Performance monitoring configuration of Beam management of UE-side model | Select configurationForBMChannelMonitoring-r19 |

Table.1 Summary of how RRC configures different cases

# 3 Conclusion

In this contribution, we have the following Observations:

Observation 1: In existing implementation, when csi-InferencePrediction-r19 is choosen, it indicates that it can be either inference configuariton of CSI prediction or data collection configuration of CSI prediction (i.e. the legacy CSI-ReportConfig simialr to Rel-18 CSI prediciton is used).

Observation 2: In existing implementation, BM and CSI prediction share the same monitoring configuration (configurationForChannelMonitoring-r19). However, non-overlapping parameters are needed for them:

- Only refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 are used for CSI prediction.

- refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are used for beam management.

Based on the observations, we have the following proposals:

Proposal 1: Reject N201, i.e. keep the CHOICE structure of predictionConfiguration-r19.

Proposal 2: To differentiate whether it is inference configuration of CSI prediction or UE data collection configuration of CSI prediction, change the type of csi-InferencePrediction-r19 to ENUMERATED {trueInference, dataCollection}.

Proposal 3: Adopt H003 with a variant of specification change:

- Rename configurationForChannelPrediction-r19 to configurationForBMPrediction-r19.

- Introduce the following one explicit indication on whether it is for inference or UE data in configurationForBMPrediction-r19: BM-Inference-r19  ENUMERATED {true}

Proposal 4: Adopt A105, i.e. separate monitoring configuration for beam management and CSI prediction:

- Introduce configurationForCSIChannelMonitoring-r19 including refToPredictionConfig-r19 and timeInstanceFor-SGCS-r19 only for CSI prediction.

- configurationForBMChannelMonitoring-r19 including refToPredictionConfig-r19, nrofBestBeamForMonitoring-r19, nrofTransmissionOccasion-r19, timeInstanceFor-RS-PAI-r19 and mappingToResourcesForChannelPrediction-r19 are only for beam management.

Proposal 5: Adopt the TP in Appendix.

# 4 References

[1] R2-2506530 - Introduction of AIML for NR air interface, Ericsson.

# Appendix

]],

[[

nrofReportedRS-v19xy                ENUMERATED {n6, n8}                         OPTIONAL,   -- Need R

reportQuantity-r19                  ReportQuantity-r19                           OPTIONAL,   -- Need R

predictionConfiguration-r19         CHOICE {

csi-InferencePrediction-r19         ENUMERATED {true}ENUMERATED {Inference, dataCollection},

configurationForChannelPrediction-r19configurationForBMPrediction-r19   SEQUENCE {

BM-Inference-r19                               ENUMERATED {true}

resourcesForChannelPrediction-r19           CSI-ResourceConfigId           OPTIONAL,   -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19               OPTIONAL,   -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19               OPTIONAL,   -- Need R

nrofReportedPredicted-RS-r19                ENUMERATED {n1, n2, n3, n4}   OPTIONAL,   -- Need R

nrofTimeInstance-r19                        ENUMERATED {n1, n2, n4, n8}     OPTIONAL,   -- Need R

timeGap-r19   ENUMERATED {ms10, ms20, ms40, ms80, ms160, spare3, spare2, spare1} OPTIONAL,   -- Need R

...

},

configurationForChannelMonitoring-r19configurationForBMChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

nrofBestBeamForMonitoring-r19               ENUMERATED {n1, n2}                          OPTIONAL,   -- Need R

nrofTransmissionOccasion-r19                ENUMERATED {n1, n3, n7, n15}               OPTIONAL,   -- Need R

timeInstanceFor-RS-PAI-r19                  ENUMERATED {n1, n2, n8, spare1}            OPTIONAL,   -- Need R

mappingToResourcesForChannelPrediction-r19  BIT STRING (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet))    OPTIONAL,   -- Need R

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}    OPTIONAL,   -- Need R

...

},

configurationForCSIChannelMonitoring-r19   SEQUENCE {

refToPredictionConfig-r19                   CSI-ReportConfigId,

timeInstanceFor-SGCS-r19                    ENUMERATED {n1, spare3, spare2, spare1}    OPTIONAL,   -- Need R

...

}

}                                                                                                           OPTIONAL    -- Need R

]]

ReportQuantity-r19 ::=   CHOICE {

none-BM-r19                 NULL,

none-CSI-r19                NULL,

p-CRI-r19                   NULL,

p-SSB-Index-r19             NULL,

p-CRI-RSRP-r19              NULL,

p-SSB-Index-RSRP-r19        NULL,

rs-PAI-r19                  NULL,

sgcs-r19                    NULL

}

Editor's Note: FFS whether/how to group the parameters (and whether/how to update the field descriptions) for prediction, monitoring, and UE-side data collection based on the beam management and CSI prediction use cases.