# 3GPP TSG RAN WG2 Meeting #131b      	                        R2-2507119
Prague, Czech Republic, Oct. 13 – 17, 2025

Agenda item:	8.1.3

Source:	Apple

Title:	Open issues on NW-side data collection (including H007/Z005/H009/H002/Z007)

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

By RAN2#131 [1], RAN2 has completed Rel-19 AI/ML for air interface. In this contribution, we discuss the following open issues related to NW-side data collection:

- Whether to introduce additional memory

- Discussion of important RIL

  - H007: Whether to discard the logged data related to the modified/released logging configuration

  - Z005/H009: Whether to capture interactions with PHY for NW-side data collection

  - H002: Retaining logged measurements during LTM

  - Z007: Whether to further capture restriction of multiplexing SON/MDT report and AIML logged data

# 2 Discussion

## 2.1 Whether to introduce additional memory

This is the open issue of UE capability discussion.

We still think additional memory is not needed for NW-side data collection in Rel-19 AI/ML:

- As we raised during online/offline discussion, the requirement to introduce additional memory in Rel-19 is not clear. In Rel-19, only L1 RSRP and beam ID are logged. Based on number provided by RAN1 reply LS (R1-2310681), we did an estimate that the typical training data size stored in AS layer memory is 100 bits8 instances (Set B) + 500 bits4 instances (Set A) = 2800 bits=0.35KB. Thus, 64KB is far beyond it.

- In addition, as RAN2 agreed, the memory will be refreshed when entering IDLE/INACTIVE/RLF, which further alleviates the need of additional memory.

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data

In all, we think proponent company should provide technique analysis on why the additional memory is needed instead of just repeating “it can be future proof”. But we are open for Rel-20 AI mobility and CSI compression, depending on detailed analysis by then.

Proposal 1: No need to introduce additional memory in Rel-19 NW-side data collection. RAN2 can revisit the conclusion in Rel-20 AI mobility and CSI compression based on specific new collected data type.

Furthermore, there was some proposal to couple additional memory with dataThresholdAvailabiltyIndication-r19. We also don’t see the need. In our understanding, they are totally separate topics.

Proposal 2: No need to couple additional memory with dataThresholdAvailabiltyIndication-r19.

## 2.2 Discussion of important RIL

In this section, we discuss some important RIL(s) raised by companies in ASN.1 review.

### 2.2.1 H007: Whether to discard the logged data related to the modified/released logging configuration

This RIL is described as follows:

[Description]: Currently it is possible for the network to provide an updated logged measurement configuration (i.e. reusing the same csi-LoggedMeasurementConfigId as already included in the UE conifguration). This may lead to ambiguity in the collected data, i.e. once it is reported it will be unclear to which configuration this data referred to. Similarly, the UE currently does not discard the collected data for a certain logging conifguration even when this conifguraiton is relased. If the UE is subsequently configured with a new clogging configuration later on, reusing the ID of the previously released configuration, similar ambiguity exists. Such situation should be avoided. [Proposed Change]: It is proposed to clarify that when the UE receives a modified logging conifguration or releases a logging configuration, the discards the logged data related to the modified/released logging configuration. This way the ambiguity mentioned above can be avoided. [Comments]: [WI CR rapporteur-v022] We suggest that companies discuss this RIL in Tdocs, since the UE behaviour would be somewhat modified.

We agree that the ambiguity exists upon reception of a modified logging configuration or release a logging configuration. However, we disagree with the solution suggested by RIL H007 (i.e. just discard the related logged data). Instead, we prefer to discard all logged data when such cases happen.

In legacy logged MDT, the UE maintains only one RAT-specific logged measurement configuration at a time, according to TS 38.331 [2]. Meanwhile, TS 37.320 [3] also captures:

- When the network provides an updated configuration, it completely replaces any previously configured logged measurement configuration.

- Any logged measurements associated with the previous configuration will be cleared. It is the network's responsibility to retrieve any relevant data before providing a new configuration

##### 5.1.1.1.2 Configuration effectiveness ===skip the unrelated part===== There is only one RAT-specific logged measurement configuration for Logged MDT in the UE. When the network provides a configuration, any previously configured logged measurement configuration will be entirely replaced by the new one. Moreover, logged measurements corresponding to the previous configuration will be cleared at the same time. It is left up to the network to retrieve any relevant data before providing a new configuration.

Observation 1: In legacy logged MDT, TS 37.320 captures:

- When the network provides an updated configuration, it completely replaces any previously configured logged measurement configuration.

- Any logged measurements associated with the previous configuration will be cleared. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

- It is left up to the network to retrieve any relevant data before providing a new configuration.

Due to similarity between NW-side data collection and logged MDT, we prefer to keep this principle for NW-side data collection. It means that all the logged data will be discarded upon reception of a modified logging configuration or release a logging configuration. Meanwhile, we think that the reasonable NW behaviour is to retrieve data before modification/releasing logging configuration. Thus, we propose:

Proposal 3 (RIL H007): Following similar principle of legacy logged MDT, NW is expected to retrieve data before providing an updated configuration, and all the remaining logged data (if any) will be discarded upon reception of a modified logging configuration or release a logging configuration.

### 2.2.2 Z005/H009: Whether to capture interactions with PHY for NW-side data collection

This RIL is described as follows (take H009 as example):

[Description]: This issue is related to an LS which RAN2 sent to RAN1 in R2-2506545. In particular, it is currently unclear whether any PHY-RRC ionteractions need to be captured in RRC and in RAN1 specifications. Currenlty there are two possibilities for NW-side data collection configurations: - Periodic logging - L3 event-based logging (either based on A1 or A2 event, i.e. L3-cell RSRP>threshold or L3-cell RSRP<threshold In our understanding, the responsibilities between the layers should be as follows: - PHY layer specifications should capture that the UE performs measurements and generates L1-RSRP/CRI based on data collection configuration in CSI-LoggedMeasurementConfig - PHY layer provides the generated measurement results to the RRC based on the configured periodicity - For periodic logging, PHY layer should perform measurements as soon as the conifguration is provided to the UE and should provide the results to RRC according to the configured periodicity - For event-based logging, PHY layer should only perform measurements and provide them to RRC when event conditions are met - RRC should be responsible for logging the measurements as provided by PHY layers and then for providing them to the network etc. In my understandning, the RRC part of the procedure should be modified as below. PHY layer specs should then capture that the UE peforms measurements and provides them to upper layers, upon being instructed by upper layers to do so (which is pending RAN1 discussion and actions based on RAN2 LS).

We agree with this RIL. Regarding to specific changes, we prefer the changes suggested by Z005, which puts a reference to TS 38.214. Thus, we propose:

Proposal 4 (RIL Z005/H007): On interactions with PHY for NW-side data collection, adopt the specification changes suggested by Z005.

### 2.2.3 H002: Retaining logged measurements during LTM

This RIL is described as follows:

[Description]: RAN2 made the following agreement: - RAN2 confirm that the solution agreed in RAN2#130 is applicable to regular HO and CHO (i.e. 1-bit indication corresponding to each candidate cell configuration in RRCReconfiguration is provided). LTM is not part of the agreement, but it seems there is nothing preventing the network from adding retainLoggedMeasurements also to the LTM candidate cells which also reuse RRCReconfiguraiton containers. [Proposed Change]: No change is needed, but RAN2 is requested to confirm that reatinLoggedMeasurements can also be used for LTM candidate configurations. [Comments]: [WI CR rapporteur-v022]: In our understanding, the intention of the proposal was to not do enhancements specific to LTM, while also not explicitly exclude it. However, for alignment, we suggest that companies discuss this RIL in a Tdoc.

In our understanding, LTM is not mentioned in RAN2#131 agreement because companies had concern that the solution agreed in RAN2#130 may not be applied to subsequent LTM. However, we think that smart NW implementation may still apply the solution to subsequent LTM. For example, NW may configure all the candidate LTM cells from the same infra-vendor based on its implementation.

Observation 2: On H002, although the solution agreed in RAN2#130 may not be applied to subsequent LTM, smart NW implementation may still apply the solution to LTM. For example, NW may configure all the candidate LTM cells from the same infra-vendor based on its implementation.

Thus, we think it is up to NW implementation whether to add retainLoggedMeasurements to the LTM candidate cells which also reuse RRCReconfiguraiton containers.

Regarding to specification change, we think it can be captured as a NOTE in stage 2 spec (TS 38.300). We provide an example text below:

NOTE:	It is up to Network implementation whether to include the indication in the RRCReconfiguration message for regular HO or CHO or LTM.

We also provide a TP in Appendix 1.

Proposal 5 (RIL H002): Capture the following NOTE in TS 38.300 (TP in Appendix 1).

NOTE:	It is up to Network implementation whether to include the indication in the RRCReconfiguration message for regular HO or CHO or LTM.

### 2.2.4 Z007: Whether to further capture restriction of multiplexing SON/MDT report and AIML logged data

This RIL is described as follows:

[Description]: According to RAN2 agreements achieved in RAN2#131bis meeting - Multiplexing of legacy SON/MDT report and AIML logged data is not supported in the same UE information response message. Up to the network to ensure that data is not requested at the same time But there is nowhere to capture this agreement in the RRC spec so that the data from SON/MDT and NW side data collection can be retrieved simultaneously from specification perspective In our understanding, it shall be captured in the field description of CSI-LogMeasReportConfig. [Proposed Change]: csi-LogMeasReportReq This field is used to indicate whether the UE shall report information about CSI radio measurements logged in RRC_CONNECTED for network-side data collection. This information element shall be absent if the logMeasReportReq is present.

CR Rapporteur provided the following response:

[Comments]: [WI CR rapporteur-v022]: We suggest to discuss this issue in a Tdoc. There were discussions in the past on whether there should be clear restrictions on the NW, by adding them in the field descriptions but some companies were not in favour of it. In the current specification, not allowing multiplexing is captured in the procedural text from clause 5.7.10.3 below, by using “else if”, which excludes the case of having both SON/MDT reports and AIML reports in the same UEInformationResponse message. It can be discussed further if this procedural text is sufficient or not to capture the RAN2 agreement. 1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else if csi-LogMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRBX; 2> discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: 2> submit the UEInformationResponse message to lower layers for transmission via SRB1.

We think the CR Rapporteur suggested way is not sufficient because it doesn’t preclude the case that both the logMeasReport and csi-LogMeasReport are included in the UEInformationResponse. When such configuration happens, the UE may follow the 1st if branch to report both logged MDT data and logged L1 measurement in SRB2.

Observation 3: On Z007, CR Rapporteur suggested way is not sufficient because it doesn’t preclude the case that both logMeasReport and csi-LogMeasReport are included in UEInformationResponse. When it happens, the UE may follow the 1st “if branch” to report both logged MDT data and logged L1 measurement in SRB2.

We noticed that the following NOTE is captured in Section 5.7.10.3:

NOTE: It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not requested in the same message.

Then, we think the simplest solution is to revise the NOTE to clarify that the NW don’t include both logMeasReport and csi-LogMeasReport in the same UEInformationResponse message.

We also provide a TP in Appendix 2.

Proposal 6 (RIL Z007): Adopt a variant of Z007: revise the existing NOTE in procedure text of Section 5.7.10.3 as follows (TP in Appendix 2).

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not simultaneously requested included in the same UEInformationResponse message.

# 3 Conclusion

In this contribution, we discuss open issues on NW-side data collection. Our Observations:

Observation 1: In legacy logged MDT, TS 37.320 captures:

- When the network provides an updated configuration, it completely replaces any previously configured logged measurement configuration.

- Any logged measurements associated with the previous configuration will be cleared. It is the network's responsibility to retrieve any relevant data before providing a new configuration.

- It is left up to the network to retrieve any relevant data before providing a new configuration.

Observation 2: On H002, although the solution agreed in RAN2#130 may not be applied to subsequent LTM, smart NW implementation may still apply the solution to LTM. For example, NW may configure all the candidate LTM cells from the same infra-vendor based on its implementation.

Observation 3: On Z007, CR Rapporteur suggested way is not sufficient because it doesn’t preclude the case that both logMeasReport and csi-LogMeasReport are included in UEInformationResponse. When it happens, the UE may follow the 1st “if branch” to report both logged MDT data and logged L1 measurement in SRB2.

Based on observations, we have the following proposals:

Proposal 1: No need to introduce additional memory in Rel-19 NW-side data collection. RAN2 can revisit the conclusion in Rel-20 AI mobility and CSI compression based on specific new collected data type.

Proposal 2: No need to couple additional memory with dataThresholdAvailabiltyIndication-r19.

Proposal 3 (RIL H007): Following similar principle of legacy logged MDT, NW is expected to retrieve data before providing an updated configuration, and all the remaining logged data (if any) will be discarded upon reception of a modified logging configuration or release a logging configuration.

Proposal 4 (RIL Z005/H007): On interactions with PHY for NW-side data collection, adopt the specification changes suggested by Z005.

Proposal 5 (RIL H002): Capture the following NOTE in TS 38.300 (TP in Appendix 1).

NOTE:	It is up to Network implementation whether to include the indication in the RRCReconfiguration message for regular HO or CHO or LTM.

Proposal 6 (RIL Z007): Adopt a variant of Z007: revise the existing NOTE in procedure text of Section 5.7.10.3 as follows (TP in Appendix 2).

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not simultaneously requested included in the same UEInformationResponse message.

# 4 References

[1] RAN2#131, Chair Notes.

[2] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[3] TS 37.320-v18.4.0, Radio measurement collection for Minimization of Drive Tests (MDT); Overall description;
Stage 2, 2025-6.

# Appendix 1 (TP on P5 in TS 38.300)

#### X.Y.2.2	Data Collection for Offline Model Training

NW-side data collection for NW-side model training can be initiated by OAM or by gNB. The following enablers are introduced for NW-side data collection for NW-side model over air interface:

-	The UE can be configured by gNB to log L1 measurements in the AS buffer and report them via a RRC message(s).

-	Both periodic and L3 measurement event-triggered data loggings are supported. The UE stores the logged data at the AS buffer. When the AS buffer for storing logged data becomes full, the UE stops measurement and logging for data collection. When the AS buffer reserved for storing logged data becomes full or reaches an absolute threshold (if configured), the UE indicates data availability to the network, as specified in TS 38.331[12].

-	When low power state is detected, the UE can indicate the low power state to the network. Upon reception of the low power state indication, the network should release the UE data collection configuration for NW-side model.

NOTE:	It is up to UE implementation how to determine the AS buffer threshold is reached and how to determine low power state.

-	Network can indicate UE whether the logged data should be kept or not during handover. When indicated to keep the logged data, UE retains it during handover and indicates its availability after handover.

NOTE:	It is up to Network implementation whether to include the indication in the RRCReconfiguration message for regular HO or CHO or LTM.

For UE-side data collection for UE-side model training, the network can configure whether UE is allowed to initiate a request for data collection configuration (e.g., UE’s preference to start or to stop data collection, preferred configuration from a list of candidate configurations provided by network). The network can also provide UE with data collection configuration or release the data collection configuration at any point in time, with or without UE request.

# Appendix 2 (TP on P6 in TS 38.331)

#### 5.7.10.3	Reception of the UEInformationRequest message

<Unrelated text Omitted>

1>	if the logMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB2;

2>	discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else if csi-LogMeasReport is included in the UEInformationResponse:

2>	submit the UEInformationResponse message to lower layers for transmission via SRBX;

2>	discard the logged measurement entries included in the csi-LogMeasInfoList from VarCSI-LogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

NOTE:	It is up to the network to ensure that logged data based on logMeasReportReq and csi-LogMeasReportReq are not simultaneously requested included in the same UEInformationResponse message.