3GPP TSG-RAN WG2 Meeting #131bis		R2-2507129

Prague, Czech Republic, 13th – 17th Oct. 2025

Agenda Item:	8.7.3 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining open issues related to RLC enhancements

Document for: 	Discussion and Decision

Introduction

This contribution discusses open issues related to RLC enhancement (i.e., RLC-V01, RLC-N01, RLC-N02) which is suggested to discuss based on contribution by the RLC spec rapporteur.

Discussion

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| vivo | When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-V01 |

For the issue RLC-V01, the company’s concern is that even though the receiving RLC entity discards obsolete RLC SDUs, the PDCP t-Reordering may be still running and waiting for receiving this outdated SDUs from the RLC layer and this can lead to unnecessary delays in packet delivery. For this, they want to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity upon t-RxDiscard expires.

In our view, as per the current specification, when the RLC entity starts a t-RxDiscard timer due to gap detection in RLC, the PDCP entity also detects a SN gap almost at the same time and starts a t-Reordering timer as well in most cases. The issue here may occur only when the length of PDCP t-Reordering timer is longer than the RLC t-RxDiscard timer, but we are wondering why the network configures longer PDCP t-Reordering timer than the RLC t-RxDiscard timer, although this configuration can cause this problem. With this understanding, the proper network configuration can handle this without spec impacts, i.e., length of both PDCP t-Reordering and RLC t-RxDiscard timer would be almost same.

Observation 1. The issue RLC-V01 can occur when the length of PDCP t-Reordering timer is longer than the RLC t-RxDiscard timer.

Proposal 1. (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N001] RX_Next in clause 7.1 The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x. Our suggestion is: a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires. | Issue Type: Not essential but important How to address it: based on companies’ contribution Issue Number: RLC-N01 |

For the issue RLC-N01, it would be good to update the definition of RX_Next considering the Rel-19 XR RLC enhancement. In order to keep the legacy text as much as possible, we have tried to update the definition of RX_Next as proposed by the rapporteur, i.e., “if t-RxDiscard is configured, xxx. Otherwise, xxxx.”. However, we eventually realize that the Nokia’s change can cover all cases, and it is the simplest one for this issue. This change is also backward compatible since, if t-RxDiscard is not configured, “the SN following the last in-sequence completely received RLC SDU” is equivalent to “the SN of the first RLC SDU not delivered to the upper layers but still waited for”.

Proposal 2. (RLC-N01) Update the definition of RX_Next as follow:

This state variable holds the value of the SN of the first RLC SDU not delivered to the upper layers but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

| Company | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| Nokia | [N002] Below NOTE in clause 5.3.3.3 NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU. Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard. Our suggestion is: NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. | Issue Type: Not essential not important How to address it: based on companies’ contribution Issue Number: RLC-N02 |

For the issue RLC-N02, considering the current RLC specification, positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN can already stop and reset t-PollRetransmit as shown below yellow highlight and there is no need to have further clarification for this in the NOTE.

#### 5.3.3.3 Reception of a STATUS report Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall: - if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit. NOTE: When all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

In our view, only new case due to Rel-19 XR enhancement should be clarified in this NOTE, i.e., when stopReTxDiscardedSDU is configured and all RLC SDUs with SNs up to and including POLL_SN are already indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit. Thus, we prefer to remove positively or negatively acknowledged case and include only the case caused by the Rel-19 XR enhancement in the NOTE.

Proposal 3. (RLC-N02) Update the NOTE in section 5.3.3.3 as follow:

NOTE:	When stopReTxDiscardedSDU is configured and all RLC SDUs with SNs up to and including POLL_SN are already indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. The issue RLC-V01 can occur when the length of PDCP t-Reordering timer is longer than the RLC t-RxDiscard timer.

Proposal 1. (RLC-V01) No need to indicate the SN of obsolete RLC SDUs to the receiving PDCP entity after t-RxDiscard expires.

Proposal 2. (RLC-N01) Update the definition of RX_Next as follow:

This state variable holds the value of the SN of the first RLC SDU not delivered to the upper layers but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

Proposal 3. (RLC-N02) Update the NOTE in section 5.3.3.3 as follows:

NOTE:	When stopReTxDiscardedSDU is configured and all RLC SDUs with SNs up to and including POLL_SN are already indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.