3GPP TSG-RAN WG2 #131bis	R2-2507151

Prague, Czech Republic, Oct. 13th - 17th, 2025

Source:                    	DENSO CORPORATION

Title:  	RRC corrections for Rel-19 NR NTN DL-CE

Document for:        	Discussion and decision

Agenda Item:         	8.8.2		RRC corrections

- Introduction

According to the chair notes for last meeting RAN2 #131[1], the following agreements are made for downlink coverage enhancement:

Agreements:

1.	RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

2	In connected mode the NW will only configure up to 4 STMC.

3.	UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

4.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

In the last meeting, it was agreed to introduce a mechanism that UE reports N closest reference locations to assist the network for configuring SMTC list. In this paper, we would like to discuss our proposals for some enhancements on current running CR.

- Discussion

First, we would like to propose to use list of indices for report of closest N reference locations instead of bitmap which is currently defined as referenceLocationReport in the running CR. Using bitmap is beneficial to reduce the data size of signaling, however, the network cannot distinguish which one is actually closest within the included reference locations. By using the list of indices, UE can indicate the order of the distance, and the information of order would be beneficial to configure more optimized SMTC list for the network.

Proposal 1: RAN2 to adopt list of indices for report of closest N reference locations instead of bitmap so that UE can indicate the order of distance

Next, we would like to propose to introduce distance threshold to determine whether a reference location is actually “close” or not. We think that the information would not be useful if the closest reference locations are actually far. Especially in case of bitmap, including specified number of reference locations without distance threshold might increase the possibility of mismatch between configured SMTC list and the actual UE position.

Proposal 2: RAN2 to introduce configurable distance threshold to determine whether a reference location is actually “close” or not

Additionally, if distance threshold is introduced and no reference location satisfies the threshold, it would not be meaningful to report the closest reference locations, and the network has no need to re-configure the SMTC list. Therefore, we would like to propose the following:

Proposal 2a: If distance threshold is introduced, UE can skip to report the reference locations if no reference location satisfies the threshold

Lastly, we would like to propose to discuss necessity of introducing prohibit timer of UEAssistanceInformation for reporting closest reference locations. As reporting the closest reference locations is just a supplementary feature, we think introducing prohibit timer is reasonable to avoid too many UEAssistanceInformation.

Proposal 3: RAN2 to discuss necessity of introducing prohibit timer of UEAssistanceInformation for reporting closest reference locations

- Summary and proposal

This paper discussed about enhancements on current running CR for downlink coverage enhancement and proposed the following:

Proposal 1: RAN2 to adopt list of indices for report of closest N reference locations instead of bitmap so that UE can indicate the order of distance

Proposal 2: RAN2 to introduce configurable distance threshold to determine whether a reference location is actually “close” or not

Proposal 2a: If distance threshold is introduced, UE can skip to report the reference locations if no reference location satisfies the threshold

Proposal 3: RAN2 to discuss necessity of introducing prohibit timer of UEAssistanceInformation for reporting closest reference locations

- References

[1] R2-2506203, “Report from Break-out session on NR-NTN and IoT-NTN”