3GPP TSG-RAN2#131bis 	R2-2507209

Prague, Czech Republic, 13th – 17th Oct., 2025

Agenda item:		8.12.2 (NR_MIMO_Ph5-Core)

Source:	LG Electronics Inc.

Title: 	Remaining issues on UEI beam reporting

Document for:	Discussion and Decision

1.	Introduction

During post e-mail discussion [1], some open issues were discussed, and RAN2 made some consensus for almost issues, but some issue seems to need further discussion.

In this contribution, we present our view on remaining issues.

2.	Discussion

During [1], RAN2 discussed some open issues. One of open issues is whether to release PUCCH resource if TAT of CG PUSCH expires while TAT of PUCCH is running, for the case where PUCCH resource and CG PUSCH resource are configured in different TAG. In addition, how to handle newly triggered UEI reporting upon TAT of CG PUSCH expiry was discussed.

RAN2 to select from one of the options for UEI beam reporting in the following scenario: Upon STAG TAT expiry associated with a SCell configured with a UEI report configuration, if the PUCCH resource of the UEI report configuration is configured on a PCell or PUCCH-SCell of a different TAG with a running TAT, the UE: - Option 1: releases the PUCCH resource. - Option 2a: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE transmits the PUCCH and does not transmit the PUSCH for the UEI beam report. - Option 2b: does not release the PUCCH resource. If a UEI beam report is triggered for the SCell, the UE does not transmit the PUCCH for the UEI beam report.

For Mode-B UEI report, there may be a case where CG PUSCH resource is configured for SCell and PUCCH resource for UEIRI (UE Initiated Report Indicator, a.k.a. first PUCCH) is configured on PCell (or PUCCH SCell), and the SCell and PCell belong to different TAG.

Currently, when TAT of the CG PUSCH resource expires and TAT of the PUCCH resource is running, the CG PUSCH resource is cleared and the PUCCH resource is not released. During [1], companies argued that, in this case, unnecessary UEIRI on PUCCH can be transmitted by newly triggered UEI reporting, and the PUCCH resource is occupied by the UE and cannot be reallocated for other uses. Thus, it was proposed to release the PUCCH resource along with clearing CG PUSCH.

However, if the PUCCH resource is released by TAT of other TAG expiry, another issue can be occurred. In the current specification, there is no restriction that the PUCCH resource for UEIRI cannot be configured for other UCIs, e.g. HARQ-ACK or CSI reporting. Thus, if the PUCCH resource is released, other UCIs associated the PUCCH resource cannot work on the PUCCH resource even if TAT of PUCCH is still running. Note that SR does not share same PUCCH resource according to RAN1 agreement.

Therefore, we think that releasing of the PUCCH resource should be handled independently per TAG, as in legacy, i.e. PUCCH resource belonging to other TAG should be not released.

Proposal 1. If CG PUSCH and PUCCH are configured in a TAG and if TAT of CG PUSCH expires in other TAG and TAT of PUCCH is running, the UE does not release the PUCCH resource.

If P1 is agreeable, for the case where the CG PUSCH is cleared but the PUCCH resource is not released, RAN2 needs to discuss how to handle newly triggered UEI reporting needs to be discussed.

When UEI reporting is triggered, UEIRI is transmitted first to indicate that there is CG PUSCH transmission subsequently. However, since CG PUSCH cannot be transmitted due to clearing the CG PUSCCH resource, there is no need to transmit UEIRI. Therefore, we propose not to transmit both PUCCH and CG PUSCH for newly triggered UEI reporting.

Proposal 2. If UEI beam report is newly triggered upon clearing CG PUSCH resource, the UE does not transmit both UEIRI on PUCCH and CG PUSCH.

The other open issue is that for Mode-A, how to stop PDCCCH monitoring after UEIRI transmission, if UL grant for UEI reporting is not received.

RAN2 already agreed that for Mode-A, Active time includes the time between the transmission of UEIRI and the reception of UL grant for UEI reporting, which is similar to SR.

When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time while: … - a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]).

However, RAN2 did not discussed how to handle the Active time if the network does not schedule UL grant for UEI reporting or the network fails to receive UEIRI, i.e. if UL grant for UEI reporting is not received. If the Active time is not stopped, the UE needs to monitor PDCCH continuously and this bring unnecessary power consumption.

Currently, when SR is sent on PUCCH and is pending, the UE stays in Active time until the pending SR is cancelled. If UL grant is not received, the UE increases SR_COUNTER for each SR transmission and if SR_COUNTER reaches to sr-TransMax, the UE initiate RA procedure and cancels the pending SR.

However, it is not easy to introduce the pending mechanism for UEI reporting procedure because UEI reporting procedure is specified in RAN1 specification and there is no pending mechanism. Thus, RAN2 needs to consider a new mechanism to handle Active time for UEI reporting.

During [1], two options are suggested.

  - Option 1: until the next PUCCH resource for the UEIRI.

  - Option 2: Introduce a maximum PDCCH monitoring time period.

In Option 1, when UEIRI is transmitted, the UE stays in Active time. When the next PUCCH resource is available, the UE stops staying in Active time. In Option 2, when UEIRI is transmitted, the UE stays in Active time and starts a timer. When the timer expires, the UE stops staying in Active time.

In our view, Option 1 may not be valid. This is because the periodicity of PUCCH resource can be configured with short value, so there may be enough time to receive UL grant for UEI reporting. In this sense, we think Option 2 is desirable way.

Proposal 3. For Mode-A, RAN2 introduce a new timer to handle Active time based on UEI reporting.

Proposal 4. For Mode-A, the new timer is started upon UEIRI transmission and is stopped upon expiry of the timer.

3.	Conclusion

In this document, we present our view on open issue, and made proposals as follows.

Proposal 1. If CG PUSCH and PUCCH are configured in a TAG and if TAT of CG PUSCH expires in other TAG and TAT of PUCCH is running, the UE does not release the PUCCH resource.

Proposal 2. If UEI beam report is newly triggered upon clearing CG PUSCH resource, the UE does not transmit both UEIRI on PUCCH and CG PUSCH.

Proposal 3. For Mode-A, RAN2 introduce a new timer to handle Active time based on UEI reporting.

Proposal 4. For Mode-A, the new timer is started upon UEIRI transmission and is stopped upon expiry of the timer.

4.	Reference

- [Post131][217][MIMO_Ph5] CR for TS 38.321 (Samsung)