3GPP TSG-RAN WG2 Meeting #131bis	R2-2507290

Prague, Czechia, 13th – 19th Oct, 2025

Agenda item:	9.7.2

Source:	Samsung

Title:	Initial discussion on voice over NB-IoT NTN

WID/SID:	IoT_NTN_Ph4

Document for:	Discussion and Decision

# Introduction

In RAN#108 the WID for IoT NTN Phase 4 was agreed [1], where the objective is the following:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

In this contribution, we have an initial discussion on voice over NB-IoT NTN.

# Discussion

## UP vs CP solution for voice support

- The Control Plane CIoT optimization was introduced in 3GPP Release 13 for NB-IoT and eMTC. This enhancement simplifies connection and data transmission procedures compared to previous releases. The UE does not need to establish security or data radio bearers in order to send data, and as a result data can be transmitted at a much earlier stage. This especially brings benefits for scenarios and deployments where the devices are transmitting infrequent and small amount of data.

- Some features of the CP CIoT optimization:

- Data can be transmitted in RRCConnectionSetupComplete-NB message

- Data is transmitted over NAS in DL/ULInformationTransfer-NB

- AS security does not need to be established and is thus not mandatory to be supported

- UE cannot be RRC Reconfigured

- No data radio bearer beyond SRB1bis (nor other radio bearers) are established

- The CP CIoT optimization is mandatory for NB-IoT devices and optional for eMTC. As a result, the majority of NB-IoT deployments are using CP CIoT. This is likely also the case for current NB-IoT NTN deployments. As a result, one of the main benefits of using CP solution for voice would be that there would be commonality with current terrestrial and non-terrestrial NB-IoT deployments.

- The UP CIoT optimization was also introduced for 3GPP Release 13 (and also for 5G NR in Release 15 – although this is not the term used). This enhancement mainly simplifies and makes the connection establishment procedure more efficient. It allows a network to suspend an RRC connection

- In order to support voice, there are certain general features that would either be required or highly beneficial:

- Support of multiple radio bearers (for voice and control of voice)

- Support of RRC-reconfiguring a UE

- Support of Robust Header Compression

- Support of QoS

- As a lot of these features are already supported for UP solution in one way or another, it would seem as if the CP solution would have to be “UP-ified” in order to support voice. Therefore, we believe that the main pro from supporting the CP solution comes from that this is the currently used solution in the market, while the main con with the CP solution is that to introduce voice over CP requires that the CP solution needs to have the same features as the UP solution.

Proposal 1: RAN2 agrees to the following observation: 
        - The main pro for supporting CP solution comes from current deployments and implementations pre-dominantly supporting CP solution
        - The main con for CP solution is that it requires significant re-designing to essentially “UP-ify” the CP-solution

- Conversely, the main pro for the UP solution is that most of the basic aspects are in place, owing to the UP solution being closer to 3G/4G, whose main use case has been the support of voice. The con is however that it is significantly less deployed in legacy networks.

Proposal 2: RAN2 agrees to the following observation: 
        - The main pro for supporting UP solution stems from the basic tools being in place for support of voice
        - The main con for UP solution is that it is significantly less deployed in legacy networks.

- As mentioned above, one of the main cons for the UP solution is that the UP solution has not been deployed on scale so far. One assumption may thus be that the deployments for current IoT use cases would have to be re-designed to support UP solution over the CP solution. This assumption may however be misguided, and we believe that CP solution could continue to be the main solution used to deliver infrequent and small data transmissions. To support this, we think that co-existence between CP and UP solution could be studied.

- Proposal 3: To address the issue of current deployments relying on CP solution, RAN2 shall study the co-existence between using CP solution for data transmission and UP solution for voice.

## Reply to LS from SA4

In RAN2#131, RAN2 received an LS from SA4 [2]. The questions to RAN2 was the following:

ACTION: SA4 kindly asks SA2 and RAN2 to comment on

- the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and

- specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In RAN2#131, a reply was discussed but it was agreed to postpone a reply.

R2-2505068	LS on the RAN simulation assumptions for ULBC (S4-251584; contact: Qualcomm)	SA4	LS in	Rel-20	FS_ULBC	To:RAN1, RAN2, RAN4, SA2, CT1	Cc:SA1

- -	Qualcomm thinks that we can try to respond with a simple answer on header sizes.

-	Vivo thinks we should wait as there will be discussions.  Samsung thinks that the header sizes will not change so we can at least respond on that.

-	ZTE thinks that in theory we can provide an answer like, if we go with CP vs. UP what would be the header size without deciding.

-	Vivo, Lenovo, Huawei and Xiaomi think that there are a lot of aspects to consider.  Lenovo doesn’t think it is urgent to provide a precise answer.

Noted

We will begin this work in RAN2#131bis and will attempt to answer to SA4 after some initial RAN2 work.

Regarding the MAC header that can be assumed, it should be noted that the current Work Item Description does not include any type of lower layer header compression, as was proposed by several companies. This means that RAN2 should be able to assume that the header sizes as already specified until 3GPP Release 19 should be applicable

Proposal 4: RAN2 assumes that currently specified header sizes and related procedure are applicable as is without enhancement.

The smallest MAC header is the R/F2/E/LCID MAC header. This MAC header can be used for NB-IoT for two cases: Fixed size MAC CEs or for the last sub-header. The last sub-header, may potentially be the only sub-header, or be a sub-header used for padding. This means that the 1 R/F2/E/LCID can only be assumed to be used for data transmissions when there is no padding and only a single MAC SDU. A single MAC SDU can potentially be assumed in a stable state where no MAC SDUs from other logical channels are triggered. This is because MAC CEs should be considered to be fairly rare. So the main issue is whether it can be assumed that there is no padding. This would mean that the size of the TBS have to match up perfectly with MAC SDU (including the voice packet) and headers. In the case that there is padding in the MAC SDU, then the MAC header is clearly not the only source of overhead, but also the padding. Calculating the overhead due to padding and the MAC headers is a lot more complicated.

Proposal 5: Reply to SA4 that 1 byte MAC sub-header can be assumed in absolutely ideal conditions, but in non-ideal conditions, the MAC sub-header will be 2 byte and there may be further overhead from MAC-related padding depending on the Transport Block Size.

## Reply to LS from SA2

For this meeting, RAN2 received an LS from SA2 [3]. Here we address the issues raised to RAN2.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

Several mechanisms can handle dynamically changing voice packets. For instance, RAN2 will introduce support for Semi-Persistent scheduling, and as part of this, in case of a network expecting the size of the voice packets to change dynamically, it is possible for the network to over-dimension transport block size by network implementation. Similarly, the network is able to increase or reduce the resources fairly dynamically. However, as always, these will not come for free in terms of coverage and network resource usage, however due to the constraints of high latency, no enhancement can completely solve this issue. RAN2 does not expect to introduce any specific enhancement to handle this, which  is also motivated by the need to keep NB-IoT simple.

Proposal 6: Reply to SA2 that there are network implementations related to SPS that may be able to handle or alleviate the issue of dynamically changing voice packets and that RAN2 does not intend to introduce enhancements specific to the issue.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

The number of DRBs supported by an UP NB-IoT UE was introduced in an attempt to make the UE simple. However, the number of supported DRBs are unlikely to make a difference in UE complexity. So RAN2 does not see any issues on supporting more DRBs such as 3 or 4 for UEs supporting voice.

Proposal 7: Reply to SA2 that it is feasible to support more than 2 DRBs without much effort.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

It would be possible to introduce a new SRB. However, there are several potential issues that may need to be resolved. For instance, it is not clear whether the current RRC messages used for the CP solution are suitable. Furthermore, an NB-IoT CP solution device would not need to implement any type of logical channel prioritization and with a new SRB, this would need to be implemented, straying further from the original CP-solution.

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

It is potentially possible to configure RLC UM for SRBs. However, there are some issues. For NB-IoT CP solution, the UE cannot be RRC-reconfigured. This means that the UE would have to be configured with an SRB with RLC UM when the UE connects to the cell, but the network does not necessarily know the capabilities of the UE when connecting. This means that the network would have to blindly configure the UE with the related parameters when UE connects.

Proposal 8: Reply to SA2 that it could be possible to define a new SRB for voice, but this may introduce issues such as the need for a CP solution NB-IoT UE to support logical channel prioritization.

Proposal 9: Reply to SA2 that it could be possible to support RLC UM for SRBs, but that there are issues for RAN2 to solve related how configure the UE with RLC UM.

## Voice connection establishment

When a normal 5G UE initiates a voice call, the UE indicates in the establishment cause that an MO voice call has been initiated. In order to prepare the network, in particular an NB-IoT network, it is important that the network is made aware that a voice call is initiated. This is so that the network can begin reserving increased resources for such a UE. This is because the number of UEs that the network can allocate for IoT traffic and for voice services would likely vastly differ.

For CP solution, an establishment cause in RRCConnectionRequest-NB would at least be required, while for UP solution an establishment/resume cause would need to be introduced in RRCConnectionRequest and RRCConnectionResumeRequest. However, for NB-IoT the resume cause reuses the establishment cause for RRC setup.

Proposal 10: At least introduce a new establishment cause for MO voice.

For establishing a connection for MO voice, it may be tempting to allow to use EDT. However, we believe that the main use for EDT, PUR and CB-Msg3-EDT is to deliver a small amount of data and to support the MO voice call establishment would be an unnecessary enhancement.

Proposal 11: EDT (including CB-Msg3-EDT) or PUR does not need to be supported for MO voice.

## Semi-persistent scheduling

One of the key solutions to make voice efficient is the introduction of semi-persistent scheduling. Semi-persistent scheduling was introduced for NB-IoT, but only for delivering Buffer Status reports in the uplink. This is can be useful but is clearly not sufficient for voice support, which is why the WID states that DL and UL SPS shall be supported.

For 4G LTE, SPS is configured via RRC, and activated via DCI with a specific RNTI. In 5G NR, there are two types of Configured Grant (Type 1 and Type 2), where in Type 1 the RRC message itself activates SPS and in Type 2 it works similar to 4G LTE. For NB-IoT, we believe that it would be suitable to follow 4G LTE, i.e Type 2.

Proposal 12: Downlink/Uplink SPS is activated using DCI as in 4G LTE SPS.

One issue with SPS is how the network is made aware of the UE supporting it. Normally, capabilities can be used to indicate to a network that SPS is supported, but there are some issues with this. For instance, it would take some time before a network is made aware that it is supported. Due to this, we believe that it could be useful to indicate at an early stage that SPS is supported. In our view, and given that SPS is crucial for voice support, we believe that the voice establishment cause shall indicate that SPS is supported by the UE.

Proposal 13: Using the voice establishment cause implicitly indicates that the UE supports SPS.

# Conclusion

In this contribution, we discussed issues related to voice over NB-IoT NTN:

Proposal 1: RAN2 agrees to the following observation: 
        - The main pro for supporting CP solution comes from current deployments and implementations pre-dominantly supporting CP solution
        - The main con for CP solution is that it requires significant re-designing to essentially “UP-ify” the CP-solution

Proposal 2: RAN2 agrees to the following observation: 
        - The main pro for supporting UP solution stems from the basic tools being in place for support of voice
        - The main con for UP solution is that it is significantly less deployed in legacy networks.

- Proposal 3: To address the issue of current deployments relying on CP solution, RAN2 shall study the co-existence between using CP solution for data transmission and UP solution for voice.

Proposal 4: RAN2 assumes that currently specified header sizes and related procedure are applicable as is without enhancement.

Proposal 5: Reply to SA4 that 1 byte MAC sub-header can be assumed in absolutely ideal conditions, but in non-ideal conditions, the MAC sub-header will be 2 byte and there may be further overhead from MAC-related padding depending on the Transport Block Size.

Proposal 6: Reply to SA2 that there are network implementations related to SPS that may be able to handle or alleviate the issue of dynamically changing voice packets and that RAN2 does not intend to introduce enhancements specific to the issue.

Proposal 7: Reply to SA2 that it is feasible to support more than 2 DRBs without much effort.

Proposal 8: Reply to SA2 that it could be possible to define a new SRB for voice, but this may introduce issues such as the need for a CP solution NB-IoT UE to support logical channel prioritization.

Proposal 9: Reply to SA2 that it could be possible to support RLC UM for SRBs, but that there are issues for RAN2 to solve related how configure the UE with RLC UM.

Proposal 10: At least introduce a new establishment cause for MO voice.

Proposal 11: EDT (including CB-Msg3-EDT) or PUR does not need to be supported for MO voice.

Proposal 12: Downlink/Uplink SPS is activated using DCI as in 4G LTE SPS.

Proposal 13: Using the voice establishment cause implicitly indicates that the UE supports SPS.

# Reference

RP-251867, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, Vivo, RAN#108, Prague, Czechia, June 2025.

R2-2505068, LS on the RAN simulation assumptions for ULBC, SA4, RAN2#131, Bengaluru, India, August 2025.

R2-2506747, LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC, SA2, RAN2#131bis, Prague, Czechia, October 2025.