3GPP TSG-RAN WG2 Meeting #131bis	R2-2507295

Agenda item:	8.1.2.2

Title:	Considerations on the remaining issue and RILs Z001/Z002 for LCM of UE side model

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#131, following agreements were reached:

Agreements on UE candidate data collection 1 Multiple preferred configurations within the list of candidate configurations provided by NW can be indicated by the UE via UAI. 2 No prohibit timer is needed for UE indicating its preferred data collection configuration. 3 On stop/start indication - The UE can send start indication (without a preferred list) to indicate preference to start data collection - The UE can send preferred list implying that it would like to start data collection on those configuration - The UE can send stop indication for all or a given actual CSI report config ID. - Rapporteur will determine best way of signaling. This doesn’t preclude merging 1 and 2. - Adopt below text in the field description of dataCollectionCandidateConfig: The UE is not expected to perform measurements solely based on the configurations provided by this IE. - Adopt the following solution: OtherConfig contains a list of candidate configurations as a list of a new IE, where each candidate configuration contains at least an identifier of the candidate configuration, CSI-ResourceConfigId for Set A, CSI-ResourceConfigId for Set B, and related associated IDs, as agreed in RAN2#130. Each candidate configuration is associated with a cell ID. We will also include individual IEs for CSI prediction case. - Ask RAN1 what IEs are needed for CSI prediction and inform them of our agreements on BM and confirm if anything else is needed.

Agreements

- The procedures for UE data collection request for the CSI prediction use case are the same as for the beam management use case.

- The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

- RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

- Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

- RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

Agreements on associated ID

- Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

- Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

- We will not define areas.  The Associated ID is 24 bits.

4	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

Agreements Other Config for option B - RAN2 confirms that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. - For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters according to RAN1#121 agreement: associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19, TimeGap-r19, nroftimeinstance-r19, applicabilityConfigId-r19. - For Option B for BM Case 1/2, inference related parameter set is configured per serving cell.

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

2	Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

This contribution provides consideration on some remaining issue and RIL for LCM of UE side model.

Discussion

Agreements on associated ID

- Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

- Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.   FFS is we should have signalling indicating multi-cell.

- We will not define areas.  The Associated ID is 24 bits.

4	If the network does not provide the associated ID, it is up to UE implementation how to determine the applicability.

RAN2 has agreed associated ID shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment. FFS if we should have signaling indicating multiple-cell.

RAN2 also agreed that both single cell and multi-cell associated ID can be supported based on NW implementation. Network can allocate Associated ID to a single cell and/or multiple cells.

In our understanding, no matter an Associated ID is allocated for a single cell or is allocated to multiple cells, the Associated ID shall be unique within a PLMN, i.e. one Associated ID can only be associated with one same/similar beam deployment. From UE’s point of view, if one Associated ID value is allocated to multiple cells, UE should assume these cells have same/similar beam configuration.

One possible reason to have single-cell/multi-cell indication is to reuse Associated ID value in more than one cells whose beam configurations are different. With the single-cell indication, the associated ID is associated to the serving cell for which it is configured. Thus the same value can be configured in another serving cell without confusion in UE side. Thus:

- A single-cell Associated ID can be reused in more than one cells.

- Multiple-cell Associated ID and single-cell Associated ID can be of the same value. The  single-cell/multi-cell indication acts as anther bit of Associated ID.

Observation 1:	With single-cell/multi-cell indication, a single-cell Associated ID can be reused in more than one cells. The single-cell/multi-cell indication acts as an additional bit of Associated ID actually.

However, we don’t think reusing/extending Associated ID value is so important. The Associated ID is 24 bits long, which means there is maximum 16 million Associated IDs. It should be enough even if each Associated ID is allocated to only one cell.

Observation 2: 	RAN2 has agreed Associated ID is 24 bits long, which is enough even if each Associated is allocated to only one cell. Thus there is no need to reuse Associated ID by adding additional single-cell/multi-cell indication.

Further, considering the intention to change Associated ID from cell-specific to PLMN specific is to assign same Associated ID to muti-cells that share same beam configuration. So we wonder if we really need to allocate single-cell Associated ID to many cells?

And reusing Associated ID value also means one associated ID value will be associated to more than one beam configurations. This is contradictory to RAN2 agreements: Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.

Observation 3: 	Reusing Associated ID with single-cell/multi-cell indication will result in a situation that one Associated ID is associated to more than one beam deployments, which is contradictory to RAN2 agreement.

Taking above considerations and the additional complexity to have both single-cell and multi-cell Associated ID into account, we don’t think such enhancement is necessary.

Proposal 1: 	Signalling indicating whether Associated ID is multi-cell or single-cell is not supported. All associated IDs are unique within a PLMN no matter it is allocated to a single cell or to more than one cells.

In RAN2#131bis, RAN2 agreed that any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report can be included in RRCReconfigurationComplete message.

However, it is still unclear whether UE can activate such configured inference configuration if it is configured with periodic CSI reporting and its applicability status is reported as applicable via RRCReconfigurationComplete.

Agreements on applicability/inapplicability

1	RRCReconfigurationComplete shall include applicability/inapplicability status for:

a)	All inference configurations included in the immediately preceding RRCReconfiguration message, and

b)	Any previously configured inference configurations for which applicability/inapplicability has already been reported and whose applicability status has changed since the last report.   [CB on how the UE handles previously configured periodic CSI config that becomes applicable]

2	Do not introduce a link, explicit or implicit, between a full inference configuration and a set of inference-related parameters. If the applicability of a full inference configuration changes and there is a corresponding set of inference-related parameters whose applicability changes at the same time, the UE shall report the applicability of both

There are two options:

- Option 1: To follow handling for the initial configured inference configuration that is included in the immediately preceding RRCReconfiguration. UE will automatically activate the inference configuration after transmitting the RRCReconfigurationComplete.

- Option 2: To follow the previous RAN2 agreements regarding the activation of inference configuration with periodic CSI reporting when the change report is transmitted via UAI. UE should not activate the inference configuration when applicability change is reported via RRCReconfigurationComplete. Network is expected to activate the inference configuration instead.

From NW point of view, if option 1 is allowed, there are two activation status of an inference configuration with periodic CSI reporting when it is reported applicable from inapplicable:

- The inference configuration is activated when it is reported via RRCReconfigurationComplete

- The inference configuration is not activated when it is reported via UAI.

Therefore, NW has to keep the resource for the inference configuration even if UE reports inapplicable in case it becomes applicable later and be activated automatically by the UE. This is not good for network side resource efficiency.

Observation 4:  If the UE automatically activates the previously configured inference configuration with periodic CSI reporting when it changes from "inapplicable" to "applicable" and the change report is transmitted via RRCReconfigurationComplete, the network will handle two different activation status: the inference configuration is activated if the report is transmitted via RRCReconfigurationComplete, and the inference configuration is not activated if the report is transmitted via UAI.

Observation 5:  If the UE automatically activates the previously configured inference configuration with periodic CSI reporting when it changes from "inapplicable" to "applicable" and the change report is transmitted via RRCReconfigurationComplete, network has to prepare the inference configuration related resources even after UE reports it as inapplicable. This is not radio resource efficient.

Thus we prefer keep it simple by using the same activation method no matter the applicability report is transmitted via RRCReconfigurationComplete or via UAI.

Proposal 2:     When the previously configured inference configuration with periodic CSI reporting changes from "inapplicable" to "applicable", and the applicability report is transmitted via the RRCReconfigurationComplete , UE shall not activate this inference configuration automatically. The handling should be consistent with that when the report is sent via UAI.

Currently, for the applicability reporting via RRCReconfigurationComplete, it has been defined as below:

#### 5.3.5.3 Reception of an RRCReconfiguration by the UE /omit for short/ 2> if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation): 3> for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows: 4> set the applicabilityCellId to the serving cell index of the cell; 4> for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference; 4> for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation): 5> include an entry in the applicabilityInfoReportList and set the content as follows: 6> set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId; 6> set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId; 6> if the applicabilityStatus is set to inapplicable: 7> if the UE prefers to release the concerned ApplicabilitySetConfig, include releaseConfigurationPreference;

IN current running CR, for both inference configuration that is configured in CSI-MeasConfig and inference related parameter set in OtherConfig, if UE report inapplicable in either RRCReconfigurationComplete or UAI, UE can include releaseConfigurationPreference. This indicator is introduced to indicate that the UE has no proper AIML model for the inference configuration, and to indicate the preference to release it by network.

In our understanding, this indicator is useful for the inference configuration only. For the inference configuration, the network needs to prepare network resources. If the UE does not indicate releaseConfigurationPreference, network may assume the inference configuration may become applicable later, e.g. after UE fetches a proper model. If UE indicates releaseConfigurationPreference for an inference configuration, network can safely release the inference configuration and related network resources.

However, for inference related parameter set this is a different story. Information included in OtherConfig is not real configuration. Network does not prepare inference related resources for it. Thus there is no hurt to keep the inapplicable inference related parameter set in UE side. There is no need for NW to explicitly remove it with additional signaling overhead.

Based on above consideration, we think ApplicabilitySetConfig should not be applied for inference related parameter sets.

Proposal 3: 	[Z001/Z002] IE ApplicabilitySetConfig is not applicable for the inference related parameter sets. RAN2 to adopt the text proposal provided in the Annex.

In last RAN2 meeting, RAN2 reached some agreements related to applicability reporting for SCG inference configuration received in RRCResume. And at the same time, RAN2 agreed that for no NR-DC enhancements are considered and will not ask RAN3 work in Rel-19[1]

Agreements

- The procedures for UE data collection request for the CSI prediction use case are the same as for the beam management use case.

- The applicability reporting procedures for CSI prediction are the same as for beam management. RAN2 confirms that option B is not supported for CSI prediction, given no parameters were provided by RAN1.

- RAN2 understand that when network releases inference configurations of poor-performed applicable functionalities, network may also provide either non-AI/ML configuration in CSI-ReportConfig or may provide full inference configuration of other applicable functionalities, if previously not configured to UE. There is no spec impact and feedback from the NW to the UE to adjust the applicability determination procedure is not supported in Rel-19.

- No enhancements are pursued for reporting applicability in RRCReestablishmentComplete.

- The UE can report applicability via RRCResumeComplete for SCG inference configurations received in RRCResume, without specification impact beyond already agreed applicability reporting procedure.

- Applicability reporting is added in RRCResumeComplete for inference configurations that exist at the UE based on legacy procedures (restored or received in RRCResume).

- RAN2 assumes for no NR DC enhancements are considered and will not ask RAN3 work in Rel-19.

As the discussion held in last RAN2 meeting, during the RRC resume procedure, including inference configuration in RRCResume message and including applicability reporting in RRCResumeComplete for SCG cell comes for free. This is because the SCG configuration is included in RRCResume as a RRC container which includes a Reconfiguration for SCG, and the corresponding RRCReconfigurationComplete is included in RRCResumeComplete and transmitted via PCell. Thus nothing needs to be changed to support inference configuration for SCG cells.

With the similar reason, configuring inference configuration and/or inference related parameter set for SCG cells via RRCReconfiguration are already supported since the SCG RRCReconfiguration use the same IE as that for MCG cells. Thus all IEs introduced for these two use cases can be applied for SCG too.

On the other hand, there is no inter-node negotiation mechanism to negotiate UE's PU capability between MN and SN. Since the MN and SN can configure inference configuration independently, they may compete for the UE’s PU capability. Considering RAN2 has already agreed there will be no NR-DC enhancement and will not ask RAN3 work in Rel-19, it is up to UE decision on the applicability of inference configuration configured for MCG cell and SCG cell if they are configured at same time. I.e. UE determines applicability according to its PU capabilities.

Based on above consideration, we have following proposal:

Proposal 4: 	RAN2 to confirm that beam management and CSI prediction use case are supported for SCG cells in NR-DC configuration without need for NR-DC enhancement. When inference configurations for both MCG cell and SCG cell are included in same RRC message, It is up to UE implementation to decide the applicability.

Conclusion

Observation 1:	With single-cell/multi-cell indication, a single-cell Associated ID can be reused in more than one cells. The single-cell/multi-cell indication acts as an additional bit of Associated ID actually.

Observation 2: 	RAN2 has agreed Associated ID is 24 bits long, which is enough even if each Associated is allocated to only one cell. Thus there is no need to reuse Associated ID by adding additional single-cell/multi-cell indication.

Observation 3: 	Reusing Associated ID with single-cell/multi-cell indication will result in a situation that one Associated ID is associated to more than one beam deployments, which is contradictory to RAN2 agreement.

Observation 4:  If the UE automatically activates the previously configured inference configuration with periodic CSI reporting when it changes from "inapplicable" to "applicable" and the change report is transmitted via RRCReconfigurationComplete, the network will handle two different activation status: the inference configuration is activated if the report is transmitted via RRCReconfigurationComplete, and the inference configuration is not activated if the report is transmitted via UAI.

Observation 5:  If the UE automatically activates the previously configured inference configuration with periodic CSI reporting when it changes from "inapplicable" to "applicable" and the change report is transmitted via RRCReconfigurationComplete, network has to prepare the inference configuration related resources even after UE reports it as inapplicable. This is not radio resource efficient.

Proposal 1: 	Signalling indicating whether Associated ID is multi-cell or single-cell is not supported. All associated IDs are unique within a PLMN no matter it is allocated to a single cell or to more than one cells.

Proposal 2:     When the previously configured inference configuration with periodic CSI reporting changes from "inapplicable" to "applicable", and the applicability report is transmitted via the RRCReconfigurationComplete , UE shall not activate this inference configuration automatically. The handling should be consistent with that when the report is sent via UAI.

Proposal 3: 	[Z001/Z002] IE ApplicabilitySetConfig is not applicable for the inference related parameter sets. RAN2 to adopt the text proposal provided in the Annex.

Proposal 4: 	RAN2 to confirm that beam management and CSI prediction use case are supported for SCG cells in NR-DC configuration without need for NR-DC enhancement. When inference configurations for both MCG cell and SCG cell are included in same RRC message, It is up to UE implementation to decide the applicability.

References

- RAN2#131 chairman notes

Annex A

NEXT CHANGE

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

<Text Omitted>

2>	if the UE has at least one stored application layer measurement configuration with appLayerIdleInactiveConfig configured which has not been successfully transmitted since entering RRC_CONNECTED state:

3>	include measConfigReportAppLayerAvailable;

2>	if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6:

3>	include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration;

2>	if, for at least one serving cell, the RRCReconfiguration message includes in csi-ReportConfigToAddModList at least one CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19; or

2>	if the RRCReconfiguration message includes at least one entry in applicabilityConfigList within applicabilityReportConfig; or

2>	if, for at least one serving cell, the UE is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation) ; or

2>	if the UE is configured with at least one entry in applicabilitySetConfigList for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either in RRCReconfigurationComplete or UEAssistanceInformation):

3>	for each serving cell associated with any of the configurations above, include an entry in the applicabilityReportList and set the content as follows:

4>	set the applicabilityCellId to the serving cell index of the cell;

4>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation):

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the csi-ReportConfigId within applicabilityInfoReportId to the corresponding reportConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

6>	if the applicabilityStatus is set to inapplicable:

7>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

4>	for each entry within applicabilitySetConfigList associated with the concerned serving cell, that is included in the RRCReconfiguration message or for which the applicability status has changed since the last transmission of a message containing applicabilityReportList (either RRCReconfigurationComplete or UEAssistanceInformation) :

5>	include an entry in the applicabilityInfoReportList and set the content as follows:

6>	set the applicabilitySetId within applicabilityInfoReportId to the corresponding applicabilitySetConfigId;

6>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityInfoReportId;

<Text Omitted>

NEXT CHANGE

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

<Text Omitted>

1>	if transmission of the UEAssistanceInformation message is initiated to report assistance information about the applicability of configurations subject to applicability determination procedure according to 5.7.4.2:

2>	include applicabilityReportList in this UEAssistanceInformation message;

2>	for each serving cell:

3>	if the cell is configured with at least one reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction , or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed; or

3>	if the associated serving cell index was included in an entry in applicabilityConfigList within applicabilityReportConfig and the applicability status for at least one of the associated entries in applicabilitySetConfigList has changed: [RIL]: N034 AIML

4>	include an entry in applicabilityReportList in the UEAssistanceInformation message, and set the content as follows:

5>	set the applicabilityCellId to the serving cell index of the cell;

5>	for each configured reportConfigId associated to a CSI-ReportConfig including csi-InferencePrediction, or including reportQuantity-r19 set to p-CRI-r19 or p-SSB-Index-r19 or p-CRI-RSRP-r19 or p-SSB-Index-RSRP-r19, for which the applicability status has changed:

6>	include an entry in the applicabilityReportConfigIdList, AIML and set the content as follows:

7>	set the csi-ReportConfigId within applicabilityReportConfigId to the corresponding reportConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

7>	if the applicabilityStatus is set to inapplicable:

8>	if the UE prefers to release the concerned CSI-ReportConfig, include releaseConfigurationPreference;

5>	for each entry within applicabilitySetConfigList that changed applicability status associated with the concerned serving cell:

6>	include an entry in the applicabilityReportConfigIdList and set the content as follows:

7>	set the applicabilitySetId within applicabilityReportConfigId [RIL]: C077, AIML to the corresponding applicabilitySetConfigId;

7>	set the applicabilityStatus to the applicability status of the configuration corresponding to the applicabilityReportConfigId;

<Text Omitted>