GPP TSG-RAN WG2#131bis   									           R2-2507305

Prague, Czech Republic, Oct. 13th-17th

Agenda Item:	8.7.3

Source: 	NEC

Title:	        XR User Plane Corrections

Document for:	 Discussion and Decision

# Introduction

In this contribution, we will discuss several XR user plane issues:

- Applied priority determination (A.1)

- Shared MAC CE format for UL rate control query from UE and UL rate control recommendation from gNB (C.3); and a common or separated QoS flow list supporting UL rate control query and UL rate control recommendation (C.3, but RRC spec relevant)

# Discussion

MAC open issue 1: applied LCH priority determination

At the beginning of XR Rel19 WID discussion, a general requirement for LCP enhancement was to avoid increasing UE online processing time, meaning that the UE should not be required to perform additional complex processing in the interval between receiving a PDCCH for a new transmission and the associated PUSCH. However, as noted in the MAC CR text below, the priority applied to a LCH (whether it is an additional or default priority) is currently determined when the UE processes a grant for a new transmission. This approach results in increased UE online processing time.

The MAC entity shall, when a new transmission is performed: 1> if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for this transmission: 2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold; or 2> if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), evaluated at the time of the first symbol of this transmission, is less than the priorityAdjustmentThreshold: 3> consider this PDCP SDU being priority adjustable. 1> allocate resources to the logical channels as follows: 2> if a logical channel has a priority adjustable PDCP SDU: 3> apply additionalPriority of this logical channel; 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain: 3> if a logical channel has applied additionalPriority in the above allocation of resources and does not have any priority adjustable PDCP SDUs left: 4> apply priority of this logical channel; 3> all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels with equal applied priority should be served equally. NOTE 1: The value of Bj can be negative.

Since the remaining time of a PDCP PDU constantly decreases, we should check if any buffered PDCP PDU's remaining time is below the threshold at the start of a transmission, consequently, if additional priority should be applied. However, LCP for a new transmission happens before the start of the new transmission itself. According to the LCP timetable below:

- T1, UE receives a PDCCH for a new transmission.

- Between T1 and T2, the UE determines the applied priority for each selected LCH, used for the first round of LCP, using all buffered PDCP PDUs' remaining times at T4.

- From T2 to T3, the UE checks whether these prioritized PDCP PDUs will be transmitted based on the first round of LCP, if yes, default priority should be applied in the second round of LCP.

- At T4, the transmission happens.

| T1 | PDCCH decoded |
| --- | --- |
| T2 | First round of an LCP |
| T3 | Second round of an LCP |
| T4 | The beginning of PUSCH transmission |

Note that this situation is same for Bj, which also continuous increasing along the time for a logical channel and decrease after first round of LCP. In principle, Between T1 and T2, UE shall determine the Bj used for first round LCP, the Bj value should be the Bj value at the transmission time T4. Moreover, Bj shall be reduced the amount of data allocated resource at first round of LCP. However, we did not specify Bj maintenance inside of LCP procedure.

From UE implementation perspective, applied priority determination has more work comparing Bj, because Bj run per logical channel, but applied priority is determined based on the remaining time of every buffered PDCP PDU. But both require working out the status of a LCH at future.

We believe we can specify applied priority maintenance as same as Bj value maintenance, to make room for UE implementation to do as little as possible during the online processing window. Because we agreed that applied priority could fall back from additional priority to default priority if there is no more priority adjustable PDUs after first round of LCH, unfortunately, UE must check if the priority adjustable PDUs has gone after first round of LCP (step3 above is unavoidable).  However at least for the first round LCP, UE could determine the priority adjustable PDUs much ahead of time, may even before PDCCH for new transmission is decoded, as long as UE implementation make sure the applied priority is right at the time when a grant is processed by LCP (step2 above could be done anytime between LCP instances).

Proposal 1: Like Bj, we propose moving specification on applied priority determination out of the LCP procedure to allow greater flexibility for UE implementation.

MAC open issue 2: UL rate control

The issue is pointed out by company as following

In section 6.1.3.x, for the description of Fi, the index may vary for UL rate control and UL rate query due to the independent configuration for supported QoS flows as specified in RRC. In order to make the description clearer and more direct, we suggest describing rate control and rate control query separately to avoid misunderstanding. Similar issue also in 6.2.1, two MAC CEs for DL-SCH and UL-SCH are named the same. Follow the legacy description, we also see clearer if we name the UL-SCH MAC CE with “UL Rate Control Query”.

Because same MAC CE format will be used for UE send UL rate control query and for serving gNB to send UL rate control recommendation, it is a question whether we should have different name for the MAC CE and further with different MAC CE description text. We think there is a middle way keeping the description simple and clear: two different MAC CE names share the same MAC CE format and its description, as we did for truncated short BSR and short BSR, TP is provided.

Proposal2: for clearness and simplicity, define two MAC CE names “UL rate control recommendation” and “UL rate control Query” but share the same MAC CE format and its description.

Moreover, we fail to see the use case of have different Qos flow list supporting UL rate control query and UL rate control recommendation. it is useless to enable UL rate control query for a Qos flow if its rate cannot be recommended/adjusted by gNB. It also makes no sense to not allow UL rate control query for a QoS flow which gNB can recommend/adjust it’s the data rate.

ul-RateControlConfigList Includes the list of QoS flows for which the UL rate control is supported.

ul-RateQueryConfigList Includes the list of QoS flows for which the UL rate query is supported.

Proposal 3: The list of QoS flow supporting UL rate control query and rate control recommendation are the same, i.e. one configurable list is enough

# Conclusion

This contribution provides following proposals:

Proposal 1: Like Bj, we propose moving specification on applied priority determination out of the LCP procedure to allow greater flexibility for UE implementation

Proposal 2: for clearness and simplicity, define two MAC CE name “UL rate control recommendation” and “UL rate control Query” but share the same MAC CE format and its description.

Proposal 3: The list of QoS flow supporting UL rate control query and rate control recommendation are the same, i.e.one configurable list is enough

TP for P1 and P2 are provided, P3 requires change to RRC specification

# References

[1] R2-2506349 Introduction of XR enhancements Phase 3 MAC CR.

[2] Remaining MAC CR open issues in Rel-19 MAC CR.

# Annex TP

## TP #1 for proposal 1

----------------------------------------------------------------------------------------------------------

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level. It is the default priority used for a logical channel in the LCP procedure, unless specified otherwise;

-	additionalPriority which is applied instead of priority for a logical channel in the LCP procedure when the conditions specified in clause 5.4.3.1.3 are met;

-	priorityAdjustmentThreshold which is used to determine whether additionalPriority or priority is applied in the LCP procedure;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

Before every instance of resource allocation in the LCP procedure:

1>	if a logical channel is configured with priorityAdjustmentThreshold and has a PDCP SDU available for transmission:

2> if the PDCP entity associated with this logical channel is configured with pdu-SetDiscard, and the PDU Set remaining time of the PDCP SDU (as defined in TS 38.323 [4]), is less than the priorityAdjustmentThreshold; or

2>	if the PDCP entity associated with this logical channel is not configured with pdu-SetDiscard, and the remaining time of discardTimer of the PDCP SDU (as defined in TS 38.323 [4]), is less than the priorityAdjustmentThreshold:

3> use additionalPriority as applied priority for this logical channel.

1> else

2> use Priority as applied priority for this logical channel

NOTE:	The exact moment(s) when the UE updates Bj and update applied priority between LCP procedures is up to UE implementation, as long as Bj and applied priority is up to date at the time when a grant and when the remaining grant are processed by LCP.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing applied priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower applied priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing applied priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels with equal applied priority should be served equally.

NOTE 1:	The value of Bj can be negative.

NOTE 2:	the applied priority of a logical channel in above two resource allocation steps may be different.

## TP #1 for proposal 2,

Note: several other places should be updated accordingly as well

----------------------------------------------------------------------------------------------------------

#### 6.1.3.x	UL Rate Control Recommendation and UL Rate Control Query MAC CE

The UL Rate Control Recommendation MAC CE and UL Rate Control Query MAC CE are identified by a MAC subheader with an eLCID as specified in Table 6.2.1-1 and Table 6.2.1-2 for available bit rate recommendation from the serving gNB and bit rate query from the UE, respectively.

The UL Rate Control Recommendation MAC CE and UL Rate Control Query MAC CE share the same MAC CE format , and the MAC CE format has a variable size and consists of the following fields:

- Fi: This field indicates the presence of bit rate for the i:th QoS flow. The index i is the ascending order of the value defined by PDU Session ID × 64 + QoS Flow Identifier, where PDU Session ID (specified in clause 5.6.9 in TS 23.501 [x]) and QoS Flow Identifier (specified in clause 5.7.3 in TS 23.501 [x]) are those of the QoS flows configured to support UL rate control (as specified in clause 5.18.x). The Fi field set to 1 indicates that a bit rate for the i:th QoS flow is included in the MAC CE. The Fi field set to 0 indicates that no bit rate information for the i:th QoS flow is included. The Fi field is reserved and set to 0, if the number of QoS flows configured to support UL rate control is less than i;

- Bit Rate: When the serving gNB sends bit rate recommendation(s) in the UL Rate Control MAC CE, this field indicates a recommended bit rate for the QoS flow indicated by the Fi field. When the UE sends bit rate query(s) in the UL Rate Control MAC CE, this field indicates a preferred bit rate for the QoS flow indicated by the Fi field. Bit rates are included in the ascending order according to the Fi field. The MAC entity shall use the bit rates specified in Table 6.1.3.x-1 to set the value of this field. Each Bit Rate field is indicated in kbits/s and has a length of 8 bits.

Figure 6.1.3.x-1: UL Rate Control Recommendation MAC CE and UL Rate Control Query MAC CE