3GPP TSG-RAN WG2 Meeting #131bis	R2-2507315

Prague, Czech Republic, Oct 13th - 17th, 2025

Agenda item:		8.7.3

Source:		Samsung

Title:			Discussion on open issues for RLC and PDCP

Document for:	Discussion & Decision

# 1 Introduction

In [POST131][507] discussion, following open issues for RLC were collected:

Proposal 1: Following open issues are suggested to be discussed based on companies’ contributions: RLC-X01, RLC-V01, RLC-N01, RLC-N02. Proposal 2: Proponent companies are invited to provide proposals on the following issues, which Rapporetur understands there is no problem: RLC-E01, RLC-H02, RLC-S01. Proposal 3: Following open issues are suggested to be discussed during the CR phase: RLC-H01, RLC-V02.

In [POST131][506] discussion, following open issues were identified for PDCP:

Proposal 2 (H001). Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive. Proposal 3 (N001). Discuss whether to capture a NOTE to specify that “whether to re-send the gap report after HO can be left to UE implementation”.

# 2 Discussion

## 2.1 [RLC-X01]

Issue description: Whether RLC calculation of the non- delay reporting data still cannot be removed

Previosuly RAN2 agreed:

- [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold

- [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

The concern raised in this issue is that PDCP may not be aware about some of the RLC packets (e.g., before DSR is triggered there has been already some data before one portion). In our understanding, this is purely implementation matter e.g., it is quite common for implementation to share the buffer across PDCP and RLC, and only respective header building and processing tasks are managed across layers. Therefore, PDCP and RLC layers would have clear visibility about the buffer status. Further, it seems not reasonable to address such aspects from the specification perspective and restricts implementations in any manner.

[RLC-X01] Proposal 1: RAN2 to not pursue [RLC-X01] and maintain the previous agreements:

- [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold

- [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

## 2.2 [RLC-V01]

Issue description: Whether the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires

Proponent company raised the following point:

- When t-RxDiscard expires, the RLC entity stops waiting to receive the obsolete RLC SDUs, and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, the PDCP t-Reordering may be still running and waiting for receiving the outdated SDUs from the RLC layer, this can lead to unnecessary delays in packet delivery. Therefore, the receiving RLC entity should indicate the SN of obsolete RLC SDUs to the receiving PDCP entity when t-RxDiscard expires.

In our view, the issue raised is a valid concern, however, it is the PDCP SN gap report based standardized mechanism that is targetted to address the PDCP reordering delays. A new interaction across the Rx RLC and Rx PDCP layers is undesired. Note that the indication itself is not feasible with current RLC and PDCP specs (Rx RLC/PDCP entities may not know the exact PDCP COUNT/SN of the obsolete RLC SDU by t-RxDiscard expiry at Rx RLC), given that there is no guaranteed 1:1 mapping between PDCP SDU SN/COUNT and RLC SDU SN, due to the existence of PDCP control PDU (that doesn’t have PDCP COUNT/SN, but with RLC SN). Moreover, any such approach at receiver is quite delayed in itself by design and does not significantly help in avoiding the delays.

[RLC-V01] Proposal 2: RAN2 to not agree to [RLC-V01] given that 1:1 mapping between PDCP SDU SN/COUNT and RLC SDU SN is not guareenteed.

## 2.3 [RLC-N01]

Issue description: Correction on the definition of RX_Next

Proponent company raised the following point and suggested a text change as follows:

- The definition of RX_Next update should reflect t-RxDiscard expiry (to be aligned with the NOTE in 5.3.4) and text - which value of the SN is pointed - should be aligned with the procedural text in 5.2.3.2.x.

a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU of the first RLC SDU not delivered to the upper layers, but still waited for, and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

The first part of the change is not aligned with the legacy specification, that is, it affects the legacy definition so there is an issue for spec compatibility. Therefore, it needs to be amended to additionally include the case related to the detection of discard of AMD PDU(s). We understand the second part i.e., “or t-Rxdiscard expires” makes sense and is still compatible to the legacy as it just adds a new condition to it.

[RLC-N01] Proposal 3: Agree to only the second part of change for the [RLC-NO1] and adopt the amended text proposal in TP 1.

TP1:

a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU or of the first RLC SDU not delivered to the upper layers upon detecting discard of AMD PDU(s), and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

## 2.4 [RLC-N02]

Issue description: Correction on the NOTE about the condition to stop and reset the running t-PollRetransmit

Proponent company raised the following points and suggested a TP as follows:

- First, we should clarify that the NOTE is for the case where stopReTxDiscadedSDU is configured because stopping/resetting based on discard indication is limited to configuration of stopReTxDiscardedSDU.

- Second, it seems RAN2 has not carefully discussed ‘NACK’ part in the NOTE. To us, it seems not reasonable to stop polling even when there is an unACKed RLC SDU still under transmission, i.e., waiting for ACK/NACK after retransmission or before discard.

NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

In our view, first point seems reasonable as this pertains to the new scenario of stopping of transmisison and restransmisison of discarded SDU as introduced in Rel-19 RLC AM enhancements.

Regarding second point, we understand the only important consideration is that all RLC SDUs with SNs up to and including POLL_SN are indicated as discarded from upper layer, thereby there is no RLC SDU available for polling and the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

Both the positive or negative acknowledgement for POLL_SN based stopping of t-pollRetranmsit is already addressed (in normative manner with “shall”) by the legacy and the legacy condition is always satisfied as it is only about POLL_SN while the condition in the NOTE is about all the RLC SDUs. That implies, the legacy normative conditions of POLL_SN ACK/NACK already suffices for stopping t-PollRetransmit and we should exclude them from the NOTE. So we propose as follows:

[RLC-N02] Proposal 4: It is proposed that

- Agree only first part of [RLC-NO2] i.e., the condition “if stopReTxDiscardedSDU is configured” is added

- Do not agree second part of [RLC-NO2] and instead adopt an amended condition in the NOTE for only considering “when all RLC SDUs with SNs up to and including POLL_SN are indicated as discarded from upper layer”. (Adopt TP2)

TP2:

NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

## 2.5 [RLC-S01]

Issue description: Whether the PDCP triggers a PDCP SN gap report when at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers

The above issue was discussed in RAN2#131, however, discussion was not quite conclusive. While almost all companies acknowledged the issue arising with the Rel-19 RLC AM enhancements, one company remarked that the issue would also be relevant for the scenario of multiple RLC entities configured with the PDCP. Particularly this should be applied “also for completely submitted SDUs” and condition for “successful delivery of discarded PDCP SDU(s) have not been confirmed” be checked. However, we think this is not appropriate as the submitted SDUs could be subject to on-going HARQ transmissions and no clear status about the successful delivery can be judged at the time of Discard SN gap report triggering.

Considering to keep the solution simple and less impact to specification, we think the new condition can specifically restricted and be addressed as proposed in TP3.

[RLC-S01] Proposal 5: PDCP triggers a PDCP SN gap report when stopReTxDiscardedSDU is configured in all RLC entities and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. (Adopt TP3)

TP3:

TS 38.323 ## 5.16 SN gap report ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is configured in all RLC entities: - at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. - else: - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.

## 2.6 [PDCP H001]

Issue description: Discuss whether to change the definition of “Non-delay-reporting PDCP SDU” so that each of a non-delay-reporting PDCP SDU associated with the i:th DSR-ReportingThreshold is exclusive

During the discussion on the PDCP open issue, companies expressed a preference for revising the definition of non-delay-reporting PDCP SDUs. This suggestion arises primarily because the current definition allows a single non-delay-reporting PDCP SDU to be associated with multiple reporting thresholds.

However, during the calculation of the delay-reporting PDCP data volume, a single non-delay-reporting PDCP SDU is counted in only the smallest reporting threshold it is associated with, by conditions for preventing duplicate counting.

Therefore, from a technical point of view, the combination of current definition and data volume calculation for non-delay-reporting PDCP SDU can operate effectively without causing erroneous UE behavior.

Given that the current definition and data volume calculation have been refined through several rounds of discussions, we prefer to maintain them rather than initiating another round of refinement for a new definition, and the new procedure text tailored for it.

[PDCP H001] Proposal 6: RAN2 to maintain current definition of non-delay-reporting PDCP SDU unless any critical technical issues are identified.

## 2.7 [PDCP N001]

Issue description: Discuss whether to capture a NOTE to specify that “whether to re-send the gap report after HO can be left to UE implementation”.

In the RAN2#131, RAN2 has agreed that:

(RLC-12) No special handling is needed in R19 for PDCP SN gap report during UE mobility. It can be left to UE implementation whether to re-send the gap report after HO.

The intent of the agreement, especially, the first sentence, suggests that no changes to the specification are expected for R19 UE regarding PDCP SN gap report during UE mobility, considering that the significance of the issue to be resolved by the enhancement did not receive sufficient empathy. Although the second sentence supplements that the re-sending of the PDCP SN gap report may be considered for UE implementation, it does not necessitate a NOTE to explicitly emphasize this option, given that the re-sending is not explicitly prohibited in the specification and the agreement can also serve to imply such possibility. It is important to note that not all permitted UE implementations are explicitly captured by NOTE in the specification.

[PDCP N001] Proposal 7: RAN2 to not introduce a NOTE specifying re-sending the PDCP SN gap report after HO can be left to UE implementation.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

[RLC-X01] Proposal 1: RAN2 to not pursue [RLC-X01] and maintain the previous agreements:

- [RLC-13] PDCP should indicate to RLC what needs to be included in the data volume calculation in RLC for each threshold

- [RLC-13] We can remove definition of delay reporting and non-delay reporting data from RLC

[RLC-V01] Proposal 2: RAN2 to not agree to [RLC-V01] given that 1:1 mapping between PDCP SDU SN/COUNT and RLC SDU SN is not guareenteed.

[RLC-N01] Proposal 3: Agree to only the second part of change for the [RLC-NO1] and adopt the amended text proposal in TP 1.

[RLC-N02] Proposal 4: It is proposed that

- Agree only first part of [RLC-NO2] i.e., the condition “if stopReTxDiscardedSDU is configured” is added

- Do not agree second part of [RLC-NO2] and instead adopt an amended condition in the NOTE for only considering “when all RLC SDUs with SNs up to and including POLL_SN are indicated as discarded from upper layer”. (Adopt TP2)

[RLC-S01] Proposal 5: PDCP triggers a PDCP SN gap report when stopReTxDiscardedSDU is configured in all RLC entities and at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. (Adopt TP3)

[PDCP H001] Proposal 6: RAN2 to maintain current definition of non-delay-reporting PDCP SDU unless any critical technical issues are identified.

[PDCP N001] Proposal 7: RAN2 to not introduce a NOTE specifying re-sending the PDCP SN gap report after HO can be left to UE implementation.

# 4 Annex (Text Proposals)

TP1:

a) RX_Next – Receive state variable This state variable holds the value of the SN following the last in-sequence completely received RLC SDU or of the first RLC SDU not delivered to the upper layers upon detecting discard of AMD PDU(s), and it serves as the lower edge of the receiving window. It is initially set to 0, and is updated whenever the AM RLC entity receives an RLC SDU with SN = RX_Next or t-RxDiscard expires.

TP2:

NOTE: If stopReTxDiscardedSDU is configured, Wwhen all RLC SDUs with SNs up to and including POLL_SN are already positively or negatively acknowledged or indicated as discarded from upper layer (e.g., PDCP), the transmitting side of an AM RLC entity may stop and reset the running t-PollRetransmit.

TP3:

TS 38.323 ## 5.16 SN gap report ### 5.16.1 Transmit operation For UM DRBs and AM DRBs configured by upper layers to send a PDCP SN gap report in the uplink (sn-GapReport in TS 38.331 [3]), the transmitting PDCP entity shall trigger a PDCP SN gap report when: - PDCP SDU(s) are discarded as specified in clause 5.3; and - there is at least one stored PDCP SDU(s) which is associated with a COUNT value larger than the COUNT value associated to the discarded PDCP SDU(s); and - if stopReTxDiscardedSDU is configured in all RLC entities: - at least one byte for the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers. - else: - the discarded PDCP SDU(s) have not been submitted by any RLC entity to lower layers.