3GPP TSG-RAN WG2 Meeting #131bis	R2-2507410

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.10.3

Source:	Nokia

Title:	Some MRO related enhancements

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core WI_CODE - Release 19

Document for:	Discussion and Decision

# 1	Introduction

Following the review of the release 19 MRO agreements relating to LTM and CHO with candidate SCGs and their implementation in the current 38.331 CR, we are proposing an enhancement related to CHO with candidate SCGs.

# 2	Discussion

The current specification of the RLF report determination fails in adding the logging of the chronological fulfillment of the two execution conditions which realized with new IE choWithCandidateSCGInfoList.

The information of the chronologic sequence is only logged in new IE choWithCandidateSCGInfoList, if the UE is confgured for CHO with candidate SCGs (CwcS), i.e., the UE possesses condReconfigIds with two execution conditions condExecutionCond and condExecutionCondPSCell. Here the corresponding part from the specification as proposed in the running CR:

1>	for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows:

Observation 1: Logging of the chronological fulfilment of the two execution conditions of CHO with candidate SCGs is only carried out when UE is configured with CHO with candidate SCGs, i.e., it includes condReconfigIds including both condExecutionCond and condExecutionCondPSCell.

Those entries are in UE’s VarConditionalReconfig when UE has been configured with CwcS and the RLF occurs during the monitoring phase having at most only one condition fulfilled (referring to a too late CwcS execution provided that the reestablishCellId ≠ failedPCellId) or when both conditions are fulfilled but the CwcS execution itself fails (i.e., in case of HOF).

However, those condReconfigIds referring to CwcS will not exist for an RLF occurring shortly after the successful CwcS completion (case 8a from the agreed RAN3 failure sceanrios), since all mobility configurations related to previous source PCell are released and UE starts clean without condReconfigId referring to CwcS.

Observation 2: Since all mobility configurations related to previous source PCell are released with connection establishment in the new target PCell, the above condition of “UE is configured with CHO with candidate SCGs” is not met and, therefore, in case of RLF shortly after handover when UE is still clean no choWithCandidateSCGInfoList is added to the RLF-report.

Proposal 1: RAN2 confirms that the current specification fails in logging the information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.

Therefore, those failures which could be, for instance, interpreted as a “CwcS execution to wrong PCell” if UE was reestablishing to a third PCell, cannot be properly analyzed without the information of the chronological fulfilment of the two execution conditions with the new choWithCandidateSCGInfoList being in the RLF report.

Observation 3: Without the information of choWithCandidateSCGInfoList a proper MRO for failure cases where RLF occurs shortly after successful CHO with candidate SCGs completing (case 8a from the agreed RAN3 failure sceanrios) is not possible.

The information about the chronological fulfilment of the two execution conditions of the preceding CwcS is only known by the UE. Typically, it might be logged in an UE internal memory during the monitoring period which starts with the conditional reconfiguration and ends with an event which is either the RLF or the handover execution. In case of RLF, the information is dumped to the IE choWithCandidateSCGInfoList which becomes part of RLF-report provided the UE is configured with CwcS.

Observation 4: UE typically holds the information about the chronological fulfilment of the two execution conditions in an internal memory. I.e., it might available for RLF shortly after.

UE also logs the time information timeConnFailure which is used to identify later on network side by the corresponding MRO instance “too early HO” or “HO to wrong cell”. But to identify that the occurred RLF is strongly related to the preceding PCell change, which would be one of the needed criteria to add the choWithCandidateSCGInfoList with information about the chronological fulfilment of the two execution conditions of the preceding CwcS to the RLF, the needs to analyze the timeConnFailure value.

Observation 5: To justify that the RLF is related to the preceding successful CHO with candidate SCGs and the stored information about the chronological fulfilment of the two execution conditions is to be added to the RLF-report, UE would need to check the value of timeConnFailure against a dedicated threshold.

This can be accomplished with following two amendments:

- UE has to be configured with new time threshold criterion (e.g. named TEH_HWC_Thres or t-Check-TEH-HWC) which is to be used to check the time being set in the field timeConnFailure.

This new UE parameter might be listed as SON-parameter parameter of the UE capabilities of TS 38.331, and it will be configured as part of otherConfig.

- Amendment of the specification with respect to RLF report content determination which ensures that the UE will check the following three conditions

    - lastHO-Type is set to choWithCandidateSCG, and

    - timeConnFailure is below TEH_HWC_Thres, and

    - UE is not newly configured yet with CwcS when RLF occurred.

This check can be realized by inserting an else-branch to the if-clause where conditional configuration is checked when not fulfilled, and then to continue checking i) and ii). If fulfilled, the information about the chronological fulfilment of the two execution conditions of the last executed CwcS still being stored in UE is included in the RLF report as part of IE choWithCandidateSCGInfoList.

Since the RLF report structure will not be changed, there is no impact on protocol and architecture. Only the MRO instance exploring the RLF report receives more information.

Proposal 2: RAN2 agrees that

- The UE can be configured with new time threshold criterion, which is used to check the time being set in the field timeConnFailure.

- A new branch is added to RLF content determination clause to add the missing information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.

# 3	Conclusion

This document has made the following observations, and proposals for RLF shortly after successful completion of CHO with candidate SCGs:

Observation 1: Logging of the chronological fulfilment of the two execution conditions of CHO with candidate SCGs is only carried out when UE is configured with CHO with candidate SCGs, i.e., it includes condReconfigIds including both condExecutionCond and condExecutionCondPSCell.

Observation 2: Since all mobility configurations related to previous source PCell are released with connection establishment in the new target PCell, the above condition of “UE is configured with CHO with candidate SCGs” is not met and, therefore, in case of RLF shortly after handover when UE is still clean no choWithCandidateSCGInfoList is added to the RLF-report.

Proposal 1: RAN2 confirms that the current specification fails in logging the information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.

Observation 3: Without the information of choWithCandidateSCGInfoList a proper MRO for failure cases where RLF occurs shortly after successful CHO with candidate SCGs completing (case 8a from the agreed RAN3 failure sceanrios) is not possible.

Observation 4: UE typically holds the information about the chronological fulfilment of the two execution conditions in an internal memory. I.e., it might available for RLF shortly after.

Observation 5: To justify that the RLF is related to the preceding successful CHO with candidate SCGs and the stored information about the chronological fulfilment of the two execution conditions is to be added to the RLF-report, UE would need to check the value of timeConnFailure against a dedicated threshold.

Proposal 2: RAN2 agrees that

- The UE can be configured with new time threshold criterion, which is used to check the time being set in the field timeConnFailure.

- A new branch is added to RLF content determination clause to add the missing information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCGs.