3GPP TSG-RAN WG2 #131bis	R2-2507426

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	8.2.2

Source:	Ericsson

Title:	Open issues for TS 38.391

Document for:	Discussion/Decision

# 1	Introduction

We focus on the open issues for specification 38.391 in this paper.

| Issue 1-3: Paging ID length field | The field to indicate the paging ID length, e.g., how many bits, taking into account of CT4 and SA2 inputs. - Relevant agreements: - 8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion. - However, in Aug meeting, SA2 sent a new LS to RAN2 in S2-2507793 on AIoT Device Permanent ID Length, indicating SA2 identified requirements for longer device ID, and the rapporteur understand this will impact the length of paging ID as well, e.g., 600 bits are needed instead of 256 bits. Therefore, in Oct meeting, RAN2 needs to discuss how to reply the SA2 LS, and how to handle the impact to paging ID length field. - Status in running CR: the field name is captured in 6.2.1.1 with the detailed format. |
| --- | --- |
| Issue 1-7: Security parameter | How to include the security parameters in Paging message. - Based on SA3 LS in S3-252392, it’s understood that security is a mandatory feature to be supported by the Rel-19 devices. But companies also raised question during online discussion whether the security parameters can be optional, for instance the feature can be activated/controlled by network. And companies also mentioned that the signaling overhead of the current paging message design is an important aspect from RAN2 point of view. Then how to capture the security parameter has been postponed and expected more discussion in Oct meeting. In this case, this issue is classified as “to be discussed by contribution”, and the rapporteur would like to suggest companies also to consider more from RAN2 perspective when designing signaling in their contributions, in terms of e.g., signaling compatibility, flexibility, signaling overhead, and scalability. - SA3 provided more information about the security design in LS S3-252933 in Aug meeting, which should be take into account. - Relevant agreements: - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - RAN2 will wait for SA3 conclusions in October on whether the “128bit random number in the paging request message” is always required or not. - Reply to SA3 (The LS is approved in R2-2506465) - RAN2 thinks it is feasible from a signalling perspective to add the 128 bits. However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible. - Indicate to SA3 that RAN2 tries to minimize number of bits required. Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG. - Indicate space pressure from all the WG - Status in running CR: not captured yet |
| Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - Relevant agreements: - The reader determines no data available case by SDU length 0. As more data indication is mandatory, the device sets this bit to "0". - The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device. - The follow-up R2D Upper Layer Data Transfer message includes the Received Data Size field with the Received Data Size field set to value 0, without including the original command. - RAN2 would like to check if there is a case where NAS doesn’t provide a response at all. If this case exists, RAN2 will discuss this issue. - Regarding the above check point, it can be up to companies’ contribution. - Status in running CR: captured. |

# 2	Discussion

## 2.1 SA2 LS on AIoT device permanent ID length

Regarding the LS on AIoT device permanent ID length [2], we would like to highlight that the total MAC PDU size for the paging message includes other control information than only paging ID itself such as security parameters and scheduling information. As estimated in the SA2 LS, filtering information will be around 600 bits in case the Identification Information is 496-bit, and around 360 bits in case the Identification Information is 256-bit. If we take into account at least 128-bit security parameter for authentication, 24-bit scheduling information, and around 30 bits for other AS level control information, the TBS for the paging message would be around 550 bits for the case of 256-bit identification information, and 780 bits for the case of 496-bit identification information.

We think it is feasible to extend the paging ID to cover both 256-bit and 496-bit value. However, considering the mindset of minimizing the number of bits required in over the A-IoT air interface, we think RAN2 can consider the extension of paging ID to only 256-bit value.

in addition, RAN2 had similar discussions on the SA3 LS about the additional security parameter in paging, and made below agreements at RAN2#130

- -     RAN2 thinks it is feasible from a signalling perspective to add the 128 bits.  However, from RAN2 perspective the less overhead the better, so SA3 should avoid adding additional parameters if possible.

- -     Indicate to SA3 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

-	Indicate space pressure from all the WG

We think RAN2 can make similar agreements in the following

- RAN2 thinks it is feasible from a signalling perspective to extend the AIoT device permanent ID to 256-bit.  However, from RAN2 perspective the less overhead the better, so SA2 should avoid adding additional parameters if possible.

- -     Indicate to SA2 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

.

- TBS for the paging message is around 550 bits and 780 bits for the case of 256-bit and 496-bit identification, respectively.

Reply to SA2 LS on permanent ID length that

        - RAN2 thinks it is feasible from a signalling perspective to extend the AIoT device permanent ID to 256-bit.  However, from RAN2 perspective the less overhead the better, so SA2 should avoid adding additional parameters if possible.

        - Indicate to SA2 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

## 2.2 Paging ID length field (Issue 1-3)

Regarding issue 1-3, paging ID length field in the list of A-IoT MAC open issues [3], there are some aspects to clarify:

- First, 8-bit paging ID length field is not currently not sufficient to indicate the actual length of device identification in some cases.

  - SA3 already agreed that for individual paging, permanent device ID can be used if privacy is not used. Irrespective of whether the security parameter is part of paging ID or not (discussed in next section), the permanent device ID itself can be up to 256 bits, as discussed above.

  - As indicated in the SA2 LS [2], the filtering information would be 360 bits for the case of 256-bit device identification information (pending SA2/CT4 discussion).

  - Other NAS control information such as security parameter, type of device ID may be part of the paging ID field (or a paging NAS container). In this case the length field should take into account not only the identification information/filtering information, but also the other NAS control information.

  - In addition, RAN2 has assumed that upper layer SDU is byte-aligned. The current A-AoT NAS specification has octet-aligned information elements. Therefore, we assume that paging ID will be byte aligned.

  - It is therefore proposed to change the unit of paging ID length field to byte instead of units.

- Second, assuming the byte unit, 8-bit paging ID length field can indicate up to 256 bytes (2048 bits) of paging ID, which already exceeds the 1000 bits maximum TBS in Rel-19, therefore we think 7 bits are sufficient for paging ID length field. This applies for both CFRA and CBRA paging messages.

- 8-bit paging ID length field is not sufficient with longer permanent device ID.

- 8-bit paging ID length field is not sufficient if other NAS control information, e.g., security parameter, paging ID type, is included in current “paging ID” field.

Paging ID length field indicates length of paging ID in unit of bytes (instead of unit of bits).

- 8-bit paging ID length field can indicate up to 2048 bits paging ID, which is way above 1000-bit maximum TBS.

Paging ID length field is 7 bits.

## 2.3 Security parameters in paging message (Issue 1-7)

When it comes to issue 1-7, security parameters in paging message, we understand from SA3 work that no additional security information than RAND_aiot_n is to be included in the paging. In addition, this security parameter is used for authentication procedure, which is transparent at A-IoT AS sublayer. There are two options to implement this in specifications. We prefer option2.

- Option 1: Security parameter is an AS field in paging message. This is flexible in case Paging ID itself is updated, e.g., including more than one IDs in the future release. But AS does not need to know/understand security information, i.e., security ID is transparent to reader and device MAC sublayers.

- Option 2: Security parameter is a NAS information element. This means the current paging ID field carries the security parameter in addition to the paging identifier. To avoid possible confusion, the field name of paging ID can be updated to paging NAS container. Similarly, the field name of paging ID length can be updated to paging NAS container length. RAN has assumed NAS SDU is byte-aligned. According to current A-IoT NAS specification, NAS information elements are also byte-aligned. RAN2 can assume the NAS container is byte-aligned.

RAN2 assume security parameter is as part of NAS container, rather than an AS information element.

Rename paging ID field name as paging NAS container and paging ID length field as paging NAS container length.

RAN2 assume paging NAS container are octet-aligned

Paging ID presence indication (PIPI) currently indicates the presence of both paging ID and paging ID length. However, as the paging ID field always includes the mandatory security parameter, it is always present. Similarly, the paging ID length is always non-zero (i.e., at least 128 bits). Therefore, we think there is no need of keeping the PIPI field. In addition, as PIPI field is not needed, upper layers rather than MAC sublayer can determine if device is selected and indicate to MAC sublayer for initiation of access procedure for all cases including page-all. The paging message format and text in MAC TS 38.391 should be updated accordingly (see also text proposal in section 5).

Remove PIPI field in paging message for CBRA.

Upper layers (rather than A-IoT MAC) always determine if device is selected and indicate to MAC sublayer for initiation of access procedure.

## 2.4 More data indication (issue 3-7)

For the issue 3-7, more data indication, we understand that A-IoT NAS specification specifies handling for unknown/unforeseen/erroneous cases in which the device shall ignore the received NAS message. As in legacy 5G NR, we think there is no need to handle such error cases at MAC sublayer.

- According to TS24.391, error cases are already handled at NAS sublayer.

RAN2 assumes no need to handle error handling cases at MAC sublayer.

Update A-IoT MAC specification with text changes and paging message formats proposed in section 5.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	TBS for the paging message is around 550 bits and 780 bits for the case of 256-bit and 496-bit identification, respectively.

Observation 2	8-bit paging ID length field is not sufficient with longer permanent device ID.

Observation 3	8-bit paging ID length field is not sufficient if other NAS control information, e.g., security parameter, paging ID type, is included in current “paging ID” field.

Observation 4	8-bit paging ID length field can indicate up to 2048 bits paging ID, which is way above 1000-bit maximum TBS.

Observation 5	According to TS24.391, error cases are already handled at NAS sublayer.

Proposal 1	Reply to SA2 LS on permanent ID length that

a.	RAN2 thinks it is feasible from a signalling perspective to extend the AIoT device permanent ID to 256-bit.  However, from RAN2 perspective the less overhead the better, so SA2 should avoid adding additional parameters if possible.

b.	Indicate to SA2 that RAN2 tries to minimize number of bits required.   Have a maximum size of 1000bits, and whatever they include has to fit in the 1000bits considering bits from all TSG.

Proposal 2	Paging ID length field indicates length of paging ID in unit of bytes (instead of unit of bits).

Proposal 3	Paging ID length field is 7 bits.

Proposal 4	RAN2 assume security parameter is as part of NAS container, rather than an AS information element.

Proposal 5	Rename paging ID field name as paging NAS container and paging ID length field as paging NAS container length.

Proposal 6	RAN2 assume paging NAS container are octet-aligned

Proposal 7	Remove PIPI field in paging message for CBRA.

Proposal 8	Upper layers (rather than A-IoT MAC) always determine if device is selected and indicate to MAC sublayer for initiation of access procedure.

Proposal 9	RAN2 assumes no need to handle error handling cases at MAC sublayer.

Proposal 10	Update A-IoT MAC specification with text changes and paging message formats proposed in section 5.

# 4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

S2-2507793, “LS on AIoT Device Permanent ID Length”, SA2-170, Goteborg, Sweden August 25-29, 2025

R2-250xxxx, “Summary of A-IoT MAC open issues (outcome of [POST131][021][AIoT] MAC Running CR)”, RAN2-131bis, Prague, CZ, Oct 13-17, 2025.

TS23.369 A-IoT architecture support.

TS24.369 A-IoT NAS specifications.

TS33.369 A-IoT security aspects.

TS38.391 A-IoT MAC specifications.

# 5	Text proposals

The below text proposals are based on the Rel-19 spec V19.0.0.

## 5.2	A-IoT paging

The purpose of this procedure is to transmit A-IoT Paging message to one or more devices. The reader includes the Paging NAS container field to select a specific device, a group of devices or all devices..

The device always monitors for the A-IoT Paging message, and initiates the access procedure upon indication from upper layers that device is selected.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall:

1>	if the Access Type field in the A-IoT Paging message indicates CBRA:

2>	if the device has no stored Transaction ID; or

2>	if the value of the Transaction ID field is different from the stored Transaction ID; or

2>	if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5:

3>	release the stored AS ID, if any;

3>	store the received value in Transaction ID field, if the device has no stored Transaction ID, or replace the previously stored Transaction ID with the current received value, if the value of the Transaction ID field is different from the stored Transaction ID;

3>	forward the content of the Paging NAS container field to the upper layers;

3>	if the upper layers indicate that the device is selected:

4>	initiate Contention-Based Random Access procedure as specified in clause 5.3.1;

1>	else (i.e., the Access Type field in the A-IoT Paging message indicates CFA):

2>	release the stored AS ID, if any;

2>	release the stored Transaction ID, if any;

2>	forward the content of the Paging NAS container field to the upper layers;

2>	if the upper layers indicate that this device is selected:

3>	initiate Contention-Free Access procedure as specified in clause 5.3.2.

#### 6.2.1.1	A-IoT Paging message

Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message.

The fields in this message are defined as follows:

-	R2D Message Type: This field indicates the message type. See the Table 6.1-1. The length of the field is 3 bits.

-	R2D TBS: This field indicates the TBS of this message. The value can be {1, 2, …, 124, 125} byte(s). The length of the field is 7 bits.

-	Access Type (AT): This field indicates CBRA (when set to 1) or CFA (when set to 0). The length of the field is 1 bit.

For CBRA, the following fields are further included:

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is 6 bits.

-	Paging NAS Container Length: This field indicates the length of the Paging NAS Container field in unit of bytes. If present, the length of the field is 7 bits.

-	Paging NAS Container: If present, this field contains  upper layer information used for determination if the device is selected and security information (as defined in TS 23.369 [4], TS 23.003 [5] , TS 33.369[x], and TS 24.369[y]). This field is of variable length.

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 18 bits.

-	K: This field indicates that the value K is 1 (when set to 0) or 4 (when set to 1) used for determining monitor window for Random ID Response message. The length of the field is 1 bit.

-	Fill Bits: This field is of variable size and is optionally present. It can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

For CFA, the following fields are further included:

-	Paging NAS Container Length: This field indicates the length of the Paging NAS Container field in unit of bytes. The length of the field is 7 bits.

-	Paging bytes: This field contains upper layer security information (as defined in TS 23.369[4], TS 33.369[x], and TS 24.369[y]). This field is of fixed length.

-	D2R Scheduling Info: This field contains the physical layer parameters used for D2R transmission. The child fields are defined in clause 6.2.1.6. The length of the field is 24 bits.

-	Fill Bits: This field is of variable size, and can be used to pad for byte alignment (1-7 bits) and/or contain future extensions. In this release, the MAC entity shall ignore the values of this field.

Figure 6.2.1.1-1: MAC PDU of A-IoT Paging message indicating CBRA

Figure 6.2.1.1-2: MAC PDU of A-IoT Paging message indicating CFA