3GPP TSG RAN WG2 #131bis	R2-2507445

Prague, Czech Republic, October 13th-17th 2025

# Title:	[Draft] Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC

# Response to:	R2-2506747/ S2-2507636

# Release:	Release 20

# Work Item:	FS_5GSAT_Ph4_ARC

Source:	RAN WG2

To:	SA WG2

Cc:	RAN WG1, SA WG4, SA WG1, CT WG1

Contact Person:

Name:	Bharat Shrestha

E-mail Address:	bshresth AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

RAN2 would like to thank SA2 for the LS. RAN2 would also like to provide the following response.

Question 1 (To RAN2): Does RAN2 have any observation on how many consecutive packets lost or erroneously decompressed will trigger the RoHC state fall back at the compressor when using RoHC?

RAN2 answer: RAN2 assumes the RoHC protocol is outside of 3GPP scope and RoHC state change depends on use of different operating modes. RAN2 assumes the use of RoHC modes is up to implementation and is not mandated by 3GPP specification. For example, as specified in RFC 3095, the decisions about RoHC state fall back are taken by the compressor on the basis of:

- variations in packet headers

- negative feedback from decompressor (Negative ACKs -- NACKs)

- periodic timeouts (when operating in unidirectional mode, i.e., over simplex channels or when feedback is not enabled)

In Bidirectional Optimistic mode (O mode), RoHC header type (i.e., UO-0 header type) can carry only 4 bits LSB of RTP SN and 3 bits CRC SN. When more than 16 consecutive packets are lost between compressor and decompressor, this may cause the LSBs of the RTP SN in compressed packets to be interpreted wrongly, because the decompressor has not moved the interpretation interval for lack of input (e.g., a kind of context damage). This can lead to CRC check failure (i.e., decompression failure). As per section 5.3.2.2.3 in RFC 3095, when the CRC check of a decompressed packet fails, context damage may be assumed and a NACK is sent. This will trigger the RoHC state fall back at compressor.

Due to GEO channel condition, local clock error (in case of time-based decompression) and wraparound issue of compressed RTP SN and/or compressed RTP timestamp, decompression failure can occur for any packet arriving at the decompressor. This can result in RoHC NACK feedback and ROHC state fall back at compressor.

RAN2 expects RAN1 to provide more detailed feedback on these issues based on Question 2.

Question 3 (To RAN2): Can RAN2 confirm whether scheduling methods for support of IMS voice over NB-IoT NTN can handle the voice packets of different sizes changing dynamically?

RAN2 answer: Due to nature of very long RTT over GEO channel, UL scheduling based on scheduling request or buffer status report would be very inefficient. This incurs large scheduling delays and older voice packets may need to be discarded as they become obsolete. Retransmission may be disabled over GEO channel. Therefore, NB-IoT over GEO channel cannot handle the voice packets of different sizes changing dynamically without significant performance degradation of voice call, e.g., loss of voice packets.

RAN2 intends to define semi-persistent scheduling that do not rely on buffer status reports assuming a fixed packet size.

Question 6 (To RAN2): Is it feasible to support more than 2 DRBs for a UE accessing NB-IoT in Rel-20?

RAN2 answer: Yes, it is feasible to extend DRB-ToAddModList-NB to add more than 2 DRBs if needed.

Question 9 (To RAN2): Can RAN2 provide feedback on whether it is possible to define a new SRB that will be used to carry voice media?

RAN2 answer: Yes, it is feasible to extend SRB-ToAddModList-NB to add a new SRB (e.g., SRB2 or SRB2bis) which is dedicated to carry voice traffic using ULInformationTransfer-NB RRC message.

Question 10 (To RAN2): If the answer to the above question is yes, can RAN2 confirm whether such SRB(s) are defined, whether it is technically feasible to support and configure RLC UM for such SRB(s) for example: in NB-IoT deployments over GEO satellite, when SRBs are used to carry voice media?

RAN2 answer: Yes. In addition to SRB1, SRB1bis was defined for RRC messages to carry NAS messages prior to the activation of AS security, i.e., bypassing PDCP layer. RLC AM is specified as the only applicable RLC mode for SRB1 and SRB1bis. In a similar way, it is possible to define a new SRB, um-Uni-Directional-UL-r15 i.e., RLC UM mode can be configured explicitly or specified as a default configuration implicitly.

# 2. Actions:

# To SA4:

ACTION:

RAN2 kindly asks SA4 to take the above response into account.

# 3. Dates of Next RAN2 Meetings:

RAN2#132						17th to 21st November 2025		Dallas

RAN2#132						9th to 13th February 2026		Gothenburg