3GPP TSG-RAN WG2 Meeting #131bis	R2-2507499

Prague, Czech Republic, Oct. 13th-17th, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	RIL S001, H403

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss RIL S001 and H403.

# Discussion

## S001

As mentioned in RIL S001, S002, S003, S006,

- cri-TypeI-SinglePanelRI-Restriction-r19 and cri-TypeI-SinglePanelN1-N2-CBSR-r19 are applied to Rel-15 codebook type “typeI-SinglePanel”;

- cri-TypeII-RI-Restriction-r19 and cri-TypeII-N1-N2-CBSR-r19 are applied Rel-16 codebook type 'typeII-r16'.

So these four parameters should be moved out of codebookType of codebookConfig-r19. The updated signalling structure to solve S001-004 is as follows.

CodebookConfig-r19  ::=               SEQUENCE  {

codebookType                          CHOICE {/***********omit the unrelted part**********/} OPTIONAL,   -- Need R

cri-TypeI-SinglePanelRI-Restriction-r19   SEQUENCE (SIZE (1..8)) OF BIT STRING (SIZE (8))		    OPTIONAL,    -- Need R

cri-TypeI-SinglePanelN1-N2-CBSR-r19        CRI-TypeI-SinglePanelN1-N2-CBSR-List-r19               OPTIONAL,     -- Need R

cri-TypeII-RI-Restriction-r19             SEQUENCE (SIZE (1..4)) OF BIT STRING (SIZE (4))         OPTIONAL,    -- Need R

cri-TypeII-N1-N2-CBSR-r19                 CRI-TypeII-N1-N2-CBSR-List-r19                          OPTIONAL     -- Need R

}

Using this structure, we also need to clarify that these four parameters are only configured when the respective Rel-15/16 codebook type is configured. They are not configured together with codebookType in codebookConfig-r19.

Proposal 1: For S001, add in field description that cri-TypeI-SinglePanelRI-Restriction-r19 and cri-TypeI-SinglePanelN1-N2-CBSR-r19 are optionally present if codebookType is set to typeI-SinglePanel, cri-TypeII-RI-Restriction-r19 and cri-TypeII-N1-N2-CBSR-r19 are optionally present if codebookType is set to typeII-r16. They are not configured together with codebookType in codebookConfig-r19.

## H403

In RIL H403, it is proposed to support pathloss offset for LTM cell switch by adding pathlossOffset in candidate TCI states of LTM candidate cells.

Note the pathloss offset for asymmetric mTRP is only supported for intra-band intra-DU non-co-located mTRP scenarios as the Rel-19 MIMO scope defined [1]. The pathloss offset for LTM cell switch should be limited to the same scope, which makes the applicable scenario very limited, thus this enhancement becomes less useful.

Even for this considered scenario, to provide pathloss offset in the LTM candidate TCI states, NW needs to measure the pathloss offset between the two TRPs in advance. However, this may not be possible before UE switching to the target cell since measurement based on SRS is needed to determine the pathloss offset for UL.

On the other hand, nothing is broken even if pathloss offset is not applied during the LTM cell switch. The asymmetric mTRP targets UL throughput improvement [1], where a UE may receive DL transmission from the macro normal TRP, but transmit UL to either the macro normal TRP or non-co-located micro UL-only TRP in order to maximize UL throughput. During LTM cell switch, the first UL transmission still can be received by the macro normal TRP without applying the pathloss offset. UL throughput is not the target of the first UL transmission during LTM cell switch. After the LTM completion, UE can apply pathloss offset if it is configured for TCI states by RRCReconfiguration, which can also be contained in RRCReconfiguration of the LTM-Candidate.

Based on these reasons, we don’t think it is motivated to support pathloss offset for LTM cell switch by adding pathlossOffset in candidate TCI states of LTM candidate cells.

Proposal 2: Not pursue H403, i.e., pathloss offset is not supported for LTM candidate TCI states.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: For S001, add in field description that cri-TypeI-SinglePanelRI-Restriction-r19 and cri-TypeI-SinglePanelN1-N2-CBSR-r19 are optionally present if codebookType is set to typeI-SinglePanel, cri-TypeII-RI-Restriction-r19 and cri-TypeII-N1-N2-CBSR-r19 are optionally present if codebookType is set to typeII-r16. They are not configured together with codebookType in codebookConfig-r19.

Proposal 2: Not pursue H403, i.e., pathloss offset is not supported for LTM candidate TCI states.

# References

- RP‑242394	WID revision: NR MIMO Phase 5.

- R2-250xxxx RIL review summary for MIMO