3GPP TSG-RAN WG2#131-bis		      					R2-2507500

Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	10.4

Source: 	Kyocera

Title: 	Consideration of mobility for 6G study

Document for:	Discussion

Introduction

RAN#108 approved the new study item on 6G Radio [1]. From the comprehensive objectives of this study, RAN2-related topics are picked up as follows.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - […] - Initial access [RAN1, RAN2, RAN4] - […] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] […] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - […] - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles […] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - […] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - […] - higher layer procedures and protocol aspects [RAN2] […]

In this contribution, the consideration on mobility for 6G study is provided.

Discussion

General Connected mode mobility principles

In 5G connected mode mobility, various types of handover schemes are specified, such as L3 HO, CHO, DAPS handover, LTM and CLTM [3]. In Rel-15, the traditional handover method, i.e., L3 HO (L3 Handover) based on L3 measurements and L3 Handover Command, was supported. Rel-16 introduced CHO (Conditional Handover) to enhance C-plane robustness, and DAPS (Dual Active Protocol Stuck) handover to achieve 0ms U-plane interruption time. In Rel-18, LTM (L1/L2 Triggered Mobility) was specified to enable high-speed handover based on L1/L2 signalling. In Rel-19, CLTM (Conditional LTM) was supported to further improve the C-plane robustness than LTM.

Since 5G connected mode mobility has been enhanced in each release, different procedures and different IEs in RRC message for similar purposes [5] have been adopted. It led to increased complexity on mobility procedures, which is regarded as a problem. Since the system simplification is required in the 6G RAN SID [1], the handover scheme in 6G should be achieved through a single procedure using a simple configuration (e.g., common RRC message IEs) as much as possible.

RAN2 should study the feasibility of achieving handover in 6G connected mode mobility through a single procedure using a simple configuration.

L3 HO has been the main approach in UMTS, LTE and NR; thus, it has a proven track record in commercial networks. Nevertheless, the standardization activity of CHO, DAPS, LTM and CLTM indicates that there are performance issues with L3 HO. The potential performance issues of handover procedure identified in NR should be resolved on 6G Day 1: so, based on L3 HO as the baseline, the pros and cons of each 5G connected mode mobility scheme should be analysed as follows.

Note: According to IMT-2030 TPR agreed in RAN#109 [2], the requirement for HO interruption time is no longer 0ms, so the analysis of DAPS HO is skipped in this paper.

L3 Handover (Baseline)

Low UE complexity

  - HO command (RRC Reconfiguration with sync) includes the entire RRC configuration of target cell [5]. The UE just applies the received configuration and starts accessing the target cell immediately, so UE’s storage memory is minimized for maintaining the pre-configurations of candidate cells.

Low NW complexity

  - The dedicated resources (e.g., radio resources and NW resources) are reserved after the admission control by the target cell), and the UE attempts to access the target cell immediately. So, the target cell is required to reserve the dedicated radio resources for a short period.

  - CU always decides to handover as the decision is only based on L3 measurements .

Potential problem on C-plane robustness

  - Upon receiving the measurement report from the UE, the NW initiates the HO preparation phase (Handover Request / Acknowledge) over the network interface (i.e., NG or Xn) [3]. As a result, there is a delay before the HO command is sent to the UE, which is caused by the backhaul latency.

  - There is a delay between when the UE receives HO command and when it access the target cell. This delay is caused by the time required for applying RRC Reconfiguration, DL synchronization and UL synchronization (i.e., RACH procedure) towards the target cell.

  - RLF may occur as a result of rapid degradation of the radio conditions at the UE before HO command is sent to the UE, or before the UE accesses the target cell, i.e., Too late HO.

  - The fast recovery is not supported, i.e., RRC Reestablishment-based recovery [3].

Potential problem on U-plane interruption time

  - The UE shall perform the RACH procedure when accessing the target cell, which causes a delay, as the RACH-less HO is not supported.

Conditional Handover

Enhancements for C-plane robustness: Conditional reconfiguration by UE

  - No signaling from the NW for triggering the handover execution is required, as the UE evaluates execution conditions and makes the handover decision [3]. It significantly improves the handover failure rate, especially caused by Too late HO.

  - The pre-configuration, i.e., with conditionalReconfiguration IE, enables the RRC pre-processing for fast RRC configuration application, and the fast recovery for avoiding RRC reestablishment and allowing continued handover execution.

Enhancements for U-plane interruption time: RACH-less handover (for specific use cases)

  - Limited to Mobile IAB and NTN, CHO supports the RACH-less HO by the pre-configured TA value, TA=0 or TA=source [3].

Moderate UE complexity: Pre-configuration and execution condition evaluation by UE

  - All stored pre-configurations of candidate cells are deleted upon each handover execution.

  - More storage memory is required to store the pre-configured RRC Reconfigurations of candidate cells.

Drawbacks in NW complexity

  - The source cell may pre-configure multiple candidate cells, and each candidate cell needs to prepare and reserve the dedicated radio resources for the UE, upon sending Handover Request Acknowledge. In addition, it’s necessary for the source cell to decide which candidate cells to configure to the UE ahead of time.

  - The UE evaluates the execution conditions for each candidate cell after receiving the pre-configuration (i.e., conditionalReconfiguration), which increases the time to access the target cell compared with L3 HO. So, the candidate cells shall reserve the radio resources for a longer time.

  - The candidate cells may waste the reserved radio resources, as the UE accesses one of the candidate cells (i.e., the target cell) at the end.

LTM (L1/L2 Triggered Mobility)

Enhancements for C-plane robustness: L1/L2-based cell switch decision by NW

  - The cell switch can be determined by L1 measurement report and indicated via L2 MAC CE, enabling better adaptation to the radio condition changes [3][4].

  - The support for RACH-less LTM is beneficial to prevent RLF since it avoids a rapid degradation of the radio quality in the RACH procedure for the cell switch.

  - Similar to CHO, the pre-configuration enables the RRC pre-processing and the fast recovery.

Enhancements for U-plane interruption time: RACH-less cell switch with Early sync

  - The pre-configuration and Early sync enable the DL/UL synchronization with candidate cells in advance of the cell switch execution, which allows the RACH-less LTM for all use cases (i.e., not limited to mIAB and NTN) [3].

Enhancements for signalling overhead: Subsequent LTM

  - The UE continues to store the pre-configuration even after LTM execution, which is used for the subsequent LTM executions towards the candidate cells [3]. So, the new serving cell does not need to re-send the configuration of candidate cells, to the UE.

Drawbacks in UE complexity

  - The UE-based TA measurement for the RACH-less LTM, which is up to UE implementation, may increase the UE complexity [3].

  - Similar to CHO, more storage memory is required to store the pre-configurations of candidate cells.

Drawbacks in NW complexity

  - Similar to CHO, the unnecessary radio resource reservation occurs. For the subsequent LTM, i.e., after a cell switch is executed once, the candidate cells (i.e., the cells except for the target cell) may further extend the time period for radio resource reservation beyond that of CHO .

  - The cell switch decisions are mainly based on L1 measurements, optionally on L3 measurements [6]. Since both the DU and the CU can make the cell switch decision, the CU-DU coordination is required.

  - The PDCCH-ordered early sync for the RACH-less LTM, requires that the NW manage the validity of TA values for each candidate cell.

Conditional LTM (CLTM)

Note that CLTM is very similar to LTM. Below are the key similarities and differences between the two.

Enhancements for C-plane robustness: Conditional cell switch by UE

  - Similar to CHO, the UE evaluates the execution conditions and makes the decision to execute LTM, so no network signaling for LTM execution is required [7]. It offers a higher C-plane robustness than LTM.

Enhancements for U-plane interruption time: RACH-less cell switch with Early sync

  - Same as LTM.

Enhancements for signalling overhead: Subsequent CLTM

  - Same as LTM.

Drawbacks in UE complexity

  - In case of PDCCH-ordered early sync for RACH-less CLTM, the UE needs to manage the validity of TA values on each candidate cell (i.e., by the CLTM TAT) [4].

Drawbacks in NW complexity

  - The CLTM TAT should be configured to UE to control the validity of TA value.

The following table provides a summary of the advantages and disadvantages of 5G connected mode mobility schemes, namely L3 HO, CHO, LTM and CLTM.

Table 1 Summary of 5G connected mode mobility pros and cons

|  |  | L3 Handover | CHO | LTM | CLTM |
| --- | --- | --- | --- | --- | --- |
| Characteristics | C-plane robustness | Baseline | Very good | Good | Very good |
|  | U-plane interruption time | Baseline | Baseline | Good | Good |
|  | UE complexity | Baseline | Moderate | Complex | Complex |
|  | NW complexity | Baseline | Complex | Complex | Complex |
|  | Radio resource waste/reservation | Baseline | Inefficient | Inefficient | Inefficient |
|  | Signalling overhead | Baseline | Inefficient | Good | Good |
| Features | Pre-configuration | Not supported | Supported | Supported | Supported |
|  | Conditional mobility | Not supported | Supported | Not supported | Supported |
|  | Subsequent mobility | Not supported | Not supported | Supported | Supported |
|  | RACH-less mobility | Not supported | Limited support | Supported | Supported |
|  | Fast recovery | Not supported | Supported | Supported | Supported |
|  | L3 measurement based cell switch decision | Supported | Supported | Supported | Supported |
|  | L1 measurement based cell switch decision | Not supported | Not supported | Supported | Supported |

As shown in Table 1, the traditional L3 HO, as the baseline, offers lower complexity for both the UE and the NW, and the handover procedure is simple. However, it has issues with respect to C-plane robustness (i.e., HOF including too early HO and too late HO) and U-plane interruption time.

CHO, LTM and CLTM improve C-plane robustness and U-plane interruption time compared to L3 HO. However, the trade-off is observed between these performance improvements and the UE and the NW complexities/radio resource efficiency.

CHO, LTM and CLTM improve C-plane robustness and reduce U-plane interruption time; however, they also increase UE and NW complexity and cause radio resource waste/reservation.

RAN2 should study 6G connected mode mobility with a focus on balancing C-plane robustness and U-plane interruption time, as well as UE/NW complexity and radio resource waste/reservation.

The additional features introduced on top of L3 HO in 5G include Pre-configuration, Conditional mobility, RACH-less mobility (PDCCH-ordered early sync, UE-based TA measurement, TA=0 or TA=source), Subsequent mobility, Fast recovery, and L1 measurement-based cell switch decision. Some of these additional features have dependencies, so it’s a little bit difficult to analyse each feature independently in some cases but it’s worth trying.

The positive/negative impacts of each additional feature on C-plane robustness, U-plane interruption time, UE/NW complexity, radio resource waste/reservation and signalling overhead are shown below:

Table 2 Impact of additional features on handover characteristics

|  |  | Features |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Pre-configuration | Conditional mobility | RACH-less mobility PDCCH-ordered early sync | RACH-less mobility UE-based TA measurement | RACH-less mobility TA = 0 or TA = source | Subsequent mobility | Fast recovery | L3 measurement based cell switch decision | L1 measurement based cell switch decision |
| Characteristics | C-plane robustness |  |  |  |  |  |  |  | Depends | Depends |
|  | U-plane interruption time |  | – |  |  |  | – | – | – | – |
|  | UE complexity |  | – |  |  | – | – | – | – | – |
|  | NW complexity |  |  |  | – | – |  | – | Depends | Depends |
|  | Radio resource waste/reservation |  | – |  |  |  |  | – | – | – |
|  | Signalling overhead |  | – | – | – | – |  | – | – | – |

The performance of cell switch decisions based on L1 or L3 measurements varies depending on the particular situation. In terms of C-plane robustness, L3 measurement, which derives the cell-level quality from multiple beams, can be considered more stable and reliable [3]. However, when radio conditions change rapidly, cell switch decision based on L3 measurement may be too late.

On the other hand, L1 measurement which derives beam quality at the physical layer [3], may be considered as less reliable for cell switch compared with L3 measurement, but it offers higher robustness in terms of its ability to follow rapid changes in radio conditions.

If only one of L1 or L3 measurements based cell switch decision is supported, there is no issue in terms of the NW complexity. However, if both are supported, coordination between CU and DU is required, which may increase the NW complexity.

All functions other than L1/L3 measurement based cell switch decision have an impact on improving C-plane robustness and reducing U-plane interruption time.

All additional features on top of L3 HO are effective in the C-plane robustness improvements.

The pre-configuration and RACH-less mobility contributes to reduce the U-plane interruption time.

The fast recovery improves C-plane robustness but does not affect other performance aspects, although the pre-configuration was required in 5G (e.g., a pre-configuration-less fast recovery may be considered for 6GR).

The other features such as Pre-configuration, Conditional mobility, RACH-less mobility and Subsequent mobility can improve the C-plane robustness and the U-plane interruption time. However, these increase the UE and the NW complexities and have a negative impact on performance with respect to radio resource waste/reservation. The problems with each of these additional features should be resolved in 6GR.

Pre-configuration, Conditional mobility, RACH-less mobility, Subsequent mobility may have issues in both increased UE/NW complexity and radio resource waste/reservation.

RAN2 should study the pros and cons of each feature in handover procedure and the methods to resolve their drawbacks for better handover performance and less options compared to NR.

Other consideration for connected mode mobility

AI mobility aspects

The AI/ML for mobility was studied in Rel-19, and the normative work to specify the RRM measurement prediction and the measurement event prediction has been approved for Rel-20 5G-Advanced [8]. The study result showed the considerable gains for the reductions of measurement processing and for the improvements of handover performance [9]. In addition, the AI-enabled radio is one of key requirements for 6GR [1]. Therefore, it’s worth considering whether these AI mobility features should be supported in 6G from Day 1.

From the mobility framework point of view, the expected impact to support AI mobility is mainly in the measurement report framework with regards to the RRM measurement prediction and the measurement event prediction, since the other big impact is LCM which should be handled by AI/ML agenda (unseen). It may be further expected that 6GR would support additional use cases, e.g., RLF prediction, AI-based CHO decision, etc., but such new use cases should be discussed later.

On the other hand, the duplicated efforts with Rel-20 5G-Advanced should be avoided. So, RAN2 should consider supporting AI mobility features and wait for 5G-Advanced normative work for now.

RAN2 should consider the support of AI mobility features, e.g., the RRM measurement prediction and the measurement event prediction, based on Rel-20 5G-Advanced progress.

MIMO harmonization aspects

The inter-cell multi-TRP operation is supported in Rel-17, which allows the DL transmission from two TRPs belonging to different cells [3]. It may be seen as the Joint Transmission in Coordinated Multi Point transmission (JT-CoMP) in LTE [10]. It may be also seen as a pathway towards the distributed MIMO or the cell-free MIMO operations. From RAN2 point of view, it’s almost seen as the cell switch, if the context transfer to the target cell is added, which, in particular, has commonalities with LTM.

In Rel-19, RAN1 introduced the UE-initiated beam management with the event conditions in PHY layer to trigger the CSI feedback transmissions [11]. On the other hand, RAN2 introduced the event-triggered L1 measurement reporting, whereby the event condition and L1 measurement reporting are defined in MAC layer [7]. These are, of course, for the different purposes (MIMO or LTM), but it’s also true that these functionalities look similar.

Since simplicity is one of the key requirements for 6GR, similar functionalities like these should be harmonized as part of the study, i.e., aiming to integrate these into a single/simple framework. On the other hand, such a harmonized framework may increase the complexity, which is a risk and is not aligned with the requirement for simplicity. So, RAN2 should study whether some MIMO features should be integrated within the mobility framework (or kept independently), involving RAN1.

RAN2 should discuss whether the mobility functions and the MIMO functions would be harmonized as a unified framework, e.g., with regards to the lower layer measurement report framework and the inter-cell multi-TRP operation.

Conclusion

In this contribution, the initial consideration for supporting 6G mobility are discussed. RAN2 is kindly asked to take into account the observations and proposals below:

Proposal 1	RAN2 should study the feasibility of achieving handover in 6G connected mode mobility through a single procedure using a simple configuration.

Observation 1	CHO, LTM and CLTM improve C-plane robustness and reduce U-plane interruption time; however, they also increase UE and NW complexity and cause radio resource waste/reservation.

Proposal 2	RAN2 should study 6G connected mode mobility with a focus on balancing C-plane robustness and U-plane interruption time, as well as UE/NW complexity and radio resource waste/reservation.

Observation 2	All additional features on top of L3 HO are effective in the C-plane robustness improvements.

Observation 3	The pre-configuration and RACH-less mobility contributes to reduce the U-plane interruption time.

Observation 4	Pre-configuration, Conditional mobility, RACH-less mobility, Subsequent mobility may have issues in both increased UE/NW complexity and radio resource waste/reservation.

Proposal 3	RAN2 should study the pros and cons of each feature in handover procedure and the methods to resolve their drawbacks for better handover performance and less options compared to NR.

Proposal 4	RAN2 should consider the support of AI mobility features, e.g., the RRM measurement prediction and the measurement event prediction, based on Rel-20 5G-Advanced progress.

Proposal 5	RAN2 should discuss whether the mobility functions and the MIMO functions would be harmonized as a unified framework, e.g., with regards to the lower layer measurement report framework and the inter-cell multi-TRP operation.

References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), RAN#108

- RP-252946, “Draft Reply LS to ITU-R_WP5D_TEMP_167  = RP-243202 on Minimum requirements related to technical performance for IMT-2030 radio interface(s)”, RAN, RAN#109

- TS38.300, “NR and NG-RAN Overall Description; Stage 2”, 3GPP

- TS38.321, “Medium Access Control (MAC) protocol specification”, 3GPP

- TS38.331, “Radio Resource Control (RRC) protocol specification”, 3GPP

- RP-251152, “Correction on S-CPAC and L3 measurements-based LTM”, RAN3, RAN#108

- RP-252777, “NR mobility enhancements Phase 4”, RAN2, RAN#109

- RP-251864, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, CATT (Moderator), RAN#108

- RP‑252876, “TR 38.744 v2.1.0 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, Guangdong OPPO Mobile Telecom., RAN#109

- TS36.300, “E-UTRA and E-UTRAN Overall description; Stage 2”, 3GPP

- RP‑251580, “Introduction of Rel-19 NR MIMO Phase 5”, RAN1, RAN#108