3GPP TSG-RAN WG2 #131bis	R2-2507552

Prague, Czech Republic, 13th – 17th October 2025

Source:	NTT DOCOMO, INC.

Title:	On remaining issues for AIoT MAC

Document for:	Discussion

Agenda Item:	8.2.2

- Introduction

This contribution is to provide our views on remaining issues spotted in the post e-mail discussion after RAN2#131 [1].

- Discussion

  - (Issue 1-3) Paging ID length field

In RAN2#131 [2] and its following post e-mail discussion [1], we have implemented the 8-bit Paging ID length field in A-IoT Paging message in accordance with following agreement, which was because RAN2 had received an LS from SA2 [3] and CT4 [4] that indicates to introduce up to 256-bit length of Paging ID (called Filtering Information in CT4 CR) field [4].

2	8-bit length field (in unit of bit) is assumed to indicate the paging ID length, based on current SA2/CT4 conclusion.

However, SA2 has sent another LS [5] in August WG meeting indicating that even longer Device permanent ID and consequent longer Filtering Information is needed for some industries. The LS indicates that the length of Filtering information (i.e., Paging ID) is approximately up to 600 bits.

Considering the information elements that currently need to be delivered via the Paging message (including those identified by RAN2 and other WGs), and the fact that the maximum length of the Paging message at the physical layer is 1000 bits, a Paging ID of up to 600 bits is feasible. Therefore, we believe that RAN2 should modify the length of the Paging ID field in accordance with SA2, as long as it remains feasible.

To indicate a maximum of approximately 600 bits, a length of 12 bits is required for the updated Paging ID length field.

Proposal 1.	(Issue 1-3) Update the length of Paging ID length field from 8 bits to 12 bits.

On the other hand, RAN2 is aware that the longer the overall paging message becomes, the more the system-wide coverage deteriorates.

Considering that SA2 stated in its LS, ‘SA2 notices the current design on AIoT Device permanent ID lengths ... has constraints and cannot meet the requirement for some industries like healthcare, global trade, etc.', the even longer Paging IDs and corresponding permanent Device IDs do not seem necessary for use cases outside of specific industries such as healthcare and global trade.

Therefore, it is good from RAN2 perspective to remind SA2 and CT4 via LS that the use of longer Paging IDs should be limited to specific use cases, and that keeping the paging message as short as possible is preferable.

Proposal 2.	(Issue 1-3) Send a reply LS to SA/CT4 to reminder that shorter Paging IDs are preferable and the use of longer Paging IDs should be limited to specific use cases.

  - (Issue 3-7) Value of More Data Indication when no NAS response is available

RAN2 made an agreement to set More Data Indication field to 0 when no data available case [2].

- The reader determines no data available case by SDU length 0.   As more data indication is mandatory, the device sets this bit to "0".

- The reader, in response to 0 SDU in the device’s MAC response, may send a follow-up R2D Upper Layer Data Transfer message at a later time to schedule another D2R Upper Layer Data Transfer message from the device.

On the other hand, some companies pointed out during the online discussion at RAN2#131 that there are cases where the device's application does not generate a NAS message. In such cases, the device should send a D2R data transfer message with no SDU content (0 SDU) to prevent the reader from detecting a D2R message failure and attempting to resend the R2D message.

Meanwhile, the reader needs to distinguish, based on the 0 SDU data transfer message sent by the device, whether the device failed to generate the NAS message in time, or whether it intentionally sent a D2R data transfer message with no SDU content because it determined that no NAS message would be generated.

The simplest way to achieve this is to distinguish between the two cases using the More Data Indication field within the D2R data transfer message that contains no SDU content. As mentioned earlier, it has already been agreed that when the device is in the process of generating a NAS message, the MDI field should be set to 0. Therefore, in a straightforward interpretation, when the device determines that it will not send a NAS message, the MDI field should be set to 1.

However, setting the More Data Indicator to 1 when the device does not intend to send a NAS message, and to 0 when the device intends to send a NAS message but has not yet generated it, goes against our intuition. Therefore, it would be better to revert the previous agreement so that the MDI field is set to 1 when the device notifies the reader that it is in the process of generating a NAS message.

Proposal 3.	(Issue 3-7) Revert a previous agreement so that the MDI field is set to 1 when the device notifies the reader that it is in the process of generating a NAS message.

Proposal 4.	(Issue 3-7) The MDI field is set to 0 when the device sends the D2R data transfer message with no SDU content because it determined that no NAS message would be generated.

- Summary and proposal

Proposal 1.	(Issue 1-3) Update the length of Paging ID length field from 8 bits to 12 bits.

Proposal 2.	(Issue 1-3) Send a reply LS to SA/CT4 to reminder that shorter Paging IDs are preferable and the use of longer Paging IDs should be limited to specific use cases.

Proposal 3.	(Issue 3-7) Revert a previous agreement so that the MDI field is set to 1 when the device notifies the reader that it is in the process of generating a NAS message.

Proposal 4.	(Issue 3-7) The MDI field is set to 0 when the device sends the D2R data transfer message with no SDU content because it determined that no NAS message would be generated.

- References

[1]	[POST131][021][AIoT] MAC spec (Huawei).

[2]	“Chair’s notes,” RAN2#131.

[3]	R2-2505058, “Reply LS on paging ID length,” SA2, RAN2#131.

[4]	R2-2505006, “Reply LS on paging ID length,” CT4, RAN2#131.

[5]	S2-2507793, “LS on AIoT Device Permanent ID Length,” SA2, SA2#170.