3GPP TSG-RAN WG2 Meeting #131-bis	R2-2507564

Prague, Oct. 13rd – 17th, 2025

# Agenda item:	10.3.3

# Source:	ETRI

# Title:	Discussion on Common User plane and Control plane

# Document for:	Discussion, Decision

- Introduction

In RAN#108, the new study item (SI) on 6G Radio (6GR) had been approved [1]. The following objectives is to study radio interface protocol procedures and AI/ML framework for 6GR and Radio Access Network.

- Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

The guidance contributions related to the Common User plane and Control plane is announced as the following:

#### 10.3.3 Common User plane and Control plane Access stratum security aspects, in alignment with requirements from SA3. Transfer of various type of data (including AI/ML data, sensing, etc. ) and general AI/ML framework considerations. NOTEs: Detailed AI/ML use case specific proposals are not expected in this meeting. Specific technical details/procedures related to sensing are not expected until RAN1 starts 6G sensing work.

This contribution proposes reusing the NR AI/ML framework as a basis and extending it for a 6G AI/ML framework to encompass new use scenarios. We discuss signaling enhancements for LCM and fast switching/transmission between AI/ML models that adapt to radio and traffic conditions at the air interface. The enhanced data collection for both AI/ML and sensing, as well as the transfer of this collected data to the server, are also discussed. Furthermore, we propose enhancing the MAC layer for 6G-R by extending and utilizing faster signaling procedures and introducing new security functions.

- Discussion

General AI/ML framework considerations

NR AI/ML LCM framework provides a comprehensive approach to manage the lifecycle of AI/ML models, including to data collection, model training, model storage, management, and inference. This framework leverages a functionality-based approach to manage UE-sided models, which is facilitated through UE capability and applicability reporting and network configuration. While the framework was being developed, a complete data collection framework was not yet available for the UE-sided model. Instead, data collection configurations were determined on a use-case basis. For the NW-sided model, a key consideration was to extend the existing MDT (Minimization of Drive Tests) framework to support data collection. When it comes to model delivery and transfer, the process occurs directly between the network and the UE. It is important to note that the network is not directly involved in the transfer of UE-sided models, which provides a degree of transparency in their delivery.

For 6G AI/ML LCM, the expanding adoption of AI/ML applications, coupled with the proliferation of devices and models, poses a challenge to Lifecycle Management. The problem is the signaling overhead, which can burden both the User Equipment (UE) and the network. For instance, a UE may execute multiple AI/ML operations. Managing the models' applicability in this scenario requires the UE to exchange signaling with the network to report applicability and control their activation/deactivation. Indivisual signaling for multiple AI/ML tasks in a UE and multiple AI/ML devices makes a signaling overheads. To tackle these issues, new signaling procedures are needed that prioritize efficiency and scalability.

For 6G AI/ML monitoring, AI/ML models require monitoring of the inference data to assess its similarity to the training environment. Inference data similar to the training environment is crucial for maintaining high performance of the AI/ML model; performance degrades significantly in dissimilar environments. For monitoring these degrading radio environments, the method must minimize the radio signaling used for monitoring and require the minimum necessary operations. Furthermore, it is essential to introduce and enhance common monitoring methods applicable across multiple AI/ML use cases within the radio environment monitoring framework.

For 6G AI/ML model switching, when the monitoring results detect a performance degradation, the system may fall back to a non-AI function, and the monitoring results should be reported to the network. Subsequently, the network initiates the process of switching to a new AI/ML model, typically proceeding through RRC signaling procedures. To address the need for reduced latency and overhead associated with these signaling procedures, which is vital for rapid model switching, we propose the following enhancement: Instead of improving the signaling procedure for each individual AI/ML model switching process, we propose establishing AI/ML model groups. The active model within a group is then configured based on the monitoring results. This approach requires signaling during the initial model group configuration, but the subsequent on-the-fly model changes within the group can be simplified and achieved without a full signaling procedure, thereby minimizing overhead and accelerating the switching time.

Observation 1: 5G NR LCM framework for NR air interface consists of data collection, model training, model storage, management, and inference, and defines interworking between them.

6G air interface data collection is intended to support diverse domains, such as AI/ML model training and inference for communication nodes and their applications.  The AI/ML is anticipated to be extensively deployed across both the radio access network (RAN) and the core network (CN) applied for a wide range of applications. To accommodate this extensive deployment, different LCM procedures have been defined across various Working Groups (WGs): R1/R2 for the radio interface, R3 for intra-RAN communication nodes, and SA for the core network. To enhance efficiency and inter-relationship, the AI/ML LCM should be harmonized and enhanced as a common function by leveraging the interdependencies with other WGs. Specifically, we must focus on a data collection strategy where radio-collected data can be utilized by the gNB/Core Network as well as by 3rd party AI/ML entities.

Proposal 1: Consider the NR LCM framework as baseline and study potential enhancements for 6G LCM

Transfer of various type of data

For NR data collection over the air interface, two primary models are currently considered. While the UE-sided model is not yet defined, the network-sided model extends the existing MDT (Minimization of Drive Tests) framework. In this network-sided approach, configuration is managed via the LoggedMeasurementConfiguration message, with reporting carried out using UEInformationRequest/Response messages transmitted. Additionally, data collection can utilize the Application Layer Measurement Report framework, where configuration is set via RRCReconfiguration (specifically the appLayerMeasConfig), and the measurement results are reported using MeasurementReportAppLayer messages transported. The aforementioned data collection procedures are transmitted as control messages over the control plane connection.

Observation 2: 5G AI/ML data collection is currently supported on the control plane as an extension of the MDT framework, meaning the aforementioned data collection procedures are transmitted as control messages over the control plane connection.

For 6G data collections, a unified data collection framework is necessary to support a wide array of AI/ML applications. The 6G AI/ML use scenarios are expanding significantly, building upon 5G-A AI/ML functions like CSI prediction, beam management, and AI mobility, and proposing new functionalities such as CSI-RS overhead reduction, AI receiver, AI modulation, and Non-linear Power Amplifier (PA) optimization. As AI/ML will be applied across all communication nodes (including the RAN radio, inter-node RAN interfaces, the core network, and OAM), the radio interface data is relevant to multiple applications. Generating data independently for each AI/ML use case would lead to data collection redundancy. To enhance efficiency, the collection of radio data must be optimized for diverse AI/ML applications. Furthermore, the sensing functionality required to detect the radio environment at communication nodes, which involves data collection and processing, must be integrated; thus, the scope of AI/ML data collection should be extended to encompass the field of radio environment sensing.

For 6G data transfer, to ensure collected data is effectively utilized by diverse AI/ML entities, including communication nodes and 3rd party AI/ML agents, establishing a data delivery framework is studied. For efficient data transport, a centralized architecture is recommended, wherein a data server centrally aggregates data collected from various measurement communication nodes. This server would then efficiently distribute the data to various requesting communication nodes for supporting diverse AI/ML and sensing applications.

In NR, data collection procedures are currently transmitted over the control plane. The communication entities handling the exchange of control messages are configured primarily to process signaling procedures. However, the transmission of collected data often involves either the frequent delivery of short data packets or the transfer of large data volumes. This characteristic makes the current control plane data transfer mechanism inadequate. Therefore, we must enhance the capabilities of the user plane (which is designed for user traffic) to transfer and distribute collected data. Furthermore, we should discuss introducing new connection type to support the functionality for delivering both AI/ML data and Sensing data. While the introduction of entirely new planes or layers might be considered if essential, this contribution specifically advocates for prioritizing the introduction of new connection type to resolve the existing transport limitations.

Observation 3: 6G data collection requires a unified data transmission structure that supports AI/ML data for RAN, sensing data, and application AI/ML data.

Proposal 2: Study a unified data transfer to provide common delivery function for transporting a variety of data types including RAN AI/ML data, sensing data, and application AI/ML data, which is separate from the control plane.

Access stratum security aspects

The NR specifications define a set of security mechanisms mainly implemented at the PDCP layer over the radio interface. These mechanisms ensure the confidentiality and integrity of communication, starting with Integrity Protection, which is mandated for all RRC control plane messages exchanged between the UE and the gNB. Furthermore, integrity protection is required for a subset of user plane traffic, specifically URLLC data. Regarding confidentiality, Ciphering and deciphering functions are fundamentally required for all traffic traversing the data plane. For the control plane (RRC messages), ciphering is defined as optional. gNB and UE at access stratum receive the KgNB anchor key from the AMF and locally derive all necessary AS traffic keys (KRRCint/enc, KUPint/enc) at the PDCP layer to secure both the control and user planes over the radio interface. For security and efficiency during mobility events (handover), the system uses two methods for updating the AS keys: vertical key derivation and horizontal key derivation.

Currently, signaling procedures exchanged at the RRC layer are forwarded to and processed by the CU. To facilitate fast signaling, certain control messages are transmitted via MAC CEs, which are handled directly by the DU, resulting in a quicker response time. However, this faster processing comes at the cost of security: MAC layer security is not provided. For instance, the cell switching command used in the LTM handover procedure is carried in a MAC CE. The absence of integrity protection at the MAC layer means that this handover signaling is inherently unsecured. Therefore, in 6G wireless signaling procedures, we propose to introduce and expand fast signaling procedures, and this must be supported by a corresponding expansion of the MAC layer signaling procedures with the necessary security provisions.

Observation 4: No support any security for NR MAC CE, but the MAC CE for HO cell switching command requires security to ensure the integrity and ciphering.

Observation 5: The utilization of MAC layer signaling is expected to increase in 6GR for the fast exchange of control information, which necessitates providing security support for these MAC signaling messages.

Proposal 3: Study security support at the MAC layer for ensuring the integrity protection for MAC layer signaling and potentially ciphering

- Conclusions

In this paper, we discuss the framework for the Common User plane and Control plane. Followings are observations and proposals made in this paper:

Observation 1: 5G NR LCM framework for NR air interface consists of data collection, model training, model storage, management, and inference, and defines interworking between them.

Observation 2: 5G AI/ML data collection is currently supported on the control plane as an extension of the MDT framework, meaning the aforementioned data collection procedures are transmitted as control messages over the control plane connection.

Observation 3: 6G data collection requires a unified data transmission structure that supports AI/ML data for RAN, sensing data, and application AI/ML data.

Observation 4: No support any security for NR MAC CE, but the MAC CE for HO cell switching command requires security to ensure the integrity and ciphering.

Observation 5: The utilization of MAC layer signaling is expected to increase in 6GR for the fast exchange of control information, which necessitates providing security support for these MAC signaling messages.

Proposal 1: Consider the NR LCM framework as baseline and study potential enhancements for 6G LCM

Proposal 2: Study a unified data transfer to provide common delivery function for transporting a variety of data types including RAN AI/ML data, sensing data, and application AI/ML data, which is separate from the control plane.

Proposal 3: Study security support at the MAC layer for ensuring the integrity protection for MAC layer signaling and potentially ciphering

- References

- RP-251395, Revised SID: Study on 6G Scenarios and requirements.

- RP-251881, New SID: Study on 6G Radio.

- TR 38.914, Study on 6G Scenarios and requirements.