3GPP TSG-RAN WG2 Meeting #131bis	R2-2507576

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	8.11.2

Source:	vivo

Title:	Discussion on UE transmit power continuity during RO type switching

Document for:	Discussion and Decision

1	Introduction

In the post RAN2#131 email discussion on MAC CR for SBFD, some MAC related issues are raised in [1], one of them is about UE transmit power continuity during RO type switching, and we will discuss this issue in this contribution.

2	Discussion on MAC-3: UE transmit power continuity during RO type switching

At RAN2#131, the following agreement on introducing a power offset to compensate the UE transmit power difference between legacy RO and SBFD RO was achieved.

For the RO type fallback between legacy RO and additional RO for RACH configuration option 2, a power offset given by the difference between the two values of preamble power ramping steps is added. Exact change to the MAC spec can be further discussed in the CR review. .

This agreement is reflected in the approved MAC CR [2] by introducing a new parameter ‘POWER_OFFSET_RO_TYPE’, which can be calculated based on the following equation.

6> set POWER_OFFSET_RO_TYPE to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (PREVIOUS_RO_TYPE_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

However, during the post RAN2#131 email discussion on MAC CR for SBFD, one company mentioned that the current agreement only considering the compensation for the potential differences of the power ramping step is insufficient, since the UE transmit power continuity is not ensured. More specifically, the UE will receive two independent RACH-ConfigCommon configurations in configuration 2, within which the values of preambleTargetReceivedPower and/or powerRampingStep may be different. If only the power offset for compensation of different powerRampingStep is considered, it may lead to a larger power ramp-down or power ramp-up of 6 dB, which is not acceptable. Therefore, the company propose that both the potential differences of the preamble target received power and the potential differences of the power ramping step need to be taken into account, when calculating the power offset which is used to compensate the UE transmit power difference between legacy RO and SBFD RO.

In our view, the power continuity issue mentioned above (i.e. UE should not change its transmit power by a value higher than 6 dB between Msg1 transmissions) is not existed. Firstly, although 6dB is the maximum value for the power ramping step as per the configuration of powerRampingStep, it doesn’t mean that the transmit power difference between Msg1 transmission cannot be larger than 6dB. As per the equation of PRACH transmit power calculation in TS 38.213 [3], the transmit power is not only related to the PRACH target received power, but also related to the pathloss. Sometimes, there may be large pathloss change between Msg1 transmission due to the high mobility or the change of environment, which will lead to that transmit power difference between Msg1 transmission is larger than 6dB, especially when a large value is configured for the power ramping step.

7.4 Physical random access channel A UE determines a transmission power for a physical random access channel (PRACH), PPRACH,b,f,c(i), on active UL BWP b of carrier f of cell c based on DL RS for cell c in transmission occasion i as PPRACH,b,f,ci=minPCMAX,f,ci,PPRACH,target,f,c+PLb,f,c [dBm], where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier f of cell c within transmission occasion i, - PPRACH,target,f,c is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER provided by higher layers [11, TS 38.321] for the active UL BWP b of carrier f of cell c, and - PLb,f,c is a pathloss for the active UL BWP b of carrier f based on the DL RS associated with the PRACH transmission on the active DL BWP of cell c and calculated by the UE in dB as referenceSignalPower – higher layer filtered RSRP in dBm, where RSRP is defined in [7, TS 38.215] and the higher layer filter configuration is defined in [12, TS 38.331]. If the active DL BWP is the initial DL BWP and for SS/PBCH block and CORESET multiplexing pattern 2 or 3 as described in clause 13, or for a non-serving cell, the UE determines PLb,f,c based on the SS/PBCH block associated with the PRACH transmission.

In other words, from Rel-15 NR, the transmit power difference between Msg1 transmissions higher than 6dB is supported.

Observation 1: The transmit power difference between Msg1 transmissions may be larger than 6dB from Rel-15 NR, due to the pathloss change and the power ramping.

Secondly, similar case is already existed for the 2-step RACH feature. When RA type is switched from 2-step RA to 4-step RA during the random access procedure, a dedicated power offset which is used to compensate the UE transmit power difference between 2-step RACH and 4-step RACH is introduced, i.e., POWER_OFFSET_2STEP_RA as defined in TS 38.321, and it is calculated based on the difference of power ramping step and the number of RACH attempt [4].

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

Similar as the case mentioned for SBFD configuration 2, UE will receive RACH-ConfigGenericTwoStepRA-r16 for 2-step RACH and RACH-ConfigGeneric for 4-step RACH, within which there are independent configurations for (msgA)powerRampingStep and (msgA)preambleReceivedTargetPower, and the configured values may be different. However，as per the MAC spec, when RA type is switched from 2-step RA to 4-step RA, the difference of the preamble target received power is not taken into account for calculating the power offset, i.e. there is no power compensation for the differences of the preamble target received power.

Observation 2: When RA type is switched from 2-step RA to 4-step RA, only the difference in the power ramping step is considered to compensate the transmit power difference.

As a summary, firstly, the maximum value of 6dB for powerRampingStep doesn’t mean that UE should not change its transmit power by a value higher than 6 dB between Msg1 transmissions. Secondly, the transmit power difference between Msg1 transmissions higher than 6dB is supported from Rel-15 NR. Thirdly, the power difference for the preamble received target power is also not taken into account to calculate the power offset when RA type is switched from 2-stepRA to 4-stepRA. Therefore, we’d like not introducing other factors for the calculation of SBFD related power offset when the RO type fallback is performed between legacy RO and additional RO for RACH configuration option 2, i.e. follow the equation defined in the approved MAC CR, and do not take preamble received target power differences into account.

Proposal 1: Preamble received target power differences is not considered for the calculation of POWER_OFFSET_RO_TYPE.

3	Conclusion

Based on the discussion, we have the following observations,

Observation 1: The transmit power difference between Msg1 transmissions may be larger than 6dB from Rel-15 NR, due to the pathloss change and the power ramping.

Observation 2: When RA type is switched from 2-stepRA to 4-stepRA, only the difference in the power ramping step is considered to compensate the transmit power difference.

And we also have the following proposal,

Proposal 1: Preamble received target power differences is not considered for the calculation of POWER_OFFSET_RO_TYPE.

4	Reference

[1] R2-2507158, Summary of Rel-19 SBFD MAC open issue discussions for maintenance, Samsung

[2] R2-2506491, Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec, Samsung

[3] 3GPP TS 38.213: "NR; Physical Layer Procedures for control"

[4] 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"