3GPP TSG-RAN WG2 Meeting #131bis									R2-2507608

Prague, Czech Republic, Oct. 13rd-17th

Agenda item:		6.1.1

Title: 	Consideration on the LS on compatibility issue for PEI and emergency PDU session

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

Before RAN2#131bis, an LS on compatibility issue for PEI and emergency PDU session [1] has been provided from RAN3. Since some solutions provided in the LS have RAN2 specification impact, we will discuss the related issues from RAN2 perspective, and give our proposals.

- Discussion

In the LS on compatibility issue for PEI and emergency PDU session [1], RAN3 has identified the backward compatibility issue in two scenarios:

RAN3 has identified the backward compatibility issue when different releases of UE/NG-RAN/AMF are deployed. Specifically, there are, among others, two typical scenarios: For both scenario 1 and 2, assumption is that UE performs registration first and after some time UE established emergency PDU session without updating its NAS network capability to monitor PEI while emergency PDU session is active. - Scenario 1, R19 UE (without the restriction of using PEIPS during emergency PDU session), R17/18 gNB and AMF (with the restriction of using PEIPS during emergency PDU session): the R19 UE will monitor PEI with PEIPS while R17R18 AMF/gNB will not use PEIPS to page the UE, thus the R19 UE will miss Paging message with PEIPS. - Scenario 2, R17/R18 UE (with the restriction of using PEIPS during emergency PDU session), R19 gNB and AMF (with no restriction of using PEIPS during emergency PDU session): the R19 AMF/gNB could use PEIPS to page the UE while the R17 UE shall not use PEIPS, but may monitor PEI using UE_ID based subgroup ID, in this case, the UE will miss Paging message.

From RAN2 perspective, the following principle are provided.

- When UE does not monitor PEI, but network send PEI, there is not critical issue, e.g. PAGING will not be missed, only some radio resource and network power will be waste.

- When UE monitors PEI, but network does not send PEI, there is critical issue, e.g. PAGING may be missed, which should be avoided.

- When UE monitors PEI, network sends PEI, but the PEI subgroup is different in UE and gNB side (e.g. UE monitors CN assigned PEI and gNB sends UE-ID based PEI), there is critical issue, e.g. PAGING may be missed, which should be avoided.

For the scenario 1 identified by RAN3, if PEIPS is allocated to Rel-19 UE, UE will monitor CN assigned PEI subgroup; but if Rel-17/Rel-18 AMF has not provided the PEIPS to gNB for there is emergency PDU session, gNB will not send PEI. There is critical issue.

For the scenario 2 identified by RAN3, if PEIPS is allocated to Rel-19 UE, Rel-19 AMF will provided the PEIPS to gNB, and gNB will wend CN assigned PEI subgroup, but Rel-17/Rel-18 UE with emergency PDU session will NOT monitor CN assigned PEI subgroup, but maybe monitor UE-ID based PEI subgroup; This mismatch may cause PAGING missing, which is also a critical issue and should be avoided.

Observation 1: The two scenarios identified by RAN3 will cause paging loss thus should be addressed.

RAN3 provides the following 6 solutions for information:

RAN3 has started to discuss six possible solutions provided below for information but think that SA2 should take the lead to solve these issues and make the necessary coordination among all working groups. - Whether it is possible for SA2 to revert the R19 agreed CRs on removing the restriction of emergency PDU session for PEI? - Whether it is possible for SA2 to remove the restriction of emergency PDU session for PEI from R17 onwards? - An unified solution to be considered to address the compatibility issue solely through modifications to the Rel-19 RAN or/and CN specification? - R17/R18 gNBs not use the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19)? - R17/R18 UEs ignore the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19)? - R17/R18 UEs initiate a registration procedure without indicating its capability of supporting PEIPS after the emergency PDU session is set up, so that AMF will not use the PEIPS?

- Solution 1: Revert the R19 CR on removing the restriction of emergency PDU session for PEI.

- Solution 2: Remove the restriction of emergency PDU session for PEI from R17 and onwards.

- Solution 3: A unified solution to address the compatibility issue solely.

- Solution 4: R17/R18 gNBs not use the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19).

- Solution 5: R17/R18 UEs ignore the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) in the presence of emergency PDU session (assuming SA2 CRs are applicable only from R19)

- Solution 6: R17/R18 UEs initiate a registration procedure without indicating its capability of supporting PEIPS after the emergency PDU session is set up, so that AMF will not use the PEIPS.

For the solution 1, although there is no mismatch, but if UE has been assigned PEIPS and has an emergency PDU session, UE still monitor UE-ID based PEI subgroup based on TS 38.304, even it does not monitor CN assigned PEI subgroup for NAS restriction. The NAS restriction has no gain and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

Observation 2: Solution 1 intends to disable PEI monitoring based on CN assigned subgroups, and may result in in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

For the solution 2, i.e. removing the restriction of emergency PDU session for PEI from R17 onwards, network will always provide PEIPS to gNB and gNB will send CN assigned PEI subgroup. The existing R17/R18 UE with emergency PDU session will monitor UE ID based PEI, there is still some mismatch. That is to say, whether it is feasible or not depends on whether there is R17/R18 UE supporting PEI in the commercial deployment.

Observation 3: Whether Solution 2 is feasible depends on whether there is R17/R18 UE supporting PEI in the existing network.

For the solution 3, NAS UE capabilityshould be introduced in NAS, and it is a usual method to deal with the compatibility issue.

Observation 4: Solution 3 with NAS UE capability is feasible way to deal with the compatibility issue.

For the solution 4, “R17/R18 gNB not use the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping)”, the UE-ID based PEI mechanism should be updated, e.g. UE does not monitor UE-ID based PEI when there is an emergency PDU session, which impactsRAN2 specification, and has no gain.

For the solution 5, “R17/R18 UEs ignore the complete PEI (i.e. both CN based subgrouping and UE ID based subgrouping) ”, it is the same as solution 4, the UE-ID based PEI mechanism should be updated, e.g. UE ignore UE-ID based PEI when there is an emergency PDU session, which impactsRAN2 specification, and has no gain.

Observation 5: Solution 4 and 5 intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change to disable PEI monitoring with UE ID based subgrouping.

For the solution 6, UE with emergency PDU session will not monitor CN assigned PEI, but will monitor UE-ID based PEI. This has the same effect as existing specification, has no gain to disable CN assigned PEI subgroup and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

Observation 6: Similar as solution 1, Solution 6  intends to disable PEI monitoring based on CN assigned subgroups, and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

Based on the consideration above;

Solution 4 and solution 5 should be rolled out, as they intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change.

For Solution 1 and solution 6, UE with emergency PDU session will not monitor CN assigned PEI but will monitor UE-ID based PEI thus should not be considered as they intend to disable PEI monitoring based on CN assigned subgrouping and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

For solution 2, whether it is feasible depends on whether there is R17/R18 UE supporting PEI in the real network. If No such UEs, then it is feasible.

For Solution 3, with NAS UE capability, the compatibility issue can be solved. And it is a usual method to deal with the compatibility issue.

Proposal: Reply LS to indicate the following consideration from RAN2 perspective:

- Solution 4 and solution 5 should be rolled out, as they intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change.

- Solution 1 and solution 6 should not be considered as they intend to disable PEI monitoring based on CN assigned subgrouping and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

- Solution 2 is feasible and preferred if there is no R17/R18 UE supporting PEI in the real network.

- Solution 3 is feasible and preferred if there is R17/R18 UE supporting PEI in the real network.

- Suggest SA2 to select solution 2 or solution 3 based on whether there is R17/R18 UE supporting PEI in the real network or not.

Based on the Proposal, the Reply LS is provided in [2].

- Conclusion

The following observations and proposals are made:

Observation 1: The two scenarios identified by RAN3 will cause paging loss thus should be addressed.

Observation 2: Solution 1 intends to disable PEI monitoring based on CN assigned subgroups, and may result in in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

Observation 3: Whether Solution 2 is feasible depends on whether there is R17/R18 UE supporting PEI in the existing network.

Observation 4: Solution 3 with NAS UE capability is feasible way to deal with the compatibility issue.

Observation 5: Solution 4 and 5 intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change to disable PEI monitoring with UE ID based subgrouping.

Observation 6: Similar as solution 1, Solution 6  intends to disable PEI monitoring based on CN assigned subgroups, and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

Proposal: Reply LS to indicate the following consideration from RAN2 perspective:

- Solution 4 and solution 5 should be rolled out, as they intend to disable PEI monitoring completely when there is emergency PDU session and would require RAN2 spec change.

- Solution 1 and solution 6 should not be considered as they intend to disable PEI monitoring based on CN assigned subgrouping and may bring in-consistent behavior for CN assigned PEI monitoring and UE-ID based PEI monitoring.

- Solution 2 is feasible and preferred if there is no R17/R18 UE supporting PEI in the real network.

- Solution 3 with NAS UE capability is feasible and preferred if there is R17/R18 UE supporting PEI in the real network.

- Suggest SA2 to select solution 2 or solution 3 based on whether there is R17/R18 UE supporting PEI in the existing network or not.

Based on the Proposal, the Reply LS is provided in [2].

- References

- R2-2506725_R3-255906 LS on compatibility issue for PEI and emergency PDU session, RAN2#131bis, October, 2025

- R2-2507609, Reply LS on compatibility issue for PEI and emergency PDU session, RAN2#131bis, October, 2025