3GPP TSG-RAN WG2 Meeting #131bis	R2-2507610

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Paging Search Space monitor for PDCCH repetition

WID/SID:	NR_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In the post-meeting email discussion [1] for 38.304, one issue was identified to be further discussed during this meeting.

Open issue 2: With the support of common PDCCH repetition, the UE, the UE will monitor the pagingSearchSpace and its linked CSS for paging reception. In the TS 38.304 spec, the PO concept should be applied to SearchSpace linked with pagingSearchSpace.

Furthermore, in the RRC ASN.1 review [2], RIL [Z256] was raised to add linked search space indication in paging search space configuration.

Since the PDCCH repetition for PDCCH Common Search Space (CSS) has been discussed and agreed upon in RAN1, this document revisits the RAN1 agreement and the discussion progress, and then proposes a way forward to address the 38.304 open issue and the related RRC RIL[Z256].

# 2	Discussion

## 2.1	Linked Search Space monitoring in 38.304

In RAN1-122 meeting, below agreement was reached to support the PDCCH repetition of PDCCH CSS.

Agreement For intra-slot PDCCH repetition of PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero,: - Specify an explicit linkage of two SS for intra-slot PDCCH repetition. That is, a RRC parameter is introduced for SS linkage and enabling intra-slot PDCCH repetition. The starting symbol of monitoring occasion of the second SS is located right after the ending symbol of monitoring occasion of the first SS. - A new parameter in SIB1 e.g. searchSpaceLinkingId-r19 is introduced or Rel-17 parameter (i.e. SearchSpaceLinkingId) is re-interpreted with modification to capture RAN1 agreements.

Based on the agreement above, PDCCH repetition for the PDCCH CSS can be applied to the Paging Search Space (i.e., PDCCH CSS Type2).  To support intra-slot PDCCH repetition, the NW may configure two linked Search Spaces by assigning a new parameter SearchSpaceLinkingId-r19 to each. If both search space are configured with the same SearchSpaceLinkingId-r19 value, they are considered linked (called as SS linkage).

Observation 1: To enable intra-slot PDCCH repetition for Paging, two paging search space can be linked by configuring them with the same SearchSpaceLinkingId-r19 value.

Furthermore, it appears that not all Rel-19 NTN UEs can support intra-slot PDCCH repetition reception (i.e., linked SS reception). RAN1 is currently discussing the UE capability feature (FG 65-1-3) to indicate whether a UE supports this functionality. However, several aspects remain to be finalized (FFS) and will require further discussion in upcoming RAN1 meetings.

Observation 2: RAN1 is discussing a UE capability to indicate whether the UE supports recetption on linked Search Spaces for paging. However, it is not concluded yet.

Furthermore, even the UE supports the reception on linkded SS, the RAN1 agreements do not clearly specify the UE behaviour regarding paging SS monitoring. While it is understood that the UE may decode a PDCCH candidate in the first SS and its duplicate in the second SS in order to combine channel symbols and potentially improve detection performance, it remains unclear whether the UE is required to monitor both SS in all cases. For example, it is not defined whether the UE may skip monitoring the second linked SS if it successfully decodes the Paging message from the first SS.

Observation 3: For UE that support PDCCH CSS reception on linked Search Space for paging, the associated UE behaviour is not clearly defined in the RAN1 agreements.

For issue on how to update section 7.1 of TS38.304, we think at least the modification should consider the possible UE capability and intened UE behaviour on paging monitor.

TS38.304: 7.1 Discontinuous Reception for paging … When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

In the case where NW configures two linked SS, and since both the UE capability for linked SS reception and the intend UE behaviour for paging monitoring have been discussed but not yet concluded in RAN1, we propose to keep the current wording in TS38.304 for paging monitoring and postpone the discussion on the further modification.

Proposal 1: RAN2 postpone discussion on updating TS38.304 regarding paging monitoring for linked search spaces until RAN1 finalizes the UE capability and expected UE behavior for PDCCH reception on linked Paging Search Spaces.

## 2.2	RIL Z256

Z256: RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. Z256; NTN; 2; Missing linked SS in paging search space configuration; None; ZTE (Zhihong); v012; ToDo. [Description]: SearchSpaceLinkingIdCE is only used to linked two SS, while searchSpaceId for the additional SS for pagingSearchSpace is still needed to be added in PDCCH-ConfigCommon [Proposed Change]: Add pagingSearchSpaceExt in PDCCH-ConfigCommon per below: PDCCH-ConfigCommon information element -- ASN1START -- TAG-PDCCH-CONFIGCOMMON-START PDCCH-ConfigCommon ::= SEQUENCE { controlResourceSetZero ControlResourceSetZero OPTIONAL, -- Cond InitialBWP-Only commonControlResourceSet ControlResourceSet OPTIONAL, -- Need R searchSpaceZero SearchSpaceZero OPTIONAL, -- Cond InitialBWP-Only commonSearchSpaceList SEQUENCE (SIZE(1..4)) OF SearchSpace OPTIONAL, -- Need R searchSpaceSIB1 SearchSpaceId OPTIONAL, -- Need S searchSpaceOtherSystemInformation SearchSpaceId OPTIONAL, -- Need S pagingSearchSpace SearchSpaceId OPTIONAL, -- Need S ra-SearchSpace SearchSpaceId OPTIONAL, -- Need S ..., [[ firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL -- Cond OtherBWP ]], [partailly omitted] [[ applyIndicatedTCI-State-r18 ENUMERATED {first, second, both, none} OPTIONAL, -- Cond FollowUTCI commonSearchSpaceListExt-r18 SEQUENCE (SIZE(1..4)) OF SearchSpaceExt-v1800 OPTIONAL, -- Need R searchSpaceMulticastMCCH-r18 SearchSpaceId OPTIONAL, -- Need R searchSpaceMulticastMTCH-r18 SearchSpaceId OPTIONAL -- Need S ]], [[ pagingSearchSpaceExt SearchSpaceId OPTIONAL, -- Cond PagingSearchSpace ]] } -- TAG-PDCCH-CONFIGCOMMON-STOP -- ASN1STOP

RIL Z2566 proposes to introduce a new IE (pagingSearchSpaceExt) in PDCCH-ConfigCommon, in order to instruct the UE to monitor Paging in the linked SS associated with the pagingSearchSpaceExt.

However, if the NW configure a Paging SS and a linked SS (by assigning the same searchSpaceLinkingId-r19 to both of the two SS), the UE can infer that the linked SS is also intented for paging monitoring. Therfore, defining an explict pagingSearchSpaceExt IE for the linked SS is unnecessary.

Proposal 2: It is unnecessary to introduce pagingSearchSpaceExt in PDCCH-ConfigCommon. The RIL Z256 can be rejected.

# 3	Conclusion

This document has made the following observations:

Observation 1: To enable intra-slot PDCCH repetition for Paging, two paging search space can be linked by configuring them with the same SearchSpaceLinkingId-r19 value.

Observation 2: RAN1 is discussing a UE capability to indicate whether the UE supports recetption on linked Search Spaces for paging. However, it is not concluded yet.

Observation 3: For UE that support PDCCH CSS reception on linked Search Space for paging, the associated UE behaviour is not clearly defined in the RAN1 agreements.

And proposed the following:

Proposal 1: RAN2 postpone discussion on updating TS38.304 regarding paging monitoring for linked search spaces until RAN1 finalizes the UE capability and expected UE behavior for PDCCH reception on linked Paging Search Spaces.

Proposal 2: It is unnecessary to introduce pagingSearchSpaceExt in PDCCH-ConfigCommon. The RIL Z256 can be rejected.

# Reference

[1] R2-25xxxxx Open issues on Rel-19 NR NTN 38.304, ZTE

[2] R2-25xxxxx NTN Comments file v021, Ericsson